3GPP TSG-SA WG6 Meeting #52	S6-223537

Toulouse, France, 14th – 18th November 2022	(revision of S6-223353, S6-223157)

Source:	BDBOS, Nokia, Nokia Shanghai Bell, Ericsson

Title:	Exchange Initial MC service UE configuration data between MC systems

Spec:	3GPP TR 23.700-38 v0.4.0

Agenda item:	9.2

Document for:	Approval

Contact:	dania.azem@bdbos.bmi.bund.de

1. Introduction

This pCR adds a solution to solve key issue 3, and key issue 5.

2. Reason for Change

This pCR adds a solution to related to:

-	Key issue 3 - change user configuration

Investigate and define how to negotiate and agree user profiles (user configuration data) that enable the owner to migrate into a partner MC system.

-	Key issue 5 – Service configuration data

Investigate and develop solutions, if required, which enable the exchange of administrative service configuration data between interconnected MC systems, which are not yet currently available.

This solution enables an authorized user to provide configuration data from a partner MC system needed to configure or update the initial MC service UE configuration data stored in an MC service UE. Proper initial MC service configuration is a pre-condition for authentication and authorization for MC services in a partner MC system. The process how an authorised user configures MC service UEs (before migration) is out of 3GPP scope, i.e., no changes to this aspect.

This solution addresses SA1 requirements as

[R-5.16.4-004] An MCX Service shall provide a mechanism to allow an authorised MCX User to request configuration information (e.g., users, groups, security level) from Partner MCX Service Systems.

[R-5.16.4-005] An MCX Service shall provide a mechanism to allow an authorised MCX User to send configuration information to Partner MCX Service Systems.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-38 v0.4.0.

* * * First Change * * * *

## 7.2	Mapping of solutions to key issues

Table 7.2-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |
| Sol #3 |  |  |  |  |  | X |
| Sol #5 |  |  | X |  |  |  |
| Sol#x |  |  | X |  | X |  |

* * * Next Change * * * *

## 7.x	Solution #x: Update MC service UE initial configuration

### 7.x.1	General

This solution addresses the following aspects:

- Key issue 3 - change user configuration

- Key issue 5 – Service configuration data

This solution enables an authorized user to provide configuration data to a partner MC system needed to update the initial MC service UE configuration data to be stored in an MC service UE enabling migration to a partner MC system. The initial MC service configuration is a pre-condition for authentication and authorization for MC services in a partner MC system.

Initial MC service UE configuration data can be exchanged between the MC systems using ACMX entities as described in the following procedure. This solution allows to keep a partner MC system up to date with regard to the required initial MC service UE configuration date needed for migration.

### 7.x.2	Solution description

### 7.x.1.2	Information flows

##### 7.x.1.2.1	Update MC service UE initial configuration

Table 7.x.1.2.1-1 describes the information flow from the primary ACMC to the primary ACMS, and from the primary ACMS to the partner ACMS, for updating initial MC service UE configuration data information that is required by the partner MC system for preparing migrating MC service UEs.

Table 7.x.1.2.1-1: Update MC service UE initial configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the primary MC system who triggers the information update. |
| Functional alias | O | The functional alias of the MC service user at the primary MC system who triggers the information update. |
| MC service ID | O (see NOTE 1) | The identity of the MC service user at the partner MC system towards which the information update is sent |
| Functional alias | O (see NOTE 1) | The functional alias of the MC service user at the partner MC system towards which the information update is sent |
| MC service ID list | M | Set of MC service IDs of the migrating MC service users that belong to the primary MC system. |
| MC service UE initial configuration data (see NOTE 2) | M | This information element contains the information as specified in table A.6-1 of 3GPP TS 23.280 [5] corresponding to the MC service IDs above. |
| NOTE 1: Either the MC service ID or the functional alias must be present. NOTE 2: The MC service UE label defined in TS 23.280 [5], Annex 6 is sent in this information element only in case the request is targeting a certain MC service UE(s), otherwise this information is not present. |  |  |

##### 7.x.1.2.2	Update MC service UE initial configuration response

Table 7.x.1.2.2-1 describes the information flow from the partner ACMC to the partner ACMS and from partner ACMS to primary ACMS.

Table 7.x.1.2.2-1: Update MC service UE initial configuration response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the partner MC system which handled the received update. |
| Functional alias | O | The functional alias of the MC service user at the partner MC system which handled the received update. |
| Result | M | Result information of the update MC service UE initial configuration. |

### 7.x.1.3	Procedure

The procedure for an authorized MC service user in a primary MC system to update the partner MC system with the initial MC service UE configuration data information is shown in figure 7.x.1.3-1.

Pre-conditions

- The primary and partner MC system(s) are configured to accept connections from relevant ACMCs via the respective ACMSs in each of the connected MC service system(s) and have been configured and authorized successfully to allow exchange of administrative configuration information

- The relevant ACMC(s) and ACMS connection authorizations have been established successfully

- The set of MC service IDs and the MC service UE initial configuration of the migrating MC service users have been already sent by the primary MC system to the partner MC system, as specified in solution #5.

Figure 7.x.1.3-1: Update MC service UE initial configuration

- The primary ACMC sends an update MC service UE initial configuration to the primary ACMS, providing the initial MC service UE configuration data as specified in 3GPP TS 23.280 [5], Annex 6 to the primary MC System.

2.	The primary ACMS performs an authorization check to verify that the MC service user is authorized to perform this action. A successful authorization check results in the Update MC service UE initial configuration being forwarded to the relevant partner ACMS. If the authorization check is failed the next steps are not followed

3.	The primary ACMS sends the update MC service UE initial configuration to the partner ACMS.

4. 	The partner ACMS processes and stores the incoming update. Based on local policies and configurations, the partner ACMS may automatically handle the received update and send an update MC service UE initial configuration response to the primary ACMS or may require request verification by the ACMC as described in steps 6 to 7.

5.	The partner ACMS may send a process pending indication to the primary MC system.

NOTE 1:	There may be a considerable pause between step 4 and 7.

6.	The partner ACMS forwards the stored update MC service UE initial configuration to the partner ACMC.

7.	The authorized MC service user of partner MC system processes the received update MC service UE initial configuration.

NOTE 2: 	It is out of the scope of this document how the received initial MC service UE configuration data is applied to MC service UEs (before migration).

8. The partner ACMC sends update MC service UE initial configuration response to the partner ACMS.

9. The partner ACMS sends update MC service UE initial configuration response to the primary ACMS.

10.	The primary ACMC stores the update MC service UE initial configuration response.

NOTE 3:There may be a considerable pause between step 10 and 11.

11.	The primary ACMS forwards the update MC service UE initial configuration response to the primary ACMC.

### 7.x.2	Solution evaluation

This solution enables an authorized user of the primary MC system to provide an update of the MC service UE initial configuration to be stored in the MC service user profiles in a partner system, enabling migration to the primary MC system. The solution addresses key issue 3 and key issue 5.

* * * End of Change * * * *