3GPP TSG-SA WG6 Meeting #52	S6-223562

Toulouse, France 14th – 18th November 2022	(revision of S6-223464 was 3317)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 149 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EDGE-5 – AC registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures and information flows related to AC’s registration with EEC are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides: - AC registration update and deregistration procedures. - Information flows related to AC registration, registration update and deregistration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AC registration related clauses will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.2, 8.14.2.1, 8.14.2.2 (new), 8.14.2.2.1 (new), 8.14.2.2.2 (new), 8.14.2.2.3 (new), 8.14.2.2.4 (new), 8.14.3, 8.14.3.1 (new), 8.14.3.2 (new), 8.14.3.3 (new), 8.14.3.4 (new), 8.14.3.5 (new), 8.14.3.6 (new), 8.14.3.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.14.2	Procedures

#### 8.14.2.1	General

Following procedures are specified for EDGE-5:

-	Registration;

-	EAS discovery request;

-	ACR request;

-	AC subscription; and

-	AC notification.

* * * Next Change * * * *

#### 8.14.2.2	Registration

##### 8.14.2.2.1	General

Following are supported for AC registration:

-	AC registration procedure;

-	AC registration update procedure; and

-	AC de-registration procedure.

##### 8.14.2.2.2	AC registration

Figure 8.14.2.2.2-1 illustrates AC registration procedure.

Pre-conditions:

1.	AC can communicate with the EEC.

Figure 8.14.2.2.2-1: AC registration procedure

1.	The AC sends an AC registration request to the EEC. The request includes the AC profile, AC's security credentials and optionally the EAS characteristics. The request may also include a list of EEC's services that AC requires the EEC to handle. The request additionally includes ECS configuration information if the AC is edge-aware and configured with the ECS configuration information.

NOTE 1:	The ASP providing the AC and the ECSP providing the ECS can have edge computing service provider service agreement as described in Annex B. The ECS configuration information configured in the AC is based on the service agreement.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC registers the information provided in the request and responds back to the AC with AC registration response. The AC registration response includes the list of EEC's services that AC is authorized for.

NOTE 2:	The mechanisms used for authentication and authorization between AC and EEC is out of scope of this specification. EEC can use local policies, user preferences, ASP services agreement(s) (see Annex B) to authorize the request from the AC.

NOTE 3:	When the ECS configuration information is provided from an AC, the EEC can use the ECS configuration for initial service provisioning for the AC that provided the ECS configuration information if there is no ECS configuration information is provided from the 5GC.

##### 8.14.2.2.3	AC registration update

Figure 8.14.2.2.3-1 illustrates AC registration update procedure.

Pre-conditions:

1.	AC is registered with the EEC.

Figure 8.14.2.2.3-1: AC registration update procedure

1.	The AC sends an AC registration update request to the EEC. The request includes the registration ID, AC's security credentials, and may include the updated AC profile, EAS discovery filters, list of requested EEC services and list of ECS information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC sends a successful registration update response, which includes an updated list of EEC services that AC is authorized for.

##### 8.14.2.2.4	AC deregistration

Figure 8.14.2.2.4-1 illustrates AC deregistration procedure.

Pre-conditions:

1.	AC is registered with the EEC.

Figure 8.14.2.2.4-1: AC deregistration procedure

1.	The AC sends an AC deregistration request to the EEC. The request includes the registration ID and AC's security credentials.

2.	The EEC checks AC's security credentials and validates the request.

3.	Upon successful authorization, the EEC deregisters the AC and sends a successful de-registration response.

* * * Next Change * * * *

### 8.14.3	Information flows

#### 8.14.3.1	General

#### 8.14.3.2	AC registration request

Table 8.14.3.2-1 describes information elements of the AC registration request sent by the AC to the EEC .

Table 8.14.3.2-1: AC registration request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile of the AC sending the registration request. |
| Security credentials | M | Security credentials of the AC sending the registration request. |
| List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |
| List of requested EEC services | O | One or more EEC services requested by the AC e.g., EAS discovery, ACR. |
| List of ECS information | O | One or more ECS configuration information |

#### 8.14.3.3	AC registration response

Table 8.14.3.3-1 describes information elements of the AC registration response sent by the EEC to the AC.

Table 8.14.3.3-1: AC registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the AC registration. |
| > List of allowed EEC services | M | List of all the EEC services AC is authorized to use. |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |

#### 8.14.3.4	AC registration update request

Table 8.14.3.4-1 describes information elements in the AC registration update request sent by the AC to the EEC.

Table 8.14.3.4-1: AC registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | AC registration identifier provided by the EEC during AC registration. |
| Security credentials | M | Security credentials of the AC sending the registration update request. |
| AC profile | O | AC profile of the AC sending the registration update request. |
| List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |
| List of requested EEC services | O | One or more EEC services requested by the AC e.g., EAS discovery, ACR. |
| List of ECS information | O | One or more ECS configuration information |

#### 8.14.3.5	AC registration update response

Table 8.14.3.5-1 describes information elements in the AC registration update response sent by the EEC to the AC.

Table 8.14.3.5-1: AC registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > List of allowed EEC services | M | List of all the EEC services AC is authorized to use. |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |

#### 8.14.3.6	AC deregistration request

Table 8.14.3.6-1 describes information elements in the AC deregistration request sent by the AC to the EEC.

Table 8.14.3.6-1: AC deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | AC registration identifier provided by the EEC during AC registration. |
| Security credentials | M | Security credentials of the AC sending the deregistration request. |

#### 8.14.3.7	AC deregistration response

Table 8.14.3.7-1 describes information elements in the AC deregistration response sent by the EEC to the AC.

Table 8.14.3.7-1: AC deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the deregistration request was successful. |
| Failure response | O | Indicates that the deregistration request failed. |
| > Cause | M | Provides the cause for deregistration request failure. |

* * * Next Change * * * *