3GPP TSG-SA WG6 Meeting #52	S6-223572

Toulouse, France 14th – 18th November 2022	(revision of S6-223441)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0086 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution on EN about UE type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Rel-18 |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.2.1 contains an EN that needs to be resolved. The MSGin5G Server does not require to know the UE type since there is no procedure in the TS that contains any information element for which this knowledge is required. (E.g., the outbound message from the Server does not include the Service ID of a Gateway.). All the routing of the outbound message from the MSGin5G Server is done at the transport level. Text in 5.3.2.1 states that when the MSGin5G Server receives an inbound message I will check its repository to determine the Service ID of the end-point, but the Service ID of the end-point is already included in the inbound message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EN in clause 4.2.1 is removed, because the present CR assumes that the MSGin5G Server does not require knowledge of the UE type. The MSGin5G Server does not have to resolve the delivery mechanism (i.e. UE type). The first bullet in 5.3.2.1 is modified accordingly Typo in next bullet. Change text in 5.3.2.2 that the Server will check for availability and reachability in its repository and not for the end-point Service ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possibly wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 5.3.2.1, 5.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | There is no difference between Rev 0 and Rev 1. |  |  |  |  |  |  |  |  |

* * First Change * *

### 4.2.1	Description

This subclause specifies the requirements for UE types supported by MSGin5G Service.

* * Next Change * *

#### 5.3.2.1	General functionalities

An MSGin5G Server provides server-side functionality to assist MSGin5G Clients with the sending and receiving of messages via the MSGin5G Service to/from Application Servers and/or other MSGin5G Service endpoints on other UEs, and collect charging information from MSGin5G message.

Functionalities of MSGin5G Server:

-	To deliver messages to an MSGin5G Service endpoint (which can be an MSGin5G UE, an Application Server or a Message Gateway) based on the terminating MSGin5G Service ID;

-	To resolve the MSGin5G Group Service ID to determine the members of the Group as specified in 3GPP TS 23.434[5];

-	Interworking with non 3GPP messaging service through the Non-3GPP Message Gateway;

-	Interworking with legacy 3GPP messaging service through the Legacy 3GPP Message Gateway;

-	Interworking with a 3GPP broadcast service through the Broadcast Message Gateway;

-	Exchanging MSGin5G messages with Application Servers, MSGin5G Clients, Legacy 3GPP Message Gateway, and Non-3GPP Message Gateway;

-	Supporting MSGin5G message segmentation according to service provider's policy;

-	Supporting UE configuration procedures as specified in TS 23.434 [5] or communicating with the SEAL Configuration Management Server to provide MSGin5G configuration data on a UE to be ready for the MSGin5G Service;

-	Managing information related to the MSGin5G Service, such as MSGin5G Client Triggering Information, MSGin5G Client availability and MSGin5G Client Supported Maximum MSGin5G segment size, and Broadcast Message Gateway;

-	Support store and forward of messages based on availability of the service endpoints, and

-	Interactions towards the CHF, as defined in TS 32.240 [17], to collect charging information from MSGin5G message according to the operator's charging policy and report charging information to CHF.

NOTE:	The details of charging function are out of scope of the present document.

#### 5.3.2.2	Target resolution

Upon receiving the MSGin5G message request to deliver the message to the recipient (which could be any of the MSGin5G UE, Legacy 3GPP UE, Non-3GPP UE or Application Server) on the terminating side, the MSGin5G Server checks the recipient's registration repository (created at the time of each MSGin5G UE/Application Server registration to MSGin5G Server, or the Message Gateway performs registration with the MSGin5G Server on behalf of the Non-MSGin5G UEs) for availability and reachability of MSGin5G service endpoints. The MSGin5G Server will attempt for delivery MSGin5G message request towards recipient based on the UE Service ID/AS Service ID and availability and reachability.

If the recipient is Non-MSGin5G UE, the Legacy 3GPP Message Gateway or the Non-3GPP Message Gateway that the Non-MSGin5G UE is registered with, will receive the MSGin5G message request on behalf of the Non-MSGin5G UE, and then delivers the message to the Non-MSGin5G UE by using the Non-MSGin5G message delivery mechanism.

If the message is a Broadcast message, the Broadcast Message Gateway will receive the MSGin5G message request and will deliver the message to all UEs in the Broadcast Area via the Broadcast messaging delivery mechanism, based on the Broadcast Area ID.

* * End of Changes * *