3GPP TSG-SA WG6 Meeting #51-e	S6-223590

Toulouse, France, 14th – 18th November 2022	(revision of S6-223393, 223098, 222865, 222362, 222059, 221871, 221520)

# Source:		InterDigital

# Title:		New Solution: Support for DAA

Spec:		3GPP TR 23.700-55

# Agenda Item:	9.9

# Document for:	Approval

Contact:		Atle Monrad (atle.monrad@interdigital.com)

1. Introduction

A standardised mechanism for support from the UAE-layer for DAA is proposed to secure that 3GPP has means that the UAVs has functionality within the UAE-layer to support the application for detect and avoid of potential interfering objects. It is proposed to add a new solution for the UAE-layer support of DAA in 3GPP TR 23.700-55 v 1.1.0.

* * * *   Start of changes   * * * *

## 7.2	Mapping of solutions to key issues

Table 7.2-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |
| Sol #2 |  | X |  |  |
| Sol #3 | X |  | X |  |
| Sol #4 |  |  |  | X |

* * * *   Next change   * * * *

## 7.6	Solution #4: UAE-layer support for DAA

### 7.6.1	Architecture enhancements

None.

### 7.6.2	Solution description

#### 7.6.2.1	General

This solution aims to address the gaps identified in Key Issue #4 "Support of detect and avoid services and applications".

The solution covers registration of the UAE clients' DAA capability to the UAE server, provisioning of the DAA policies from the UAS application specific server to the UAE server and the UAE client and support for DAA from the UAE-layer to the UAS application layer.

The DAA capability of the UAE client is provided to the network to inform the network that the UAE client support reception of DAA-policies from the network and has functionality in the UAE-layer to assist and coordinate support for DAA in the UE

A DAA policy has two components:

- The DAA application policy. This policy is used by the application and transparently provided from the UAS application specific server to the UAS client. The DAA application policy is timestamped and may be stored at the UAE server, to let the operator know when and optionally what information was provided to the UAS application.

- The DAA support policy. This policy is used by the UAE-layer, and contains information pertaining to the UAE-layer. This is parameters/rules for the UAE-layer to provide support for DAA applications.

NOTE:	The complete list of parameters for the DAA support policy will be specified during the normative work.

The Detect and Avoid operations performed by the UAS application layer is out of scope of the solution.

#### 7.6.2.2	Registration of DAA capability

Pre-conditions:

-	The UAE client has discovered the UAE server and is aware of the address of the UAE server (e.g., FQDN).

NOTE:	How the UAE client is provisioned with the UAE server information is outside the scope of the current document.

-	The UAV has already been assigned with the UAV ID.

Figure 7.6.2.2-1: Registration of DAA capability of the UAE client

1.	The UAE client sends a registration request to the UAE server. The UAE client includes an indication of its DAA capability.

2.	The UAE server performs authentication and authorization check (e.g., based on pre-provisioned security information or by interacting with UAS application specific server).

3.	The UAE server sends a registration response to the UAE client indicating success or failure of the registration for DAA support.

#### 7.6.2.3	Provision of DAA policies

##### 7.6.2.3.1	DAA support management procedure

Figure 7.6.2.3.1-1 illustrates the DAA support management procedure where the UAE server receives an application request for managing the DAA configuration parameters from the UAS application specific server.

Pre-condition:

-	The UAV has received its UAS ID from the UAS application specific server.

-	The UAV has performed the UAS UE registration procedure.

Figure 7.6.2.3.1-1: DAA support management procedure

1.	The UAS application specific server sends to the UAE server a DAA support management request. The request includes the UAV (UAE client) identifier and the DAA policies.

2.	The UAE server sends to the UAS application specific server a DAA support management response with a positive or negative acknowledgement of the request.

3.	The UAE server executes the DAA configuration according to clause 7.6.2.3.2.

4.	After successful execution of DAA configuration, the UAE server notifies the UAS application specific server with DAA support management complete.

##### 7.6.2.3.2	DAA support configuration procedure

Figure 7.6.2.3.2-1 illustrates the DAA support configuration procedure. This procedure enables the configuration of the UAE client, based on a request from UAS application specific server to configure DAA policies to the UAE client.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE server has established a UAE session with the respective UAE clients as the UAE clients are successfully registered to the UAE server.

3.	UAE server has performed the DAA support management procedure according to clause 7.6.2.2.2.

Figure 7.6.2.3.2-1: DAA support configuration procedure

1.	The UAE server sends a DAA support configuration request to the UAE client. The UAE client receives a DAA support configuration request from the UAE server that includes the DAA configuration parameters.

NOTE:	Details in case of e.g. removal of DAA policies will be specified during the normative work.

2.	The UAE client stores or removes the DAA configuration parameters as per the information received in step 1.

3.	The UAE client sends a DAA support configuration response to the UAE server.

#### 7.6.2.4	UAE-layer support for DAA applications

#### 7.6.2.4.1	Client initiated DAA support

The Figure 7.6.2.4.1-1 illustrates the procedure with client initiated DAA support.

Pre-conditions:

1.	UAE server has provided DAA policies to the UAE client.

Figure 7.6.2.4.1-1: Client initiated DAA support

0.	The UAE-layer has, e.g. based on the DAA support policy and/or information provided by the U2X layer (see 3GPP TR 23.700-58 [6]) detected UAVs in proximity.

1.	The UAE client sends a DAA support information (i.e. U2X layer detected information) to the UAE server indicating a detected flight path conflict with one or more UAVs in proximity.

If the UAE client considers an emergency situation (e.g., due to lack of response from the UAE server and/or UAS application specific server), the UAE client shall inform the application layer (i.e. UASC) based on the DAA support policy.

2.	The UAE server records the DAA support event with current timestamp. UAE server requests UAE client location information from the SEAL location services. The UAE server records the received location information with current timestamp. The UAE server sends the DAA support information to the UAS application specific server.

NOTE:	The UAE server needs to provide trusted and timely network based location information to the USS which can be used as critical input for USS to handle or record DAA situations. The USS can provide deconflicting instructions to the UAV based on provided location information or handle properly potential flight path deviation due to DAA that is deconflicted locally.

3.	The UAS application specific server provides a DAA support information acknowledgement to the UAE server. The UAS application specific server may include more information in the acknowledgement (e.g. other UAVs detected information by network).

4.	The UAE server provides a DAA support information acknowledgement to the UAE client, and the UAE client provides the application layer (i.e. UASC) with the consolidated information from the UAS application specific server.

##### 7.6.2.4.2	Server initiated DAA support

Figure 7.6.2.4.2-1 illustrates the procedure with UAS application server initiated DAA support.

Pre-conditions:

1.	UAS application specific server has provided DAA configuration parameters to the UAE client.

Figure 7.6.2.4.2-1: Server initiated DAA support

0.	The UAS application specific server has discovered a conflict related to DAA (e.g presence of other UAVs in proximity of the UAV), and will provide the UAE client with relevant information.

NOTE:	An example of such a conflict is that an UAV with U2X capabilities, see clause 7.6.2.4.1 step 0, provides information about objects in proximity to the UAS application specific server. The UAS application specific server can, based on this, e.g. provide information to one or more surrounding UAVs that does not have U2X capability.

1.	The UAS application specific server sends a DAA support information to the UAE server which includes information of other UAVs in the proximity of the UAV. The UAE server verifies that the request is authorized as described above before sending the DAA support information to the UAE client.

2.	The UAE server sends a DAA support information from the UAS application specific server to the UAE client. Coordination with Real-Time UAV connection status monitoring and location reporting is performed by the UAE server, see 3GPP TS 23.255 [3] clause 7.5 and 3GPP TS 23.434 [5], clause 9.3.

Further, UAE client provides the application layer with the consolidated information from the UAS application specific server.

3.	The UAE client sends to the UAE server a DAA support information acknowledge.

5.	The UAE server sends the DAA support information acknowledge to the UAS application specific server.

### 7.6.3	Solution evaluation

Key Issue #4 outlines the following to be investigated further with respect to the impact on the application layer functional model for UAS:

a)	Whether and how the UAE-layer and/or SEAL services can be enhanced to support DAA services and applications for collision avoidance considering the Stage 1 requirements.

b)	How the UAE-layer can support DAA scenarios where UAVs belong to multiple PLMNs.

This solution addresses the bullet a) and bullet  b) in Key Issue #4:

A summary of the UAE-layer capabilities are:

1)	Management for provison of DAA-policies from the UAS application specific server the UAS application client is provided by the UAE-layer.

2)	UAE-layer provides support for handling of DAA support information.

3)	Provision of support for DAA is PLMN-agnostic.

* * * *   Next change   * * * *

### 9.2.1	General

All the key issues, solutions and architecture enhancements specified in this technical report are listed in Table 9.2.1-1.

Table 9.2.1-1 provides a mapping of the key issues to the related solutions. It also indicates whether the solution requires enhancement to the Release-17 architecture and lists the dependencies on other working groups.

Table 9.2.1-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Architectural enhancement (clause reference) | Enhancements required | Dependency on other working groups |
| --- | --- | --- | --- | --- |
| KI #1 Direct communication between UAVs |  |  |  | SA2 |
| KI #2: Support for multi-USS deployments | Solution #1: Change of USS during flight NOTE | 7.3 | None | None |
|  | Solution #2: Support for USS re-mapping for a UAS | 7.4 | None | None |
| KI #3: Coordination between Uu and PC5 for direct UAV-to-UAV or UAV-to-UAV-C communication | Solution #3: Support for C2 direct mode feasibility reporting | 7.5 | None | SA2 |
| KI #4: Support of detect and avoid services and applications | Solution #4: UAE-layer support for DAA | 7.6 | None | None |
| NOTE: Change of DN/EDN to avoid disruption while in flight due to change of USS is not covered by this solution. |  |  |  |  |

* * * *   End of changes   * * * *