3GPP TSG-SA WG6 Meeting #52	S6-223604

Toulouse, France 14th – 18th November 2022	(revision of S6-223560_223408_223174)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0138 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EEC triggering via SMS over NAS to perform service provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | <2022-11-07> |
| Category: | B |  |  |  |  |  | Release: |  |  | <Rel-18> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#1 in FS_eEDGEAPP is concluded to address, in the normative work, service provisioning triggering via SMS over NAS (utilizing application triggering specified in 3GPP TS 23.501). In this regard, this CR proposes how to enable EEC and EES/ECS to enable such application triggering for the porcedures over EDGE-1 and EDGE-4. This approach does not require EEC to persistently keep the session with ECS or EES (e.g, for receiving the updated EDN configuration information). |  |  |  |  |  |  |  |  |
| Summary of change: |  | General text for EEC triggering to initiate the procedure over EDGE-1 and EDGE-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures over EDGE-1 and EDGE-4 cannot be triggered triggering via SMS over NAS is not supported in Rel18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.x	EEC triggering to initiate procedures over EDGE-1 or EDGE-4

8.x.1	General

The functional entities of the Edge Enabler Layer can utilize the Application Triggering services (or Device Triggering) provided by the 3GPP core network via NEF or SCEF (see clause 4.4.5 in 3GPP TS 23.501 [2], clause 4.13.2, 5.2.6.5.2 in TS 23.502 [3], and clause 4.5.1 TS 23.682 [17]) to enable for ECS or EES to trigger EEC to send request for service provisioning or EAS discovery, respectively.

The EEC indicates to the ECS or EES that Application Triggering or Device Triggering is supported. If a UE is registered to 5GC, the EEC hosted in the UE determines that the UE supports Application Triggering (e.g., SMS over NAS) by checking if the UE is capable of it and if the UE is allowed by the core network to use Application Triggering (e.g. whether the UE is indicated by the core network that SMS over NAS is allowed during Registration procedure or UE Configuration Update procedure as specified in TS 23.502 [3]).

Editor's Note:	It is FFS how the EEC checks if Device Triggering is available when the UE is attached to EPC.

When the EDN configuration information in the ECS is updated, if EEC triggering service is available for the ECS and EEC, the ECS triggers the EEC to initiate service provisioning request or EAS discovery request by invoking application triggering service as specified for Nnef_ Trigger_Delivery service to in clause 4.13.2 of TS 23.502 [3] or for Device triggering procedure via T8 in clause 5.17 of TS 23.682 [17]. Likewise, when the EAS profile in the ESS is updated, if the EEC triggering service is available for the EES and EEC, the EES triggers the EEC to initiate the EAS discovery request.

Editor's Note:	It is FFS how to describe the EEC triggering procedure and its API description according to the decision on either having it as a separate procedure or updating the existing procedure

* * * End of Change * * * *