3GPP TSG-SA WG6 Meeting #53	S6-230518

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0066 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMR-3GPP Location Interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a need to exchange location information between LMR systems and the 3GPP mission critical Location Management Server to support situational awareness among all users of those systems. All public safety users need to be able to know the location of other public safety users to increase the safety of all, regardless of whether they use 3GPP or LMR technology. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference point IWF-4 is defined between the LMS and the IWF. IWF-4 is defined to use the same protocols as CSC-22 defined in TS 23.280. Information flows and procedures over the IWF-4 reference point are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to exchange location information between LMR systems and 3GPP mission critical systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.2, (all the following are new) 7.4.4, 10.11.4, 10.11.4.1, 10.11.4.1.1, 10.11.4.1.2, 10.11.4.1.3, 10.11.4.1.4, 10.11.4.1.5, 10.11.4.1.6, 10.11.4.1.7, 10.11.4.2, 10.11.4.2.1, 10.11.4.2.1.1, 10.11.4.2.1.2, 10.11.4.2.2, 10.11.4.2.2.1, 10.11.4.2.2.2, 10.11.4.2.3, 10.11.4.2.3.1, 10.11.4.2.3.2, 10.11.4.2.4, 10.11.4.2.4.1, 10.11.4.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * FIRST CHANGE * * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

E2EE	End-to-End Encryption

IWF	InterWorking Function

KEK	Key Encryption Key (TETRA)

KMS	Key Management Service

MC	Mission Critical

MCPTT	Mission Critical Push To Talk

LMR	Land Mobile Radio

LMC	Location Management Client

LMS	Location Management Server

OTAK	Over-The-Air-Key Management (TETRA)

OTAR	Over-The-Air Rekeying (P25)

P25	Project 25

SDS	Short Data Service

TETRA	TErrestrial Trunked Radio

UE	User Equipment

UKEK	Unique Key Encryption Key (P25)

URI	Uniform Resource Identifier

* * * * * NEXT CHANGE * * * * *

## 7.2	Functional model description

Figure 7.2-1 shows the functional model for the application plane for interworking between MC systems and LMR systems. Functional entities and interfaces depicted on the right-hand side of the IWF-x interfaces are defined in 3GPP TS 23.280 [5], 3GPP TS 23.379 [7], and 3GPP TS 23.282 [6].

Figure 7.2-1: Functional model for application plane for interworking

* * * * * NEXT CHANGE * * * * *

### 7.4.4	Reference point IWF-4 (between the IWF and the LMS)

The IWF-4 reference point, which exists between the IWF and the LMS, provides location information exchange between an LMR system and the MC system. IWF-4 is based upon CSC-22, as defined in 3GPP TS 23.280 [5] with some differences, as specified in the present document.

* * * * * NEXT CHANGE * * * * *

### 10.11.4	Information flows for location information between the IWF and the LMS

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.1	Location information services between the IWF and the LMS

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.1.1	Location information report

#### Table 10.9.2.2-4 in 3GPP TS 23.280 [5] describes the information flow to support the handling of a location information report from the LMS to the IWF and from the IWF to the LMS.

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.1.2	Location information request

Table 10.9.2.3-4 in 3GPP TS 23.280 [5] describes the information flow to support the handling of a location information request from the LMS to the IWF and from the IWF to the LMS.

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.1.3	Location information subscription request

Table 10.9.2.5-3 in 3GPP TS 23.280 [5] describes the information flow from the LMS to the IWF and from the IWF to the LMS for a location information subscription request.

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.1.4	Location information subscription response

Table 10.9.2.6-1 in 3GPP TS 23.280 [5] describes the information flow from the LMS to the IWF and from the IWF to the LMS for a location information subscription response.

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.1.5	Location information notification

Table 10.9.2.7-3 in 3GPP TS 23.280 [5] describes the information flow from the LMS to the IWF and from the IWF to the LMS for a location information notification.

* * * * * NEXT CHANGE * * * * *

##### 10.11.4.1.6	Location information cancel subscription request

Table 10.9.2.8-3 in 3GPP TS 23.280 [5] describes the information flow from the LMS to the IWF and from the IWF to the LMS for a location information cancel subscription request.

* * * * * NEXT CHANGE * * * * *

##### 10.11.4.1.7	Location information cancel subscription response

Table 10.9.2.9-3 in 3GPP TS 23.280 [5] describes the information flow from the LMS to the IWF and from the IWF to the LMS for a location information cancel subscription response.

* * * * * NEXT CHANGE * * * * *

#### 10.11.4.2	Location information procedures between the IWF and the LMS

* * * * * NEXT CHANGE * * * * *

##### 10.11.4.2.1	On-demand request of location information procedure

##### 10.11.4.2.1.1	On-demand request of location information procedure (LMS to IWF)

The MC service server or location management client in the MC system can request an LMR user's location information, which is in the LMR system, at any time by sending a location information request to the IWF at the LMR system.

The LMR user appears to the MC system as an MC service user. The IWF provides interworking to obtain location information for the LMR user associated with the MC service identity it receives. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.1.1-1 illustrates the high level procedure of on-demand request of location information.

Figure 10.11.4.2.1.1-1: On-demand request of location information procedure

1.	The MC service server or a LMC in the MC system requests from the LMS on-demand location information of the LMR user that appears as an MC service user.

2.	The LMS in the MC system checks if the provided information along with the configuration permit the request to proceed.

NOTE 1:	Whether the authorization check is a specific MC service user based check or is a general policy check is outside the scope of this procedure.

3.	The LMS in the MC system determines that the request has a target in the LMR system.

4.	The LMS in the MC system sends the location information request to the IWF in the LMR system according to the described information flow in clause 10.11.4.1.2.

NOTE 2:	MC gateways can exist between the LMS in the MC system and the IWF in the LMR system. These are transparent to this procedure and are not shown in figure 10.11.4.2.1.1-1.

5.	The IWF in the LMR system can authorize the request.

6.	The IWF at the LMR system determines the location information for the LMR user associated with the MC service user identified in the request.

7.	The IWF at the LMR system sends the location information report, described in clause 10.11.4.1.1, to the LMS in the MC system. The LMS forwards the location information report to the MC service server or the location management client.

* * * * * NEXT CHANGE * * * * *

###### 10.11.4.2.1.2	On-demand request of location information procedure (IWF to LMS)

The IWF in the LMR system can request an MC service user's location information, which is in the MC system, at any time by sending a location information request to the LMS at the MC system. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.1.2-1 illustrates the high level procedure of on-demand request of location information.

Figure 10.11.4.2.1.2-1: On-demand request of location information procedure

1.	The IWF in the LMR system determines that location information is needed for an MC service user.

2.	The IWF in the LMR system determines that the MC service user is in the MC system.

3.	The IWF in the LMR system sends the location information request to the LMS in the MC system according to the described information flow in clause 10.11.4.1.2.

NOTE :	MC gateways can exist between the LMS in the MC system and the IWF in the LMR system. These are transparent to this procedure and are not shown in figure 10.11.4.2.1.2-1.

4.	The LMS in the MC system authorizes the request.

5.	The LMS in the MC system determines the location information for the MC service user identified in the request.

6.	The LMS in the MC system sends the location information report, described in clause 10.11.4.1.1, to the IWF in the LMR system.

* * * * * NEXT CHANGE * * * * *

##### 10.11.4.2.2	Location information notification procedure

##### 10.11.4.2.2.1	Location information notification procedure (IWF to LMS)

The IWF in the LMR system provides location information, based on some decision or event, to the LMS. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.2.1-1 illustrates the high level procedure of notification of location information.

Figure 10.11.4.2.2.1-1: Event-triggered location information notification procedure

1.	The IWF in the LMR system determines that it has location information available that is to be notified to the LMS in the MC system.

2.	The IWF in the LMR system sends the location information notification to the LMS in the MC system, according to the described information flow in clause 10.11.4.1.5.

NOTE :	MC gateways can exist between the LMS in the MC system and the IWF in the LMR system. These are transparent to this procedure and are not shown in figure 10.11.4.2.2.1-1.

* * * * * NEXT CHANGE * * * * *

###### 10.11.4.2.2.2	Location information notification procedure (LMS to IWF)

The LMS in the MC system provides location information to the IWF in the LMR system. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.2.2-1 illustrates the high level procedure of notification of location information.

Figure 10.11.4.2.2.2-1: Event-triggered location information notification procedure

1.	The LMS in the MC system determines that it has location information available that is to be notified to the IWF in the LMR system.

2.	The LMS in the MC system sends the location information notification to the IWF in the LMR system, according to the described information flow in clause 10.11.4.1.5.

NOTE :	MC gateways can exist between the LMS in the MC system and the IWF in the LMR system. These are transparent to this procedure and are not shown in figure 10.11.4.2.2.2-1.

* * * * * NEXT CHANGE * * * * *

##### 10.11.4.2.3	Location information subscription procedure

##### 10.11.4.2.3.1	Location information subscription procedure (LMS to IWF)

A location information subscription request is sent from the MC system to the IWF. The IWF in the LMR system sends a location information subscription response. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.3.1-1 illustrates the high level procedure of subscription to location information from the LMS in the MC system to the IWF in the LMR system.

Figure 10.11.4.2.3.1-1: Location information subscription procedure

1.	The MC service server or the LMC in the MC system sends a request for subscription to event-triggered location information of LMR users that appear as MC service users in the LMR system by sending a location information subscription request to the LMS in the MC system, according to the described information flows in clause 10.9.2.5 in 3GPP TS 23.280 [5].

2.	The LMS in the MC system checks if the provided information along with the configuration permit the request to proceed.

NOTE :	Whether the authorization check is a specific MC service user based check or is a general policy check is outside the scope of this procedure.

3.	The LMS in the MC system determines that the request has a target in an LMR system.

4.	The LMS in the MC system sends the location information subscription request to the IWF in the LMR system, according to the described information flow in clause 10.11.4.1.3.

5.	The IWF in the LMR system can check if the provided information along with the configuration permit the request to proceed.

6.	The IWF in the LMR system applies the subscription.

7.	The IWF in the LMR system sends the location information subscription response to the LMS in the MC system according to the described information flow in clause 10.11.4.1.4. The LMS in the MC system can respond to the LMC or MC server in the MC system per the procedures of 3GPP TS 23.280 [5].

* * * * * NEXT CHANGE * * * * *

###### 10.11.4.2.3.2	Location information subscription procedure (IWF to LMS)

A location information subscription request is sent from the MC system to the IWF. The IWF in the LMR system sends a the location information subscription response. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.3.2-1 illustrates the high level procedure of subscription to location information from the LMS in the MC system to the IWF in the LMR system.

Figure 10.11.4.2.3.2-1: Location information subscription procedure

1.	The IWF in the LMR system determines that it needs to subscribe to location information notifications for an MC service user.

2.	The IWF in the LMR system determines that the request has a target in the MC system.

3.	The IWF in the LMR system sends the location information subscription request to the LMS in the MC system, according to the described information flow in clause 10.11.4.1.3.

4.	The LMS in the MC system checks if the provided information along with the configuration permit the request to proceed.

5.	The LMS in the MC system applies the subscription.

6.	The LMS in the MC system sends the location information subscription response to the LMS in the MC system according to the described information flow in clause 10.11.4.1.4.

* * * * * NEXT CHANGE * * * * *

##### 10.11.4.2.4	Location information cancel subscription procedure

##### 10.11.4.2.4.1	Location information cancel subscription procedure (LMS to IWF)

The LMC in the MC system receives location information updates according to the subscriptions requested in the LMR system per clause 10.11.4.2.3. Those subscriptions can be cancelled anytime from the MC system. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.4.1-1 illustrates the high level procedure of the subscription cancellation to location information from the MC system to the LMR system.

Figure 10.11.4.2.4.1-1: Location information cancel subscription procedure

1.	The MC service server or the LMC in the MC system requests the cancellation of subscriptions to event-triggered location information of LMR users that appear as MC service users in the LMR system by sending location information cancel subscription requests to the LMS in the MC system, according to the described information flows in clause 10.9.2.8 in 3GPP TS 23.280 [5].

2.	The LMS in the MC system checks if the provided information along with the configuration permit the request to proceed.

NOTE :	Whether the authorization check is a specific MC service user based check or is a general policy check is outside the scope of this procedure.

3.	The LMS in the MC system determines that the request has a target in an LMR system.

4.	The LMS in the MC system sends the location information cancel subscription request to the IWF in the LMR system, according to the described information flow in clause 10.11.4.1.6.

5.	The IWF in the LMR system can check if the provided information along with the configuration permit the request to proceed.

6.	The IWF in the LMR system cancels the subscription.

7.	The IWF in the LMR system sends the location information cancel subscription response to the LMS in the MC system, according to the described information flow in clause 10.11.4.1.7. The LMS can forward the location information can subscription response per the procedures in 3GPP TS 23.280 [5].

* * * * * NEXT CHANGE * * * * *

###### 10.11.4.2.4.2	Location information cancel subscription procedure (IWF to LMS)

The IWF in the LMR system receives location information updates according to the subscriptions requested in the MC system per clause 10.11.4.2.3.2. Those subscriptions can be cancelled anytime from the MC system. The IWF can translate, as needed, between MC service identities and identities used within the LMR system.

Figure 10.11.4.2.4.2-1 illustrates the high level procedure of the subscription cancellation to location information from the LMR system to the MC system.

Figure 10.11.4.2.4.2-1: Location information cancel subscription procedure

1.	The IWF in the LMR system determines that a location information subscription is to be cancelled.

2.	The IWF in the LMR system determines that the request has a target in the MC system.

4.	The IWF in the LMR system sends the location information cancel subscription request to the LMS in the MC system, according to the described information flow in clause 10.11.4.1.6.

5.	The LMS in the MC system checks if the provided information along with the configuration permit the request to proceed.

6.	The LMS in the MC system cancels the subscription.

7.	The LMS in the MC system sends the location information cancel subscription response to the IWF in the LMR system, according to the described information flow in clause 10.11.4.1.7.

* * * * * END CHANGES * * * * *