3GPP TSG-SA WG6 Meeting #53	S6-230542

Athens, Greece, 27th February – 3rd March 2023	(revision of S6-23xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0225 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of port number to UE identifier API call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In LS response (S6-220505/S2-2302164) SA2 highlight that the Nnef_UEId_Get operation has been enhanced to accept a NAT’ed IP address as input in cases that the UPF has access to the NAT. The enhancement requires that the AF provides the port number associated with the provided IP address. However, this requirement is not reflected in the EDGEAPP UE identifier API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Building on S6-230494, CR156rev2, as a baseline, this CR proposes to highlight that the EAS should include a Port Number associated with a UE’s IP address if it recognize the address received is a public IP address when calling the UE identifier API. The text attempts to align with that provided in agreed contribution S2-2301393 (23.502 CR 3666). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user information provided as input to the UE identifier API procedure will not be adequate for the EES to successfully call the Nnef_UEId_Get operation in case the IP address provided to the EES by the EAS is NAT’ed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5.2 & 8.6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to KI#16/Sol#23 in TR 23.700-98. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 8.6.5	UE Identifier API

#### 8.6.5.1	General

EES exposes UE Identifier API to the EAS and EEC in order to provide an identifier uniquely identifying a UE. This API is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. hasn’t already cached). This identifier, called UE ID, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 or EDGE-7. The UE ID is specific to the given EAS and it is represented as a GPSI assigned by the 3GPP Core Network.

Editor's note:	[SA3] [SA2] Check pre-condition #3 to ensure EES is Ok to pass on EAS ID in NEF’s Nnef_UEId as opposed to its own AF ID.

#### 8.6.5.2	Procedure

Figure 8.6.5.2-1 illustrates the interactions between the EES and the EAS.

Pre-conditions:

1.	The EAS or EEC is authorized to discover and to use UE Identifier API provided by the EES.

2.	When the EEC is used to invoke the UE Identifier API with the UE IPv6 address as the input parameter, the UE IPv6 address may or may not be NATed. If NATed however, the IPv6 may not be reused (i.e. assigned to more than one UE simultaneously).

3.	EAS is considered an AF behind EES (as another AF) and EES is authorized to pass EAS ID instead of its own AF ID when it needs to interact with the NEF’s Nnef_UEId_Get (as per TS 23.502 clause 4.15.10 “AF specific UE ID retrieval”).

Editor's note:	[SA3] [SA2] Check pre-condition #3 to ensure EES is Ok to pass on EAS ID in NEF’s Nnef_UEId as opposed to its own AF ID.

Editor's note:	It is FFS whether we need to split this Procedure, into two: the interaction between the EEC with the EES, and the EAS with the EES.

Figure 8.6.5.2-1: UE Identifier API

1.	The EAS or EEC invokes UE Identifier API exposed by the EES. If it is the EAS invoking the API and it recognizes that the UE’s IP address is a public IP address, i.e., the UE is behind a NAT, the Port Number and associated IP address should be included in user information.

2.	The EES uses the received user information in the step 1 (e.g. IP address) and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3].

3.	The EES provides the obtained UE identifier as UE ID to the EAS or to EEC (i.e. whichever invoked the API).

Editor's note:	[SA3] Whether and how user's consent is obtained to share the UE identifier with a particular EAS or EEC is SA3's responsibility.

4.	The EAS uses the UE ID received in step 3 to invoke capability exposure API(s) provided by the EES over EDGE-3.

Editor's Note: It is FFS if to find out if there is a need to provide further details on the UE Identifier API procedure when the API is used by EEC? (details such as, the use case where EEC uses this API, the triggers for the EEC invoking the API and the necessity for the usage of UE’s IPv6 address in calling the API, etc)?

#### 8.6.5.3	Information flows

##### 8.6.5.3.1	General

The following information flows are specified for UE Identifier API:

-	UE Identifier request and response.

##### 8.6.5.3.2	UE Identifier API request

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| User information (see NOTE) | M | Information about the User or UE available in the EAS or EEC, e.g. IP address. |
| EAS ID list | M | Identifier of the EAS(s) (aka AFId(s)) for which the UE IDs are requested for by EAS or EEC given the User information (e.g. IP address). |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Security Credentials | M | Security credentials of the EAS or EEC. |
| NOTE: When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. When EAS invokes the API, it may recognize the UE IP address is a public IP address different from the actual UE IP address (private IP address), i.e., the UE is behind a NAT, and should therefore include the Port Number and associated IP address as part of the User information. |  |  |

Editor's Note: [SA3] Whether the EAS ID and the EAS Provider ID are part of the security credential is SA3's responsibility.

Editor's Note: Support for obtaining UE ID when IPv6 addresses are NATed (and how to overcome IP address overlap issues) is FFS and requires coordination with SA2.

Editor's Note: [SA3] It is FFS whether the OS can verify the IP address (i.e. User information) provided in the request is indeed the UE’s and whether the UE provided infomation (IP Address) can be trusted.

Editor's Note: It is FFS to determine if EASID (clause 7.2.4) definitions need to be updated in order to be consistent with its use (as per EAS ID list IE) as an AF ID in UE Identifier API.

Editor's Note: The use of Edge UE ID and the defintion of Edge UE ID when the API is used by EEC is FFS.

##### 8.6.5.3.3	UE Identifier API response

Table 8.6.5.3.3-1: UE Identifier API response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the UE identifier request was successful. |
| > UE ID list | O | List of all the AF-specific UE IDs uniquely identifying the UE(s). |
| >> UE ID | M | AF-specific UE ID (i.e. EAS-specific UE ID) |
| >> EAS ID | M | Associated EAS ID |
| Failure response | O | Indicates that the UE identifier request failed. |
| > Cause | O | Indicates the cause of UE identifier request failure |