3GPP TSG-SA WG6 Meeting #53	S6-230543

Athens, Greece, 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0226 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of UE ID Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Nnef_UEId_Get operation returns an AF specific UE identifier in the form of an external identifier. The UE Identifier API of clause 8.6.5 indicates that the UE ID is a GPSI, which is appropriate since the UE ID may also be provided by an EEC or EAS (i.e., the NEF is not the only UE ID source in the EDGEAPP architecture). A GPSI can be in the form of an External Identifier or MSISDN. However, clarification is required that the GPSI returned by the UE Identifier API will only be in the form of an External Identifier and that identifier can be used on both EDGE-3 and EDGE-7. Furthermore, clarification is required that the EASID is used as the AF Identifier when invoking the Nnef_UEId_Get operation (with text aligned with CR0156r2 Rel-18 S6-230494 for EAS ID description). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarifying text to UE Identifier API clause to highlight that the UE ID return type is a GPSI in the form of an AF specific UE identifier and that the EAS is able to use the UE ID as the UE identifier over EDGE-7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification lacking in clarification could lead to erroneous implementations, giving the impression that the UE Identifier API could return a MSISDN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5.1, 8.6.5.2, 8.6.5.3.2, 8.6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.6.5	UE Identifier API

#### 8.6.5.1	General

EES exposes UE Identifier API to the EAS in order to provide an identifier uniquely identifying a UE. This API is used by an EAS to obtain the identifier of the UE if the EAS does not have it. This identifier, called UE ID, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 or EDGE-7. The UE ID is specific to the given EAS and it is represented as a GPSI (in the form of an External Identifier) assigned by the 3GPP Core Network as an AF specific UE Identifier.

#### 8.6.5.2	Procedure

Figure 8.6.5.2-1 illustrates the interactions between the EES and the EAS.

Pre-conditions:

- The EAS is authorized to discover and to use UE Identifier API provided by the EES.

Figure 8.6.5.2-1: UE Identifier API

1.	The EAS invokes UE Identifier API exposed by the EES.

2.	The EES uses the received user information in the step 1 (e.g. IP address) and obtains the AF specific UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3].

3.	The EES provides the obtained AF specific UE identifier as UE ID to the EAS.

Editor's note:	[SA3] Whether and how user's consent is obtained to share the AF specific UE identifier with a particular EAS is SA3's responsibility.

4.	The EAS uses the UE ID received in step 3 to invoke capability exposure API(s) provided by the EES over EDGE-3 or EDGE-7.

#### 8.6.5.3	Information flows

##### 8.6.5.3.1	General

The following information flows are specified for UE Identifier API:

-	UE Identifier request and response.

##### 8.6.5.3.2	UE Identifier API request

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| User information | M | Information about the User or UE available in the EAS, e.g. IP address. |
| EAS ID | M | Identifier of the EAS(aka AF Identifier) for which the UE IDs are requested for by EAS given the User information (e.g. IP address). |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Security Credentials | M | Security credentials of the EAS. |

Editor's Note: [SA3] Whether the EAS ID and the EAS Provider ID are part of the security credential is SA3's responsibility.

##### 8.6.5.3.3	UE Identifier API response

Table 8.6.5.3.3-2: UE Identifier API response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the UE identifier request was successful. |
| > UE ID | M | AF specific UE Identifier uniquely identifying the UE, represented as a GPSI in the form of an External Identifier. |
| Failure response | O | Indicates that the UE identifier request failed. |
| > Cause | O | Indicates the cause of UE identifier request failure |