3GPP TSG-SA WG6 Meeting #53	S6-230550

Athens, Greece 27th February – 3rd March	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0169 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Procedure and information flow to add location QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion of KI#2 in TR 23.700-96, the solution#7 will be considered in the normative phase. According to the sol#7, the LMS enhanced with FLF could select one or more access types or location methods for the target UE based on the requested location QoS. And according to TS23.273, “It is optional for LCS client or the AF to provide the LCS Quality of Service in the location request”. So the related procedure and information flow will be update in TS 23.434 to support solution#7 to add the requested location QoS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To update the procedure and information flow to add Location QoS according to the conclusion of Sol#7 in TR 23.700-96. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G-enabled fused location service capability will not support LCS QoS for SEAL location management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,9.3.2.3,9.3.2.5,9.3.4,9.3.7,9.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[x]                        3GPP TS 29.572: “5G System; Location Management Services; Stage 3”

* * * Next Change * * * *

9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Location QoS | O | Definition of the location Quality of Service for which the location information is requested (see NOTE). |
| NOTE: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [y] and the clause 6.1.6.2.13 of TS 29.572 [x]. |  |  |

Editor's Note: It's FFS whether and how the LMS need to identify the VAL service when the VAL UE ID is used for location request.

Editor's Note: It's FFS the security aspects for LM-Uu and LM-S in relation to the VAL service ID that need to be coordinated with SA3.

* * * Next Change * * * *

9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server or location management client to the location management server for location information subscription request.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Location QoS | O | Definition of the location Quality of Service for which the location information is requested (see NOTE). |
| NOTE: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [y] and the clause 6.1.6.2.13 of TS 29.572 [x]. |  |  |

* * * Next Change * * * *

9.3.4	On-demand location reporting procedure

The location management server can request UE location information at any time by sending a location information request to the location management client, which may trigger location management client to immediately send the location report.

Figure 9.3.4-1: On-demand location information reporting procedure

1.	Based on configurations such as periodical location information timer, or location information request from other entities (e.g., another location management client, VAL server), location management server initiates the immediately request location information from the location management client.

2.	The location management server sends a location information request to the location management client which may include the location QoS that contains the location accuracy, response Time and QoS class as defined in clause 4.1b of  TS 23.273[y].

3.	VAL user or VAL UE is notified and asked about the permission to share its location. VAL user can accept or deny the request

4.	The location management client immediately responds to the location management server with a report containing location information identified by the location management server and available to the location management client. The location management client may retrieve the available and proper access type and positioning methods for the target VAL UE based on the received location QoS, and send the location information report to the location management server with the selected access type.

5.	Upon receiving the report, the location management server checks if the obtained location report of the VAL UE could meet the location QoS requirements (if any) and then updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

* * * Next Change * * * *

9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for intiating tracking a UE's location.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information and the location QoS which contains the location accuracy, response Time and QoS class as defined in clause 4.1b of  TS 23.273[y].

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request and may retrieve the available and proper access type and positioning methods for the target VAL UE based on the received location QoS. Further, the location management server may initiate location reporting configuration optionally including the retrieved access type and positioning methods with the location management client of the UE for immediate reporting.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE with the retrieved access type and positioning methods in step 2. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network.

4.	The location management server determines the UE location information of the UE as received in steps 3 and 4 and checks if it meets the location QoS requirements (if any) or not.

5.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

* * * Next Change * * * *

9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Figure 9.3.9-1: On-demand usage of location information procedure

1.	VAL server sends a location information request to the location management server which may include the location QoS that contains the location accuracy, response Time and QoS class as defined in clause 4.1b of  TS 23.273[y].

2.	The location management server acquires the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator, and checks if the obtained location of the VAL UE could meet the location QoS requirements (if any).

3.	Then, location management server immediately sends the location information report including the latest location information acquired of one or more VAL users or VAL UEs.

4.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 4 can be skipped if not needed.

* * * End of Changes * * * *