3GPP TSG-SA WG6 Meeting #53	S6-230565

Athens, Greece Feb. 27th – Mar. 03rd 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0230 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Explaining usage of EES’s capability exposure by EEC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently clause 8.6 “EES capability exposure to EAS” title is worded such that the EES exposed service can only be used to EAS. However, as part of the agreed S6-230494 in #52-e Bis, EEC also became a new client to the EES’s exposed services (i.e. “UE Identifier API”). As a result, the title of clause 8.6 is updated to also indicate that EEC may also use EES exposed capabilities. And hence resolving the following Editor’s Note from S6-230494 (see its new revision S6-230563): Editor's note: It is FFS whether we need to split this Procedure, into two: the interaction between the EEC with the EES, and the EAS with the EES. Additionally, more details regarding the invocation of “UE Identifier API” by EEC is provided in clause 8.6.5 in order to resolve the following Editor’s Note from S6-230494 (see its new revision S6-230563): Editor's Note: It is FFS to find out if there is a need to provide further details on the UE Identifier API procedure when the API is used by EEC? (details such as, the use case where EEC uses this API, the triggers for the EEC invoking the API and the necessity for the usage of UE’s IPv6 address in calling the API, etc)? The changes to clause 8.6.5 by this CR are in addition to the changes provided in the agreed S6-230494. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 8.6 “EES capability exposure to EAS” title is updated to include EEC as another client of EES’s exposed service. - Clause 8.6.5 provides details regarding the invocation of “UE Identifier API” by EEC. - Table 6.7.2-2 (in clause 6.7.2) is updated to include EEC as another client of EES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec won’t be clear to the reader. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.6 and 8.6.5 (note: The changes to clause 8.6.5 is in addition to the changes which have already been provided in the agreed S6-230494) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Changes to clause 8.6.5 in this CR are in addition to the changes which have already been agreed upon in S6-230494. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

## 8.6	EES capability exposure to EAS and EEC

### 8.6.1	General

This clause describes service capability APIs exposed by the EES to the EAS(s) and EEC(s). The service capability APIs exposed include EES capabilities and exposed 3GPP Core Network capabilities. The 3GPP Core Network capabilities may be exposed from EES to the EAS(s) and also to the EEC(s).

The 3GPP Core Network capabilities APIs which are enhanced for exposure are also specified in this clause.

NOTE:	When the ACR occurs, the new EAS can re-subscribe the EES capability exposure services on the T-EES.

# * * * Change 2 * * * *

### 8.6.5	UE Identifier API

#### 8.6.5.1	General

EES exposes UE Identifier API to the EAS in order to provide an identifier uniquely identifying a UE. This API is used by an EAS to obtain the identifier of the UE if the EAS does not have it. This identifier, called UE ID, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3. The UE ID is specific to the given EAS and it is represented as a GPSI assigned by the 3GPP Core Network.

The EAS’s direct invocation of the UE Identifier API of the EES may result in UE ID not found response (e.g. if the UE’s private IPv4 address is NATed by the core network). Under such circumstances, the EAS may choose to signal its AC to trigger the UE ID query onto the EEC over EDGE-5 (see clause 8.14.2.x). In turn, the EEC would invoke the EES’s UE Identifier API using the UE’s CN assigned IP addresses (i.e. IPv4 and/or IPv6) which should result in return of the UE ID to the EEC and from thereon to the AC and the EAS.

NOTE 1:	To overcome CN UE’s assigned private IP address reuse issue (e.g UE’s IPv4 reuse by 5GC), the EES would either need to be perconfigured with the public IP address range (used by the NAT function over N6) and its associated IP domain or the UPF would need to support NAT function.

NOTE 2:	EEC retrieval of the UE’s IP address from the device is out of scope.

# * * * Change 3 * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES | 8.9.4.3 |
| Eees_SelectedTargetEAS | EAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS | 8.8.3.8 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

# * * * * End of Changes * * * *