3GPP TSG-SA WG6 Meeting #53	S6-230604

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0234 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR procedure btw EAS and CAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedures with CES to support CAS is transposed from TR to the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the procedure for CAS with CES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No procedure support for ACR btw. CAS and EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8, 8.XX (new), 8.XX.1 (new), 8.XX.2 (new), .8.XX.2.1 (new), 8.XX.2.2 (new), 8.XX.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#11, Sol#24. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.8	Service continuity between EASs

## 8.XX	Service continuity between CAS and EAS

Editor's note:	Service continuity btw. CASs (due to load re-balancing) will be addressed later.

### 8.XX.1	General

Service continuity between CAS and EAS can be supported with CES or without CES, corresponding to the architecture options described in clause 6.DD and 6.EE.

Both options share the same EDGEAPP application server discovery/service provisioning procedure when trying to discover appropriate application server. If there is CES available and registered in the ECS in the network to provide enabling service for CAS, the EDGEAPP mechanism returns CES in service provisioning response to the EEC/S-EES or the S-EAS obtains CAS from CES via S-EES based on the request from EEC/S-EES or the S-EAS to initiate ACR to Cloud and EDGEAPP mechanism follows; otherwise, regular DNS is used to find CAS due to EDGEAPP mechanism failure (i.e. no enabling service available in the target area).

### 8.XX.2	Service continuity with CES

#### 8.XX.2.1	ACR Scenarios

Figure 8.XX.2-1: UE moves from one EDN to Cloud DN then to another EDN

As depicted in figure 8.XX.2-1, since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to communicate with the CAS.

CES(s) are registered in the ECS via ECI-3 reference point so that appropriate CES can be found during service provisioning procedure. CAS(s) are registered in the CES via CLOUD-1 reference point which enables the CES to provide appropriated CAS to the EEC or S-EAS.

The EES of the source EDN may interact with the CES via ECI-4 reference point and application context is transferred from source EAS to the CAS. Later, if the UE is moving to an area with edge coverage, the CES may interact with the EES in the target EDN via ECI-4 reference point and application context is transferred from the CAS to target EAS.

For UE moving from EDN to Cloud DN, the ACR scenarios are similar to those ACR scenarios in clause 8.8.2 with the following differences:

-	the CES replaces the T-EES; and

-	the CAS replaces the T-EAS.

For UE moving from Could DN to EDN, the ACR scenario are similar to those ACR scenarios in clause 8.8.2 with the following differences:

-	the CES replaces the S-EES; and

-	the CAS replaces the S-EAS.

##### 8.XX.2.2	Procedures

Procedures defined for EAS and EES in this specification (except for specific procedures to support the other option without CES support) are applicable for CAS and CES. For instance, the CAS and the CES registration procedure are similar to the EAS registration procedure as described in clause 8.4.3 and the EES registration procedure as described in clause 8.4.4 without service area restriction.

##### 8.XX.2.3	APIs

The CES APIs can re-use the APIs defined for EES. The CES can be seen as a special EES without service area restriction.

* * * END of Change * * * *