3GPP TSG-SA WG6 Meeting #53	S6-230605

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0235 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS discovery in edge node sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | GSMA OPG LS S6-230506 confirms the edge node sharing requirements. In GSMA’s view, the UE of leading OP is not aware of the OP partner and the EAS is only used to serve the UE of leading OP. The solution #43 in TR 23.700-98 is transposed to TS in this CR with necessary modifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The architecture to support edge node sharing is added. 2. The deployed application information in partner OP is shared to leading OP (including publish/unpublish and fetch operations). 3. The EAS discovery procedure is enhanced for edge node sharing considering cases w/ and w/o published application information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for edge node sharing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6F (new), 8.X (new), 8.X.1 (new), 8.X.2 (new), 8.X.2.1 (new), 8.X.2.2 (new), 8.X.2.3 (new), 8.X.2.4 (new), 8.X.3 (new), 8.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#22, Sol#43 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 6F	Architecture in edge node sharing

The application layer architecture addresses how to discover an EAS that is deployed in the partner’s OP (i.e. OP-A) in edge node sharing case and is based on architecture enhancement in EDGEAPP with ECS-ER (Edge Repository).

Figure 6F-1 shows two operator platforms can communicate with each other via EDGE-9 reference point and EDGE-18 reference point. EEC only communicates with EES and ECS of OP-B via EDGE-1 reference point and EDGE-4 reference point, respectively. EAS is registered in EES of OP-A via EDGE-3 reference point.

NOTE:	EAS is deployed by OP-B in Edge hosting environment of OP-A, it can only be used to serve operator B’s subscribers.

Figure 6F-1: EDGEAPP architecture in edge node sharing

## 8.X	EAS discovery in edge node sharing

### 8.X.1	General

In a region, an operator may not deploy EASs in its edge hosting environment everywhere. When a UE, which is a subscriber of the operator, moves to a location in a region and the AC in the UE needs to connect to an EAS, the operator can facilitate the UE to find appropriate EAS(s) deployed in its partner’s edge hosting environment.

In the following clauses, the OP-A is a partner of OP-B and OP-B is the leading OP to serve the UE.

It is assumed that the OP-B’s EESs are deployed everywhere in a region and ECS-ER of OP-A can share EAS information and associated EES information with ECS-ER of OP-B.

### 8.X.2	Procedures

#### 8.X.2.1	General

Since the EAS is deployed in the partner’s data network, when the leading OP (OP-B) receives a request from the EEC, the leading OP (OP-B) needs to contact the partner OP (OP-A) to discover the EAS. This clause provides ways for the leading OP to discover adequate EES(s) of the partner OP for subsequent communication with the partner OP’s EES.

#### 8.X.2.2	Publish/unpublish and fetch application

Figure 8.X.2.2-1: publish and unpublish application information between ECS via ECS-ER

In step 1, the EES of OP-A is registered in ECS of OP-A over EDGE-6 reference point. Based on the information sharing policy in the ECS of OP-A, the ECS of OP-A sends Application info publish request with a list of EAS IDs and optionally EES information of OP-A in step 2. The EES information of OP-A includes EES endpoint and may include EES provider ID, EES service area and/or EES Service continuity support. Then ECS of OP-B stores the received information and responds with Application info publish response in step 3. Step 2 and 3 are sent via corresponding ECS-ERs (ECS-ER (OP-A) and ECS-ER (OP-B)).

The ECS of OP-A may receive EES registration update from the EES of OP-A in step 4, then the ECS of OP-A updates the previously published information (EAS ID list and/or EES info of OP-A) towards the ECS of OP-B in step 5. The ECS of OP-B updates the previously stored information and responds with Application info publish update response in step 6. Step 5 and 6 are sent via corresponding ECS-ERs (ECS-ER (OP-A) and ECS-ER (OP-B)).

The ECS of OP-A may receive EES de-registration from the EES of OP-A in step 7, then the ECS of OP-A requests to remove all previously published information towards the ECS of OP-B in step 8. The ECS of OP-B removes all previously stored information and responds with Application info unpublish response in step 9. Step 8 and 9 are sent via corresponding ECS-ERs (ECS-ER (OP-A) and ECS-ER (OP-B)).

NOTE 1:	The application information published/shared to OP-B can also be done in a notification message under subscribe-notify communication model.

Figure 8.X.2.2-2: Fetch application information from partner

The ECS (OP-B) may fetch application information from its partner OP (e.g. OP-A) via corresponding ECS-ERs (ECS-ER (OP-A) and ECS-ER (OP-B)) as shown in figure 8.X.2.2-2, periodically. In such a fetch operation, the fetched information includes a list of EAS IDs and EES information of OP-A.

NOTE 2:	If the ECS (OP-B) does not receive EES information of OP-A from the published/notified application information, the ECS (OP-B) can also fetch it from the ECS (OP-A) via the fetch operation.

NOTE 3:	ECS (OP-B) can also subscribe to application info changes from ECS (OP-A).

Editor's note:	The relationship between this procedure for sharing application information in OP and procedure via OAM is FFS.

#### 8.X.2.3	EAS discovery without published application info

This procedure is applicable for EAS discovery without published application information between ECSs as described in clause 8.X.2.2.

Figure 8.X.2.3-1: EAS discovery for edge node sharing, without published application info

In step 0, the EAS may be registered in EES of OP-A. The EAS profile includes the allowed MNO name indicating the allowed operator from which its subscriber can consume the EAS services.

NOTE 1:	The EES (OP-A) also registers into the ECS (OP-A), which is not shown for simplicity.

In step 1, the EEC sends service provisioning request to the ECS (OP-B).

In step 2, the ECS (OP-B) cannot find any desired EAS ID in the EES profile being registered so it determines to use edge service from its partner OP-A.

In step 3, ECS (OP-B) discovers T-EES from ECS (OP-A).

NOTE 2:	The ECS (OP-B) can have SLA with other multiple OPs and need to repeat step 3 with more than one partner OP in order to discover T-EES.

In step 4, the service provisioning response includes the EES (OP-A) endpoint and the ECS (OP-B) determined EES (OP-B) endpoint. The endpoint of EES (OP-B) is determined based on SLA with OP-A so that the EES (OP-B) can be authorized by the EES (OP-A). The endpoint of EES (OP-A) is sent by EES in a way that is transparent to EEC.

In step 5, the EEC sends EAS discovery request to EES (OP-B) and include the endpoint of EES (OP-A) in the request. The endpoint of EES (OP-A) is transferred between the ECS (OP-B) and the EES (OP-B) transparently to the EEC.

NOTE 3:	How the EES (OP-A) endpoint is transferred in a transparent way to EEC between the ECS (OP-B) and the EES (OP-B) is in the scope of SA3.

In step 6, the EES (OP-B) validates the edge service SLA and sends EAS discovery request to the EES (OP-A). The EAS discovery request includes the required MNO name indicating the name of operator B.

In step 7, the EES (OP-A) validates the edge service SLA and returns EAS discovery response including the discovered candidate EAS(s) to the EES (OP-B). The candidate EAS(s) are discovered based on the required MNO name received from the EES (OP-B) and allowed MNO name registered by the EAS.

In step 8, the EEC receives the EAS discovery response sent by the EES (OP-B). The EEC may select an EAS for the AC.

#### 8.X.2.4	EAS discovery with published application info

This procedure is applicable for EAS discovery with published application information between ECSs as described in clause 8.X.2.2.

Figure 8.X.2.4-1: EAS discovery for edge node sharing, with published application info

In step 0, the EAS may be registered in EES of OP-A. The EAS profile includes the allowed MNO name indicating the allowed operator from which its subscriber can consume the EAS services.

NOTE 1:	The EES (OP-A) also registers into the ECS (OP-A), which is not shown for simplicity.

In step 1, the EEC sends service provisioning request to the ECS (OP-B).

In step 2, the ECS (OP-B) cannot find any desired EAS ID in the EES profile being registered so it determines to use edge node sharing service from its partner OP-A based on its partner’s published application information.

In step 3, the service provisioning response includes the ECS (OP-B) determined EES (OP-B) endpoint. The endpoint of EES (OP-B) is determined based on SLA with OP-A so that the EES (OP-B) can be authorized by the EES (OP-A).

In step 4, the EEC sends EAS discovery request to EES (OP-B).

In step 5, the EES (OP-B) executes for EAS discovery but cannot find any desired EAS so it determines to use edge node sharing service based on edge service SLA.

In step 6, the EES (OP-B) sends Retrieve EES request to the ECS (OP-B). The request may include an edge node sharing indication indicating edge node sharing is requested so that the ECS (OP-B) can skip checking T-EES(s) registered in the ECS (OP-B).

For the received EAS ID, since the ECS (OP-B) has EES information that was published by partner OP, the ECS (OP-B) discovers T-EES (i.e. EES(s) from partner) in step 7 and returns discovered EES(s) of OP-A in the Retrieve EES response to the ECS (OP-B) in step 8.

In step 9 and 10, the EAS discovery procedure happens, which is triggered by the EES (OP-B). The EAS discovery request includes the required MNO name indicating the name of operator B. The EES (OP-B) receives the discovered candidate EAS(s). The candidate EAS(s) are discovered based on the required MNO name received from the EES (OP-B) and allowed MNO name registered by the EAS.

NOTE 4:	The OP-B can have SLA with other multiple OPs so that step 9 and step 10 can be executed with more than one partner OP in order to discover candidate EAS(s).

In step 11, the EEC receives the EAS discovery response sent by the EES (OP-B). The EEC may select an EAS for the AC.

### 8.X.3	Information flows

### 8.X.4	APIs

* * * END of Change * * * *