3GPP TSG-SA WG6 Meeting #53-e	S6-230628

Athens, Greece 27th February – 3rd March 2023	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0108 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization revocation by resource owner |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CAPIF needs to support authorization revocation by the resource owner. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause (8.X) is added to specify the revocation procedure and information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorization revocation by the resource owner will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.X (new), 8.X.1 (new), 8.X.2 (new), 8.X.2.1 (new), 8.X.2.2 (new), 8.X.2.3 (new), 8.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.X	Authorization revocation by resource owner

8.X.1	General

The resource owner may revoke the authorization of the API invoker for accessing the service APIs which was given previously. When the authorization function receives the authorization revocation request from the resource owner client, the authorization function invalidates the authorization information, and the API invoker cannot invoke the service APIs with the same authorization information anymore. If the resource revocation requires recovery of certain resource (e.g., the resource owner requests to revoke the session with QoS API and the QoS is expected to be downgraded), the API exposing function may interact with the 3GPP core network to modify the resource.

8.X.2	Information flows

8.X.2.1	Authorization revocation request

Table 8.X.2.1-1 describes the information flow, authorization revocation request, from the resource owner client to the authorization function.

Table 8.X.2.1-1: Authorization revocation request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |

8.X.2.2	Authorization revocation response

Table 8.X.2.2-1 describes the information flow, authorization revocation response, from the authorization function to the resource owner client.

Table 8.X.2.2-1: Authorization revocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of authorization revocation request. |

8.X.2.3	Authorization revocation notify

Table 8.X.2.3-1 describes the information flow, authorization revocation notify, from the authorization function to the API exposing function.

Table 8.X.2.3-1: Authorization revocation notify

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker whose authorizatio has been revoked |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |

8.X.3		Procedure

Figure 8.X.3-1 illustrates the procedure for the resource owner client to revoke the authorization given to the API invoker.

Pre-conditions:

1.	The resource owner client can communicate with the authorization function.

Figure 8.32.3-1: Procedure for obtaining authorization information in a nested API invocation

1.	The resource owner client sends the authorization revocation request to the authorization function with the details of the API invoker and the service API.

2.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the authorization function invalidates the authorization corresponding to the service API.

3.	The authorization function sends the authorization revocation response to the resource owner client.

4.	The authorization function sends the authorization revocation notify to the API exposing function with the details of the API invoker and the service API.

5.	Upon receiving the authorization revocation notify, the API exposing function judge whether it is necessary to undo the resource modification (e.g., to revert the QoS level)

6.	If necessary, the API exposing function may interact with the 3GPP core network to recover the resource.