3GPP TSG-SA WG6 Meeting #53	S6-230637

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0175 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with 24.544 in the VAL group creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2023-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 Work Group implemented the stage 2 requirements (see clause 10.3.3 of 3GPP TS 23.434) for the group creation via the introduction of the membership type for each provided VAL UE ID in the group creation request. The CT1 implementation (see clause 6.2.5.1.1 of 24.544 in Release-16 clause 6.2.2.4 of 24.544 in Release-17) defines the three membership types: - administrator (the VAL UE has special authorities to manage the group. The administrator is not required to perform explicit registration to join the group); - explicit (the VAL UE is not an administrator of the group, and the group management client shall provide the group registration request to join the group); and - implicit (the VAL UE is not an administrator of the group, and the group management client is not required to provide the group registration request to join the group). The implemented mechanism by CT1 requires the correction in the group creation request information flow (clause 10.3.2.1). Thus, the alignment of 23.434 with 24.544 in the group creation is needed to keep the alignment between stage 2 and stage 3 specifications. Also, the group configuration procedure defined in clause 10.3.8.2 needs to be aligned with the terminology in clause 10.3.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The membership type information element is added to the group creation procedure and information flow. - In the group configuration procedure, the correction is provided to indicate that the group management clients shall perform explicit registration to join the group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistent stage 2 requirements that may cause interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 10.3.2.1; 10.3.3; 10.3.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

VAL user: An authorized user, who can use a VAL UE to participate in one or more VAL services.

VAL user ID: A generic name for the user ID of a VAL user within a specific VAL service.

VAL UE: A UE that can be used to participate in one or more VAL services.

VAL client: An entity that provides the client side functionalities corresponding to the vertical applications.

SEAL client: An entity that provides the client side functionalities corresponding to the specific SEAL service.

VAL service: A generic name for any service offered by the VAL service provider to their VAL users.

SEAL service: A generic name for a common service (e.g. group management, configuration management, location management) that can be utilized by multiple vertical applications.

SEAL provider: Provider of SEAL service(s).

VAL server: A generic name for the server application function of a specific VAL service.

SEAL server: An entity that provides the server side functionalities corresponding to the specific SEAL service.

VAL system: The collection of applications, services, and enabling capabilities required to support a VAL service.

Primary VAL system: VAL system where the VAL user profiles of a VAL user are defined.

Partner VAL system: A VAL system that has a business relationship with the primary VAL system such that service can be offered to primary VAL system users.

VAL group: A defined set of VAL UEs or VAL users configured for specific purpose in a VAL service.

NOTE:	The set could be of either VAL UEs or VAL users depending on the specific VAL service.

Administrator of the VAL group: A VAL group member who has special authorities to manage the group and does not need to register explicitly to join the VAL group.

Explicit VAL group member: A VAL group member who is not an administrator of the VAL group and needs to register explicitly to join the VAL group.

Implicit VAL group member: A VAL group member who is not an administrator of the VAL group and does not need to register explicitly to join the VAL group.

VAL group home system: The VAL system where the VAL group is defined.

VAL group member: A VAL service user, whose VAL user ID is listed in a particular VAL group.

Vertical: See vertical domain.

Vertical application: An application catering to a specific vertical.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.104 [2] apply:

Vertical domain

* * * Next change * * * *

#### 10.3.2.1	Group creation request

Table 10.3.2.1-1 describes the information flow group creation request from the group management client to the group management server.

Table 10.3.2.1-1: Group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| Identity list | M | List of VAL user IDs or VAL UE IDs that are part of the group to be created corresponding to the list of the configured services |
| Membership type list | M | Indicates the membership type for each VAL user ID or VAL UE ID defined in the "Identity list" information element, e.g., administrator, explicit, or implicit (see the definitions in clause 3.1). |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

* * * Next change * * * *

### 10.3.3	Group creation

Figure 10.3.3-1 below illustrates the group creation operations by authorized VAL user/UE/administrator to create a group. It applies to the scenario of normal group creation by a VAL administrator or by authorized user/UE.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is aware of the users' identities which will be combined to form the VAL group.

Figure 10.3.3-1: Group creation

1.	The group management client of the authorized VAL user/UE/administrator requests group create operation to the group management server. The identities of the users or UEs being combined and the information of the VAL services that are enabled on the group shall be included in this message. The group management client of the authorized VAL user/UE/administrator shall indicate the membership type (e.g., administrator, explicit, or implicit) for each VAL user or VAL UE ID provided in the group creation request.

2.	During the group creation, the group management server creates and stores the information of the group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s).

NOTE:	The exact policies are out of scope of the present document.

3.	The group management server may conditionally notify the VAL server regarding the group creation with the information of the group members.

4.	The VAL group members of the VAL group are notified about the newly created VAL group configuration data.

a.	if the VAL group member is defined as an administrator or implicit in the group creation request, VAL group member is not required to register explicitly to join the group; and

b.	if the VAL group member is defined as an explicit in the group creation request, VAL group member and needs to register explicitly to join the VAL group.

5.	The group management server provides a group creation response to the group management client of the administrator/authorized VAL user/UE.

* * * Next change * * * *

#### 10.3.8.2	Procedure

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL clients belong to the same VAL system.

2.	The VAL server is aware of the users' identities and is authorized to form a VAL group.

Figure 10.3.8.2-1: Procedure for establishing VAL group communication between the group management server and group management client1.

1.	The VAL server determines group information and the identity list to which the group announcement shall be sent. The decision can be based on the list of authorized UEs and other criteria (e.g. user consent, service, or vehicle driving profile).

2.	The VAL server configures VAL group for Uu communication defined by VAL Group ID for one or more VAL services with list of VAL Service ID with the group management server.

3.	The group management server creates an empty group based on the information provided in the Configure VAL group request.

4.	The group management server announces the VAL group to the group management clients. The group management client defined in the announcement need to register explicitly to join the VAL group.

5.	The group management client registers to VAL group communication using the VAL Group ID.

6.	The group management server records the users who have registered to be the members of the group.

7.	The group management server sends a VAL group registration response to the group management client.

8.	The group management server sends a configure VAL group response to the VAL server.

NOTE:	Step 8 may occur any time after step 4.

9.	The group management server sends identity list notification about the newly registered users to the other members of the group and VAL server, whose subscription to receive notifications of newly registered VAL UE IDs is successful in step 7 and step 8 respectively.

10.	The group management client may inform VAL client about the updated identity list.

* * * End of changes * * * *