3GPP TSG-SA WG6 Meeting #53	S6-230643

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0239 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EDGEAPP and ETSI MEC aligned deployment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei Sweden AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EDGEAPP and ETSI MEC alignment is one of the objectives in R18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update annex C to provide aligned deployment option and mapping between overlapping APIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EDGEAPP and ETSI MEC alignment is not addressed in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Key issue #5, Sol#36 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

# C.3	EDGEAPP architecture and ETSI ISG MEC aligned deployment

## C.3.1 General

Figure C.3.1-1 shows the aligned deployment of EDGEAPP architecture and ETSI ISG MEC. MEC application and MEC platform in ETSI MEC correspond respectively to EAS and EES in 3GPP specification. EDGE platform may consist of functionalities provided by EES and/or MEC platform.

Figure C.3.1-1: EDGEAPP architecture and ETSI ISG MEC aligned deployment

## C.3.2	Alignment of EAS registration and MEC application registration

Registration of an instance of an application is supported by both ETSI MEC and 3GPP. In order to support registration of an application on EES it is required that the registration request includes at least the mandatory IEs that are required for EAS registration, i.e. EAS ID and EAS endpoint. On the other hand, according to ETSI GS MEC 011 [16] the application registration request must include AppName and App Provider.

AppInfo which is defined in ETSI GS MEC 011 [16] includes AppName and App Provider as mandatory IEs and endpoint as an optional IE. AppName can be considered equivalent to EAS ID. Similarly, App Provider corresponds to EAS Provider Identifier. Endpoint in ETSI GS MEC 011 [16] can be directly mapped to EAS endpoint. See Table C.3.2-1 for a comparison between the EAS profile and appInfo.

Table C.3.2-1: Comparison between EAS Profile and appInfo

| IE in EAS profile | Status | IE in appInfo [16] | Status [16] | Remarks |
| --- | --- | --- | --- | --- |
| EASID | M | appName | M | Clause 7.2.4: The EASID identifies a particular application for e.g. SA6Video, SA6Game etc. For example, all Edge SA6Video Servers will share the same EASID. NOTE: The definition of the EASID is out of scope of this specification. ETSI MEC specification [16]: Name of the application. It shall be consistent with the appName in the AppD, if an AppD is available. In AppD, it is defined as: Name to identify the MEC application. In both specifications, the IE is used to identify a particular application. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS Endpoint | M | endpoint | O | Clause 8.2.4: Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. ETSI MEC specification [16]: Endpoint information (e.g. URI, FQDN, IP address) of the application server, which is part of the application functionalities. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| ACID(s) | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Provider Identifier | O | App Provider | M | Clause 8.2.4: The identifier of the ASP that provides the EAS. ETSI MEC specification [16]: Provider of the application. It shall be consistent with the appProvider in the AppD, if an AppD is available. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS Type | O | appCategory | O | Clause 8.2.4: The category or type of EAS (e.g. V2X) ETSI MEC specification [16]: Category of the application. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS description | O | AppD > appDescription | O | Clause 8.2.4: Human-readable description of the EAS. ETSI MEC specification [16]: Human readable description of the MEC application. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS Schedule | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Geographical Service Area | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Topological Service Area | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Service KPIs | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS service permission level | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Feature(s) | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Service continuity support | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| List of EAS DNAI(s) | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| List of N6 Traffic Routing requirements | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Availability Reporting Period | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Status | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
|  |  | appDId | O | ETSI MEC specification [16]: The application descriptor identifier. It is managed by the application provider to identify the application descriptor in a globally unique way. Shall be present if the application instance is instantiated by the MEC Management. |
|  |  | appInstanceId | O | ETSI MEC specification [16]: Identifier of the application instance. Shall be present if the application instance is instantiated by the MEC Management. |
|  |  | appServiceRequired | O | ETSI MEC specification [16]: Describes services a MEC application requires to run. ServiceDependency is defined in ETSI GS MEC 010-2 [YY]. It shall shall not be provided if an AppD is available. |
|  |  | appServiceOptional | O | ETSI MEC specification [16]: Describes services a MEC application may use if available. ServiceDependency is defined in ETSI GS MEC 010-2 [YY]. It shall shall not be provided if an AppD is available. |
|  |  | appFeatureRequired | O | ETSI MEC specification [16]: Describes features a MEC application requires to run. FeatureDependency is defined in ETSI GS MEC 010-2 [YY]. It shall shall not be provided if an AppD is available. |
|  |  | appFeatureOptional | O | ETSI MEC specification [16]: Describes features a MEC application may use if available. FeatureDependency is defined in ETSI GS MEC 010-2 [YY]. It shall shall not be provided if an AppD is available. |
|  |  | isInsByMec | O | ETSI MEC specification [16]: Indicate whether the application instance is instantiated by the MEC Management. Default to FALSE if absent. |

NOTE 1:	The appInfo appears in ETSI GS MEC 011 [16]. In the ETSI MEC specification there is an additional optional IE, appProfile, for providing EAS profile. This IEs is not applicable to Table C.3.2-1, which analyses correspondence between IEs in EAS profile and appInfo.

The application may perform registration on EES according to the mapping between appInfo and EAS profile.

## C.3.3	Alignment of EDGE-9 and Mp3

EDGE-9 interface in EDGEAPP architecture is used to support mobility of user from one EES to another EES. On the other hand, Mp3 reference point between MEC platforms is used for control communication between MEC platforms [15] with a separate application mobility service [XX] being provided in support of mobility of users between MEC hosts within a MEC system. Since ETSI MEC has not specified APIs over Mp3, no alignment is provided in this release.

# * * * Change 2 * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedure for the 5G System (5GS); Stage 2".

[4]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[5]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[8]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[9]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[10]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[12]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.434: "Service enabler architecture layer for verticals; Functional architecture and information flows; Stage 2".

[14]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows ".

[15]	ETSI ISG MEC ETSI GS MEC 003 V2.1.1 (2019-01), "Multi-access Edge Computing (MEC); Framework and Reference Architecture"

[16]	 ETSI GS MEC 011 "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[17]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	GSMA Whitepaper OPG.01: "Operator Platform: Telco Edge Proposal", https://www.gsma.com/futurenetworks/wp-content/uploads/2020/10/GSMA-Operator-Platform-Proposal-Oct-2020.pdf

[20]	3GPP TS 23.548: "5G System Enhancements for Edge Computing".

[21]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[22]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[23]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications".

[24]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[25]	3GPP TS 32.257: "Telecommunication management;Charging management;Edge computing domain charging".

[XX]	ETSI GS MEC 021 V2.2.1 (2022-00), "Multi-access Edge Computing (MEC); Application Mobility Service API".

[YY]	ETSI GS MEC 010-2: "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

# * * * End of changes * * * *