3GPP TSG-SA WG6 Meeting #53	S6-230647

Athens, Greece 27th February – 3rd March 2023

Source:	Huawei

Title:	Add missing APIs of NSCE adaptation

Spec:	3GPP TS 23.435 v0.5.0

Agenda item:	8.10

Document for:	Approval

Contact:	Xiaoqian JIA, jiaxiaoqian@huawei.com

1. Introduction

Procedures and information flows of VAL server-triggered  and network-based network slice adaptation and VAL UE-triggered and network-based network slice adaptation for VAL application are captured in TS 23.435 v0.5.0, but the APIs for the latter is missing.

Besides, the source network slice identifier is needed in “Network slice adaptation request” to indicate the current network slice in case whether the current network slice status crosses the requested threshold is to be checked.

2. Reason for Change

The APIs of VAL UE-triggered and network-based network slice adaptation for VAL application are missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v0.5.0.

* * * First Change * * * *

9.11.3	Information flows

9.11.3.1	Network slice adaptation request

Table 9.11.3.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCE server.

Table 9.11.3.1-1: Network slice adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation may corresponds to. |
| List of VAL UE IDs | M | List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds |
| Requested Network slice related identifier(s) | O | Identifier of network slice for which the request applies |
| Source Network slice related identifier(s) | O | Identifier of network slice by which the VAL service is supported currently |
| Requested DNN | O | Indication of the new DNN which is requested. |
| Requested Adaptation threshold | O (see NOTE) | The threshold of network slice adaptation |
| >Requested adaptation threshold of the delay of network slice | O | The network slice delay defined clause 5.1.1.1, 5.1.1.2, 5.1.1.3, in 3GPP TS 28.552 [19] and key performance indicators in clause 6.3, in TS 28.554 [20], and network slice related analytics report in clause 8.4.2.4.3, in 3GPP TS 28.104 [21]. |
| NOTE: The NSCE is requested to adapt UE to the requested network slice only when the current network slice status crosses the requested threshold. |  |  |

* * *Next Change * * * *

9.11.4	APIs

9.11.4.1	General

Table 9.11.4.1-1 illustrates the APIs for VAL server-triggered and network-based network slice adaptation.

Table 9.11.4.1-1: List of APIs for network slice adaptation

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ NSCE_NetworkSliceAdaptation | Network_slice_adaptation | VAL server | Request /Response |

Table 9.11.4.1-2 illustrates the APIs for VAL UE-triggered and network-based network slice adaptation

Table 9.11.4.1-2: List of APIs for network slice capability adaptation trigger

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ NSCE_NetworkSliceAdaptationTrigger | Network_slice_adaptation_trigger | VAL server | Request /Response |