3GPP TSG-SA WG6 Meeting #53	S6-230666

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | <CR#> | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge Node Sharing solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#22, Solution #44 was studied in TR 23.700-98. GSMA provided feedback on the solution: Quote: “• GSMA OPG response: Similar to the Solution #43, 3GPP EDGEAPP scenario assumes that the EAS can be shared to both OP A and OP B, while in Edge Node Sharing scenarios, the workload/EAS deployed on the shared edge node would not be available to subscribers other than those connected to the network to which the node is being shared, as described in clause 3.3.5 of GSMA PRD OPG.02.” Solution #44 needs to be modified to make sure that EAS deployed on shared resource are not available to the subscriber of the partner ECSP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - EAS profile change to indicate the federation for which the EAS is deployed - Change in EAS discovery procedure to support EAS discovery from partner OP - Procedure for EAS discovery from partner EES - Procedure for Get list of all registered EAS from partner OP - Change in service provisioning response related to Retrieve T-EAS procedure - Procedure between ECS-ER to ECS-ER |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Edge Node Sharing feature will not be supported in the Rel-18 EDGEAPP work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 8.5.2.2, 8.5.2.a (New), 8.5.2.b (New), 8.3.3.3.3, 8.x (New), 8.x.y (New), 8.x.z (New), 8.x.z.1 (New), 8.x.z.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#22, Solution #44 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.4	EAS Profile

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| General context holding time duration (NOTE) | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |
| Federation identifier | O | Identifies the federation for which the EAS is deployed by lead ECSP on shared resources of partner ECSP |
| > List of PLMN IDs | O | List of PLMN IDs whose subscribers are allowed to access the EAS |
| NOTE: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). |  |  |

* * * Next Change * * * *

#### 8.5.2.2	Request-response model

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.5.2.2-1: EAS Discovery procedure

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location. If the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration. The EES may select one EAS and determine to perform application traffic influence for this AC based on AC’s service KPI or EAS’s service KPI in desired response time, when the EES does not perform traffic influence in advance.

When EAS discovery filters are not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

Upon receiving the request from the EEC, the EES may trigger the EAS management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

If the required EAS is not registered with the EES, the EES determines to provide application from partner ECSP. If required, the EES may perform Retrieve T-EES procedure as specified in clause 8.8.3.3 to retrieve information about EES of partner ECSP where the required EAS is registered. The EES discovers EAS information from EES of the partner ECSP as specified in clause 8.5.2.a.

3.	If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs. For discovered EASs, this includes endpoint information. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request.

Upon receiving the EAS discovery response, the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

NOTE 4:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command).

NOTE 5:	The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document.

NOTE 6:	The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required.

If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request.

* * * Next Change * * * *

#### 8.5.2.a	EAS discovery from partner EES

Figure 8.5.2.a -1 illustrates the procedure for EAS discovery for Edge Node Sharing scenario, where the EES (ECSP-B) requests list of all registered EASs from EES (ECSP-A) or perform EAS discovery based on the discovery filters provided by the EEC.

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

2.	EES (ECSP-B) has received EAS discovery request where required EAS is not available with the EES (ECSP-B).

3.	EES (ECSP-B) may have performed retrieve T-EES procedure to receive details about EES (ECSP-A).

Figure 8.5.2.a-1: EAS discovery for edge node sharing

1)	The EES (ECSP-B) sends EAS discovery request over EDGE-9 reference point to the EES (ECSP-A). The request includes EAS discovery filters as provided by EEC along with federation identifier, ECSP identifier and PLMN ID.

2)	The EES (ECSP-A) validates the edge service SLA and returns EAS discovery response including the discovered candidate EAS(s) to the EES (ECSP-B).

Upon receiving registered EAS information from EES (ECSP-A), the EES (ECSP-B) sends the EAS discovery response to EEC including matching EAS details.

NOTE:	If the EAS discovery request is received from the EEC belonging to the subscriber of the PLMN other than the PLMN with which the resources are shared, the EES (ECSP-A) reject the EAS discovery request.

* * * Next Change * * * *

#### 8.5.2.b	Get list of all registered EAS from partner OP

Based on service provider policy, the EES (ECSP-B) either requests list of all registered EASs from EES (ECSP-A) as specified in this clause, or perform EAS discovery based on the discovery filters provided by the EEC as specified in clause 8.5.2.a.

Pre-conditions:

1.	ECSP-1 and ECSP-2 have a service level agreement to share edge services.

2.	EES (ECSP-B) has received EAS discovery request where required EAS is not available with the EES (ECSP-B).

3.	EES (ECSP-B) may have performed retrieve T-EES procedure to receive details about EES (ECSP-A).

Figure 8.5.2.b-1: Get list of all registered EASs from EES (ECSP-A)

Following steps shows procedure to request list of all registered EASs from EES-A.

1)	If the EES (ECSP-B) does not have full application knowledge at the time of EAS discovery request from the EEC, the EES triggers request to EES (ECSP-A) to provide list of all registered and allowed EAS information. The request includes required parameters like federation identifier, ECSP identifier, EESID and security parameters.

2)	Upon receiving the request from EES (ECSP-B), the EES (ECSP-A) checks whether EES (ECSP-B) is authorized to receive registered EAS list or not based on service level agreement. If authorized, the EES (ECSP-A) sends registered EAS information response. The response includes the registered and allowed EAS for edge node sharing service between ECSP-A and ECSP-B, EASID and EAS Geographical Service. Otherwise, the EES-A sends the failure response. Upon receiving the success response, the EES-B stores the information to be used during EAS discovery procedure.

NOTE:	The request to get list of all registered EASs from EES-A to EES-B can be triggered anytime based on implementation.

* * * Next Change * * * *

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (NOTE 2) | O | List of EASIDs registered with the EES. |
| > instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| > EES Provider identifier | O | The identifier of the EES Provider (such as ECSP) |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| > Federation identifier | O | Identifies the federation for which the EAS is deployed by lead ECSP on shared resources of partner ECSP |
| >> List of PLMN IDs | O | List of PLMN IDs whose subscribers are allowed to access the EAS |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). |  |  |

* * * Next Change * * * *

## 8.x	Support for roaming and federation

### 8.x.y	Publish/unpublish EAS information

Since the application instance is deployed in the partner ECSP-A, when the leading ECSP-B receives a request from the UE, the leading ECSP-B interacts with the partner ECSP-A to discover the EAS. This clause provides ways for the leading ECSP-B to discover EAS of the partner ECSP-A for subsequent communication.

Pre-conditions:

1.	The federation agreement exists between ECSP-A and ECSP-B;

2.	The EAS is deployed by lead ECSP-B on the shared resource of partner ECSP-A; and

3.	The EAS is instantiated and registered with EES-A;

Figure 8.x.y-1: publish and unpublish registered EAS information to ECS-ER

0)	Once EAS is instantiated, the EAS is registered with EES-A. Upon EAS registration, based on the EAS profile, the EES-A determines whether to publish the registered EAS information if the EAS is deployed by lead ECSP.

1)	Based on service level agreement between ECSP-A and ECSP-B if EAS is deployed and shared to ECSP-B, and also based on EAS registration status, the EES-A sends registered EAS information publish or unpublish request to an ECS-ER (of ECSP-A). The request includes the registered and allowed EAS for federation between ECSP-A and ECSP-B, EASID and EAS Geographical Service.

2)	The ECS-ER checks whether EES is authorized to publish or unpublish registered EAS list or not based on service level agreement.

3)	If authorized, the Edge repository stores the EAS information if the request is to publish the information, or removes the EAS information if the request is to unpublish the information. The ECS-ER sends registered EAS information publish or unpublish response.

* * * End of Change * * * *

### 8.x.z	EAS information exchange between lead ECSP and partner ECSP

#### 8.x.z.1	Request response model

The ECS-ER (ECSP-A) fetches application information from its partner ECSP-B) as shown in figure 8.x.z.1-1, periodically.

Pre-conditions:

1.	The federation agreement exists between ECSP-A and ECSP-B;

Figure 8.x.z.1-2: Getting registered EAS information

1)	The ECS-ER of ECSP-B sends request to fetch registered EAS information to ECS-ER of ECSP-A. The request includes federation identifier for the federation between ECSP-A and ECSP-B, ACID, PLMN ID and ECSP ID.

2)	If the ECS-ER is authorized to access the information, the ECS-ER (ECSP-A) sends success response. The response includes a list of EAS IDs (which are registered within ECSP-A and deployed by ECSP-B on shared resources of ECSP-A) and EES information of OP-B.

#### 8.x.z.2	Subscribe notify model

The ECS-ER (ECSP-A) subscribes for application information from its partner ECSP-B and receives notifications as shown in figure 8.x.z.2-1 and figure 8.x.z.2-2.

Pre-conditions:

1.	The federation agreement exists between ECSP-A and ECSP-B;

Figure 8.x.z.2-1: Subscribe for application information

1)	Based on service level agreement between ECSP-A and ECSP-B, the ECS-ER (ECSP-B) sends subscription request to ECS-ER (ECSP-A) to receive registered EAS information and its including service area. The request includes required parameters like federation identifier, ECSP identifier, ECS identifier, required application identifiers and security parameters.

2)	Upon receiving the request from ECS-ER (ECSP-B), the ECS-ER (ECSP-A) checks whether ECS-ER (ECSP-B) is authorized to receive registered EAS list or not based on service level agreement.

3)	The ECS-ER (ECSP-A) sends subscription response – which indicates result of the subscription (success or failure) and subscription identity (if the result is success).

Figure 8.x.z.2-2: Notification for application information

1)	The list of registered and allowed EAS is changed on the ECS-ER (ECSP-A) – that is either new EAS is registered or EAS registration is updated or EAS is deregistered or new EAS is published or already published EAS is unpublished.

5)	The ECS-ER (ECSP-A) sends notification to the ECS-ER (ECSP-B). The notification includes the registered and allowed EAS for edge node sharing service between ECSP-A and ECSP-B, EASID and EAS Geographical Service. Upon receiving the notification, the Edge repository (of OP-A) stores the information to be used in other procedures (for EAS discovery or get registered EAS information request).

* * * End of Change * * * *