3GPP TSG-SA WG6 Meeting #53	S6-230706

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0113 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update of clause 8.2.8 Constrained device selecting MSGin5G Proxy UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the constrained UE communicates with the MSGin5G Client in the middle via MSGin5G-6 reference point, and relay feature is used, there is no additional procedure for MSGin5G service. The Relay UE can be selected by using the 5G ProSe UE-to-Network Relay Discovery procedure specified in TS23.304. If the proxy feature is used, a new procedure is needed. Therefore, the procedure is needed to be updated and the MSGin5G Client in the middle acts as MSGin5G Proxy UE, |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of clause 8.2.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MSGin5G Client in the middle will acts as both MSGin5G Relay UE and MSGin5G Proxy UE. It will make other related procedures very complicated and unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.8	Constrained device with MSGin5G Client selecting MSGin5G Proxy UE

The signalling flow for MSGin5G Client in the MSGin5G UE-2 (which is a constrained device) to request other MSGin5G UEs having proxy service capabilities to provide their configuration is illustrated in figure 8.2.8-1.

Pre-condition:

1)	MSGin5G UE-2 is not able to communicate with MSGin5G Server directly;

2)	MSGin5G UE-2 discovers nearby MSGin5G UEs using 5G ProSe direct discovery procedure as specified in 3GPP TS 23.304 [18].

3)	An MSGin5G UE-1 has roxy capability.

Figure 8.2.8-1: Constrained UE requesting to configuration of the proxy UE

1)	The MSGin5G Client-2 in MSGin5G UE-2 (which is a constrained UE) sends MSGin5G proxy request message to all surrounding MSGin5G UE (using 5G ProSe Direction communication as specified in 3GPP TS 23.304 [18]) via MSGin5G-6 reference point to provide their proxy configuration if proxy service is supported by the MSGin5G UE. The MSGin5G proxy request includes information elements as specified in Table 8.2.8-1.

Table 8.2.8-1: Information elements for MSGin5G proxy request

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-2 |
| UE service ID | M | UE service identifier of the UE-2 |
| MSGin5G UE ID | O | MSGin5G device identifier of the UE-2 |
| Required packet size | M | Maximum allowed packet size of the UE-2 |

2)	Upon receiving the MSGin5G proxy request from the MSGin5G Client of MSGin5G UE-2 via MSGin5G-6 reference point, if MSGin5G UE-1 supports the proxyservice capability, the MSGin5G Client-1 sends the MSGin5G proxy response for the proxy configuration. The MSGin5G proxy response message includes information elements as specified in Table 8.2.8-2.

Table 8.2.8-2: Information elements for MSGin5G proxy response

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-1 |
| UE service ID | M | UE service identifier of the UE-1 |
| MSGin5G UE ID | O | MSGin5G device identifier of the UE-1 |
| Allowed packet size | M | Maximum allowed packet size of the UE-2 |

3)	If the MSGin5G UE-2 receives responses from multiple different proxy UEs (i.e. MSGin5G UE-1 with support for the proxy service), then the MSGin5G Client of the MSGin5G UE-2 selects MSGin5G UE-1 (with the proxy service) whose value of the Allowed packet size is equal or more than the value of the Required packet size of the MSGin5G UE-2. If value of the Allowed packet size is less than the value of the Required packet size for all MSGin5G UE-1s (i.e. proxy UEs), then the MSGin5G Client of the MSGin5G UE-2 selects the MSGin5G UE-1 (with proxy service) with the maximum value of the Allowed packet size among all MSGin5G UE-1s and further sets the maximum allowed packet size of the constrained UE to the value of the Allowed packet size of the selected MSGin5G UE-1 (i.e. proxy UE).

Once MSGin5G UE-2 selects the MSGin5G Proxy UE, the MSGin5G UE-2 indicates the selected MSGin5G UE by registering to use the proxy service. The signalling flow for MSGin5G Client in the MSGin5G UE-2 (which is a constrained device) to register itself to use proxy service of the MSGin5G Proxy UE is illustrated in figure 8.2.8-2.

Figure 8.2.8-2: Constrained UE registering to use proxy UE

1)	Upon selecting the proxy UE, the MSGin5G Client-2 of the MSGin5G UE-2 sends Proxy register request to the selected MSGin5G Proxy UE to indicate it about the use of proxy service by the MSGin5G UE-2 (i.e. constrained device). The Proxy register request message includes information elements as specified in Table 8.2.8-3.

Table 8.2.8-3: Information elements for Proxy register request

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-1 |
| UE service ID | M | UE service identifier of the UE-1 |
| Credential information | M | UE-1 credential information |
| Expire | O | the time till when constrained device is intended to use the proxy service |

2)	Upon receiving the request from the MSGin5G Client-2 of the MSGin5G UE-2, the MSGin5G Client-1 checks whether the MSGin5G UE-2 is authorized to use the proxy service or not. And if the MSGin5G UE-2 is authorized, the MSGin5G Client-1 sends the Proxy register response message with the status of the registration. If the registration is success, the Proxy register response includes information elements as specified in Table 8.2.8-4..

Table 8.2.8-3: Information elements for Proxy register response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |
| Expire | O | the time till when constrained device is allowed to use the proxy service |