3GPP TSG-SA WG6 Meeting #53	S6-230708

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0304 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows and procedures for the ad hoc group call for SDS and FD services of MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TR 23.700-76 V1.3.0 has defined the information flows for the adhoc group communication. The conclusion in the clause 9 for the Key issue #1 indicates that the information flows in the clause 7.1.2 of solution 1 is agreed without the “AHGC share security material command”, and exluding the “MC service ad hoc group ID” & “Preconfigured ad hoc group identity” IEs from the ad hoc group communication request from the MCData client to the MCData server.This CR is trying to propose the corresponding normative text in the normative work phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new clause to provide the information flows and procedures for the adhoc group call for SDS and FD session based communication. Added the information flows as defined in the clause 7.1.2 of TR 23.700-76 V1.3.0 into new clause 7.17 of TS 23.282 as a normatie text. The adhoc group call procedures are defined as common information flows and common procedures. Each MCData sub services, SDS and FD can use the informations flow and procedures in conjuction with their respective procedures. High level procedure are defined which indicates that both common ad hoc group procedures and respective procedures are required and used. The information flows between MCData servers for “Ad hoc group call request” and “Ad hoc group call response” are added which were missing in the TR 23.700-76 V1.3.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group call support will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (7.17, 7.17.1, 7.17.2, 7.17.2.1, 7.17.2.2, 7.17.2.3, 7.17.2.4, 7.17.2.5, 7.17.2.6, 7.17.2.7, 7.17.2.8, 7.17.2.9, 7.17.2.10, 7.17.2.11, 7.17.2.12, 7.17.2.13, 7.17.2.14, 7.17.3, 7.17.3.1, 7.17.3.1.1, 7.17.3.1.2, 7.17.3.1.3, 7.17.3.1.4, 7.17.3.1.5, 7.17.4, 7.17.4.1, 7.17.4.2, 7.17.4.3, 7.17.4.3.1, 7.17.4.3.2) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.17	Ad hoc group call

### 7.17.1	General

This subclause contains procedures for ad hoc group call across a single MCData system for both SDS and FD services, and associated functions such as emergency call, Imminent peril call, broadcast call and others.

The ad hoc group calls can use the participants list provided by either an initiator of the call or MCData server. The MCData server can use the criteria provided by the initiator of the call to determine the participants list along with its own criteria or local policies. The resulting ad hoc group uses the configuration of a separate preconfigured MCData group.

NOTE 1:	A preconfigured group which is intended only to provide configuration for the ad hoc group is identified by a parameter in the group configuration described in 3GPP TS 23.280 [5].

The preconfigured MCData group that provides the configuration is not used for the MCData group communication, it only provides configuration for one or more adhoc group calls. The MCData group ID of the ad hoc group call is provided by the MCData server when the ad hoc group call is originated. In order to establish security context for the end-to-end secured ad hoc group call, the security related information is used from this preconfigured group.

NOTE 2:	The configurations defined for the adhoc group call in user profile and service configuration document as described in the Annex A.3 and A.5 is applicable only for single MCData system.

### 7.17.2	Common Information flows

#### 7.17.2.1	Ad hoc group call request (MCData client – MCData server)

Table 7.17.2.1-1 describes the information flow ad hoc group call request from the MCData client to the MCData server.

Table 7.17.2.1-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MC service ad hoc group ID (see NOTE 1) | O | The MC service group ID which is generated by the MC service user to be associated with the ad hoc group communication |
| Encryption supported (see NOTE 2) | O | Indicates whether this ad hoc group call supports end-to-end encryption |
| MCData ID list (see NOTE 3, NOTE 5) | O | MCData IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCData clients |
| Imminent peril indicator (see NOTE 4) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 4) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| MCData ID list (see NOTE 3) | O | List of participants required to acknowledge the ad hoc group call before start of the audio transmission |
| Location information | O | Location of the calling party. |
| Criteria for determining the participants (see NOTE 5) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: If this information element is not included, the MC service server assigns an MC service ad hoc group ID to be used for the ad hoc group communication. This information element is returned to the calling party to use in the ad hoc group communication. NOTE 2: This information element is present and set to true only if this ad hoc group call is encrypted. When the ad hoc group call is initiated with participants provided by the initiator this acts as an indicator that subsequent requests follow targeting the individual participants and carrying the relevant key material. If this information element is set to false or not present, then this ad hoc group call is unencrypted. NOTE 3: This element is included only when the originating client sends the list of participants. NOTE 4: If used, only one of these information elements is present. NOTE 5: Only one of these information elements is present. |  |  |

#### 7.17.2.2	Ad hoc group call request return (MCData server – MCData client)

Table 7.17.2.2-1 describes the information flow ad hoc group call request return from the MCData server to the MCData client. This response could be intermediate response to provide the server assigned MCData ad hoc group ID.

Table 7.17.2.2-1 Ad hoc group call request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| MCData ad hoc group ID | O | The MCData group ID to be associated with the ad hoc group call which is assigned by the MCData server.This information element shall be present if the authorization result is success. |
| Authorization result | M | Indicate if authorization is success or failure |

#### 7.17.2.3	Ad hoc group call request (MCData server – MCData server)

Table 7.17.2.3-1 describes the information flow ad hoc group call request between the MCData servers.

Table 7.17.2.3-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group call |
| MCData ID list (see NOTE 1, NOTE 3) | O | MCData IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCData server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

#### 7.17.2.4	Ad hoc group call request (MCData server – MCData client)

Table 7.17.2.4-1 describes the information flow ad hoc group call request from the MCData server to the MCData client.

Table 7.17.2.4-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group call |
| SDP offer | M | Media parameters of MCData server |
| Broadcast indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| NOTE 1: If used, only one of these information elements is present. |  |  |

#### 7.17.2.5	Ad hoc group call response (MCData server – MCData client)

Table 7.17.2.5-1 describes the information flow ad hoc group call response from the MCData server to the MCData client.

Table 7.17.2.5-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCData group ID | M | The MCData group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the group call request (success or failure) |

#### 7.17.2.6	Ad hoc group call response (MCData server – MCData server)

Table 7.17.2.6-1 describes the information flow ad hoc group call response between the MCData servers.

Table 7.17.2.6-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the target MC service user |
| Functional alias | O | The functional alias of the target MC service user |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

#### 7.17.2.7	Ad hoc group call response (MCData client – MCData server)

Table 7.17.2.7-1 describes the information flow ad hoc group call response from the MCData client to the MCData server.

Table 7.17.2.7-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the target MC service user |
| Functional alias | O | The functional alias of the target MC service user |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

#### 7.17.2.8	Ad hoc group call release request (MCData server – MCData client)

Table 7.17.2.8-1 describes the information flow ad hoc group call release request from the MCData server to the MCData client.

Table 7.17.2.8-1 Ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCData ad hoc group ID | M | The MCData group ID of the ad hoc group call on which call is released |

#### 7.17.2.9	Ad hoc group call release response (MCData client – MCData server)

Table 7.17.2.9-1 describes the information flow ad hoc group call release response from the MCData server to the MCData client.

Table 7.17.2.9-1 Ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCData group ID | M | The MCData group ID of the ad hoc group call on which call is released |

#### 7.17.2.10	Ad hoc group call notify (MCData server – MCData client)

Table 7.17.2.10-1 describes the information flow ad hoc group call notify from MCData server to MCData client.

Table 7.17.2.10-1: Ad hoc group call notify

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group call |
| MCData ID list (see NOTE) | O | The list of the invited MC service users who did not acknowledge the ad hoc group call request |
| NOTE: Only applicable to acknowledged group calls. |  |  |

#### 7.17.2.11	Modify ad hoc group call participants request (MCData client – MCData server)

Table 7.17.2.11-1 describes the information flow Modify ad hoc group call participants request from the MCData client to the MCData server.

Table 7.17.2.11-1: Modify ad hoc group call participants request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group call whose participants list needs to be modified |
| MCData ID list (see NOTE) | O | List of additional MCData users to be added to the on-going ad hoc group call |
| MCData ID list (see NOTE) | O | List of MCData users to be removed from the on-going ad hoc group call |
| NOTE: Either one or both of these information elements is present |  |  |

#### 7.17.2.12	Modify ad hoc group call participants response (MCData server – MCData client)

Table 7.17.2.12-1 describes the information flow Modify ad hoc group call participants response from the MCData server to the MCData client.

Table 7.17.2.12-1 Modify Ad hoc group call participants response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |
| MCData ID list | O | List of MCData users who are not allowed to be added to the on-going ad hoc group call. This list is provided if the operation is partially success. |

#### 7.17.2.13	Ad hoc group call leave request (MCData server – MCData client)

Table 7.17.2.13-1 describes the information flow ad hoc group call leave request from the MCData server to the MCData client.

Table 7.17.2.13-1 Ad hoc group call leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the ad hoc group call participant leaving the call |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group call on which call is on-going |
| Reason to leave | O | Carries the reason of why the MCData client is being asked to leave the ongoing ad hoc group call |

#### 7.17.2.14	Ad hoc group call leave response (MCData client – MCData server)

Table 7.17.2.14-1 describes the information flow Ad hoc group call leave response from the MCData client to the MCData server.

Table 7.17.2.14-1 Ad hoc group call leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the ad hoc group call participant leaving the call |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group call on which call is on-going |

### 7.17.3	Common Ad hoc group call procedures

#### 7.17.3.1	Ad hoc group call procedures in single MCData system

##### 7.17.3.1.1	Ad hoc group call setup

Figure 7.17.3.1.1-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user.

Pre-conditions:

1.	The authorized user at MCData client 1 wants to invite MCData users at MCData client 2, MCData client 3 and MCData client 4 for the ad hoc group call.

2.	Number of participants being invited for the ad hoc group call is within the limit for non pre-configured approach.

3.	End-to-End encryption is supported for this ad hoc group call.

4.	MCData client 1 is aware of the MCData IDs of the participants.

NOTE 1:	Selection of MCData IDs of the participants can be manual or from the user profile configuration data or by any other means. This is left for the implementation.

Figure 7.17.3.1.1-1: Ad hoc group call setup

1.	User at MCData client 1 would like to initiate an ad hoc group call. The MCData client 1 initiates the ad hoc group call by sending the ad hoc group call request containing the list of participants to the MCData server. Encryption supported information element shall be set to true since end-to-end encryption is supported. An SDP offer containing the MCData client media parameters is included. If the MCData user of MCData client 1 has selected a functional alias, then the ad hoc group call request contains that functional alias.

If the MCData user at MCData client 1 initiates an MCData emergency ad hoc group call or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i.	the MCData ad hoc group call request shall contain an emergency indicator;

ii.	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

2.	The MCData server accepts the ad hoc group call request if the ad hoc group call is supported and authorized. Otherwise reject the ad hoc group call request and do not continue with the rest of the steps. If authorised, it validates whether the number of invited participants is within the configured limit before proceeding with the call setup.

If functional alias is present, the MCData server checks whether the provided functional alias allowed to be used and has been activated for the user.

If location information was included in the ad hoc group call request, the MCData server checks the privacy policy of the MCData user to decide if the location information of MCData client 1 can be provided to other users on the call (refer to Annex A.3 "Authorisation to provide location information to other MCData users on a call when talking").

If an emergency indicator is present in the received MCData ad hoc group call request, the MCData ad hoc group is considered to be in the in-progress emergency state until this ad hoc group call is terminated; and

If an imminent peril indicator is present in the received MCData ad hoc group call request, the MCData ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group call is terminated.

The MCData server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

3.	The MCData server shall send the ad hoc group call request return message to MCData client 1 containing the below:

i.	The MCData ad hoc group ID generated by the MCData server; and

ii.	Result of whether the ad hoc group call is authorized or not

If the ad hoc group call request is not authorized, MCData client 1 shall not proceed with the rest of the steps.

4a-4c.	The MCData server sends the ad hoc group call requests towards the MCData clients of the invited users based on step 1. While sending the ad hoc group call requests, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients (e.g. MCData ID list of the users who are required to acknowledge)

5a-5c.	The receiving MCData clients are notified about the incoming ad hoc group call.

6a-6c.	The receiving MCData clients accept the ad hoc group call requests and send ad hoc group call responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

7.	The MCData server sends the ad hoc group call response to MCData client 1 through the signalling path to inform about successful call establishment.

NOTE 2:	Steps 5 to step 7 can start to occur before all of step 4 is completed since the MCData server do not require to wait for the previous ad hoc group call request to complete before sending the ad hoc group request to another participant.

8.	If the initiating MCData user requires the acknowledgement from the invited MCData users, and the required MCData users do not acknowledge the call setup within a configured time (the "acknowledged call setup timeout"), then the MCData server may proceed with or abandon the call and then notify the initiating MCData user that the acknowledgements did not include all required members according to ad hoc group call policy from the user profile configuration. The MCData server may notify the initiating MCData user of all MCData users who did not acknowledge the ad hoc group call request within the configured time. This notification may be sent to the initiating MCData user by the MCData server more than once during the call when MCData users join or leave the MCData ad hoc group call.

9.	MCData client 1, MCData client 2, MCData client 3 and MCData client 4 establish media plane for data communication.

NOTE 2:	Step 9 can occur any time following step 7 if the conditions to proceed with the call are met.

##### 7.17.3.1.2	Release ad hoc group call

The procedure focuses on the case where the MCData server releases an ongoing MCData ad hoc group call for all the participants of that ad hoc group call, since at least one of the release conditions are met e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving.

Figure 7.17.3.1.2-1 below illustrates the signalling control plane procedure for the MCData server initiating termination an ongoing ad hoc group call.

Figure 7.17.3.1.2-1: Release ad hoc group call

1.	It is assumed that MCData users on MCData client 1, client 2 and client 3 are already part of the ongoing ad hoc group call.

2.	MCData server would like to release the MCData ad hoc group call which is ongoing e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving.

3.	MCData server identifies the participants of the ongoing ad hoc group call and generates ad hoc group call release request to release ongoing session. The MCData server cancels the in-progress emergency state or in-progress imminent peril state of the ad hoc group if the ad hoc group call is an emergency or imminent peril call respectively.

4.	MCData server sends ad hoc group call release request via SIP core towards each participant of the ongoing ad hoc group call.

5.	MCData users are notified about the release of the ad hoc group call.

6.	MCData client(s) receiving ad hoc group call release request, acknowledge towards the MCData server by sending an ad hoc group call release response.

NOTE:	If the initiator of the ad hoc group call does not supply the participants list, the MCData client(s) may choose to store the list of participants for easy re-initiation of another ad hoc group call with the same participants.

7.	MCData client 1, client 2 and client 3 have successfully released the media plane resources associated with the ad hoc group call that is terminated and the ad hoc group ceases to exist (i.e., further call is not possible over the same ad hoc group.

##### 7.17.3.1.3	Ad hoc group call setup with MCData server determining the participants lists

Figure 7.17.3.1.3-1 below illustrates the ad hoc group call setup procedure initiated by the MCData user and MCData client 1 wherein the list of participants is determined by the MCData server based on the citeria received from the MCData client.

Pre-conditions:

1.	The MCData user at MCData client 1 is authorized to initate ad hoc group call.

2.	The MCData user at MCData client 1 wants to invite MCData users who are satisying certain criteria for the ad hoc group call.

Figure 7.17.3.1.3-1: Ad hoc group call participants determined by MCData server

1.	User at MCData client 1 would like to initiate an ad hoc group call in-order to invite the participants satisfying specific criteria. The MCData client 1 initiates the ad hoc group call by sending the ad hoc group call request containing the details of the criteria to be applied by the MCData server for determining the participants list. If end-to-end encryption is supported, the Encryption supported information element shall be set to true and pre-configured MCData group whose configuration is to be applied is included. An SDP offer containing the MCData client media parameters is included. If the MCData user of MCData client 1 has selected a functional alias, then the ad hoc group call request contains that functional alias.

If the MCData user at MCData client 1 initiates an MCData emergency ad hoc group call or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i.	the MCData ad hoc group call request shall contain an emergency indicator;

ii.	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

2.	The MCData server accepts the ad hoc group call request if the ad hoc group call is supported and authorized. Otherwise reject the ad hoc group call request and do not continue with the rest of the steps.

If functional alias is present, the MCData server checks whether the provided functional alias is allowed to be used and has been activated for the user.

If location information was included in the ad hoc group call request, the MCData server checks the privacy policy of the MCData user to decide if the location information of MCData client 1 can be provided to other users on the call (refer to Annex A.3 "Authorisation to provide location information to other MCData users on a call when talking").

If an emergency indicator is present in the received MCData ad hoc group call request, the MCData ad hoc group is considered to be in the in-progress emergency state until this ad hoc group call is terminated; and

If an imminent peril indicator is present in the received MCData ad hoc group call request, the MCData ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group call is terminated.

3.	The MCData server shall send the ad hoc group call request return message to MCData client 1 containing the below:

i.	The MCData ad hoc group ID generated by the MCData server; and

ii.	Result of whether the ad hoc group call is authorized or not

If the ad hoc group call request is not authorized, MCData server and client 1 shall not proceed with the rest of the steps.

4.	The MCData server determines the list of participants to be invited for the ad hoc group call based on the information present in the information element Criteria for determining the participants. This information element could carry either criteria or indicator identifying pre-defined criteria or a combination of both.

NOTE 1:	The content of the Criteria information element, the details of the pre-defined criteria, and the way how their MCData server determines the list of participants are left to implementation.

5.	The MCData server sends the ad hoc group call requests towards the MCData clients 2 and 3. While sending the ad hoc group call requests, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. The MCData server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

6.	The receiving MCData clients are notified about the incoming ad hoc group call.

7.	The receiving MCData clients accept the ad hoc group call requests and send ad hoc group call responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

8.	The MCData server sends the ad hoc group call response to MCData client 1 through the signalling path to inform about successful call establishment.

9.	The MCData server may notify the initiating MCData user of all MCData users who acknowledged the ad hoc group call request and joined the ad hoc group call. This notification may be sent to the initiating MCData user by the MCData server more than once during the call when MCData users join or leave the MCData ad hoc group call.

NOTE 2:	The authorized user can learn who is currently affiliated to the current adhoc group call.

10.	MCData client 1, MCData client 2 and MCData client 3 establish media plane resources.

##### 7.17.3.1.4	Modification of ad hoc group call participants by the initiator

Figure 7.17.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by the initiator of the ad hoc group call.

Pre-conditions:

1.	The MCData client 1 is the initiator of the ad hoc group call.

2.	The MCData users on MCData client 1, MCData client 3 to MCData client n are on an ongoing ad hoc group call.

3.	The MCData user 1 requests to remove the user of MCData client 3 from the ad hoc group call and add user of MCData client 2 into the on-going ad hoc group call.

Figure 7.17.3.1.4-1: Modification of ad hoc group call participants by the initiator

1.	The MCData client 1 sends the modify ad hoc group call participants request to the MCData server in order to remove MCData client 3 from the ongoing ad hoc group call and add MCData client 2 into it.

2.	The MCData server verifies whether the MCData client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCData server sends modify ad hoc group call participants response to the MCData client 1.

4.	The MCData server sends the ad hoc group call leave request to the MCData client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCData client 3 notifies the user of the ad hoc group call leave request.

6.	The MCData client 3 sends the ad hoc group call leave response to the MCData server.

7.	The MCData server removes the MCData client 3 from the ongoing ad hoc group call.

8.	The MCData server sends the ad hoc group call request towards MCData client 2.

NOTE:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCData client 2 notifies the user about the incoming ad hoc group call.

10.	The MCData client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

11.	The MCData server may notify the initiating MCData user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCData user by the MCData server more than once during the call when MCData users join or leave the ad hoc group call.

12.	The MCData server may notify the participants about the change in the participants list of on-going ad hoc group call.

##### 7.17.3.1.5	Modification of ad hoc group call participants by the MCData server

Figure 7.17.3.1.5-1 below illustrates the modification of ad hoc group call participants procedure by the MCData server.

Pre-conditions:

1.	The MCData client 1 is the initiator of the ad hoc group call.

2.	MCData server determined the participants for the ad hoc group call based on the criteria specified by the MCData client 1 while initiating the ad hoc group call.

3.	MCData server continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

4.	The MCData server detects that the MCData client 5 satisfies the criteria and MCData client 4 stops to meet the criteria specified by the MCData client 1.

Figure 7.17.3.1.5-1: Modification of ad hoc group call participants by the MCData server

1.	The ad hoc group call is established and on-going with the participants MCData client 1, MCData client 2, MCData client 3 and MCData client 4. The participants list is determined by the MCData server based on the criteria specified by the MCData client 1 while initiating the call

2.	The MCData server detects that the MCData client 5 satisfies the criteria specified by the MCData client.

3.	The MCData server sends the ad hoc group call request to the MCData client 5.

4.	The MCData client 5 notifies the user about the incoming ad hoc group call.

5.	The MCData client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCData server.

6.	The on-going ad hoc group call is updated by adding MCData client 5 which satisfies the criteria specified by the MCData client 1.

7.	The MCData server detects that the MCData client 4 is no more satisfying the criteria to be the participant of the ad hoc group call.

8.	The MCData server sends the ad hoc group call leave request to the MCData client 4 and removes it from the on-going ad hoc group call.

9.	The MCData client 4 notifies the user of the ad hoc group call leave request.

10.	The MCData client 4 sends the ad hoc group call leave response to the MCData server.

11.	The on-going ad hoc group call is updated by removing MCData client 4, which no more satisfies the criteria specified by the MCData client 1.

### 7.17.4	Ad hoc group short data service data communication procedures

#### 7.17.4.1	General

This subclause contains procedures for ad hoc group short data service data communication across a single MCData system, and uses the common procedures defined in the subclause 7.17.3 in conjuction with the procedures defined in this subclause. Other common procedures specified in the 7.17.3 are used at any time after Ad hoc group short data service data communication establishment based on the need of it. These procedures are applicable for Group standalone short data service using media plane and Group short data service session as specified in the subclause 7.4.2.6 and subclause 7.4.2.7 respectively.

#### 7.17.4.2	Information flows for short data service specific

Table 7.17.4.2-1 describes the information elements that are specific to short data service and are used along with the common information flows defined in the subclause 7.17.2 for the Ad hoc group short data service data communication. These information elements are included when an Ad hoc group call request sent from the MCData client to the MCData server, between the MCData servers, and from the MCData server to the MCData client.

Table 7.17.4.2-1: short data service specific

| Information element | Status | Description |
| --- | --- | --- |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Application identifier (see NOTE 1) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| NOTE 1: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

#### 7.17.4.3	Ad hoc group short data service data communication procedures in single MCData system

##### 7.17.4.3.1	General

The initiation of Ad hoc group SDS session results in ad hoc group participants exchanging SDS data.

##### 7.17.4.3.2	Procedure

The procedure in figure 7.17.4.3.2-1 describes the case where an MCData user is initiating SDS data communication session with an MCData ad hoc group for exchanging SDS data transactions between the ad hoc group participants, with or without disposition request.

Pre-conditions:

1.	As defined in the subclause 7.17.3.1.1.

Figure 7.17.4.3.2-1: Ad hoc group SDS data communication session

1.	User at MCData client 1 initiate an ad hoc group call as specified in the subclause 7.17.3.1.1.

2.	MCData client 1 and the ad hoc group call request accepted clients have successfully established media plane for data communication and either MCData client can transmit SDS data. The MCData data request may contain disposition request if indicated by the client sending data. If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the SDS data receiving MCData users may be notified, otherwise those MCData users shall not be notified.

3.	If MCData data disposition was requested by the user, then the SDS data receiving MCData client initiates a MCData data disposition notification for delivery, read reports to the disposition requesting user. The MCData data disposition notification from the receiving MCData clients may be stored by the MCData server for disposition history interrogation from authorized users.

4.	Based on the MCData user action or conditions to release, the established media plane for SDS data exchange is released as specified in the subclause 7.17.3.1.2.

### 7.17.5	Ad hoc group file distribution communication procedures

#### 7.17.5.1	General

This subclause contains procedures for ad hoc group file distribution communication across a single MCData system, and uses the common procedures defined in the subclause 7.17.3 in conjuction with the procedures defined in this subclause. Other common procedures specified in the 7.17.3 are used at any time after Ad hoc group file distribution communication establishment based on the need of it.

#### 7.17.5.2	Information flows for file distribution specific

Table 7.17.5.2-1 describes the information elements that are specific to file distribution and are used along with the common information flows defined in the subclause 7.17.2 for the Ad hoc group short data service data communication. These information elements are included when an Ad hoc group call request sent from the MCData client to the MCData server, between the MCData servers, and from the MCData server to the MCData client.

Table 7.17.5.2-1: file distribution specific

| Information element | Status | Description |
| --- | --- | --- |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed report is expected or not |
| Download indication | O | Indicates mandatory download (i.e. auto accept this media plane setup request) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |

#### 7.17.5.3	Ad hoc group file distribution communication procedures in single MCData system

##### 7.17.5.3.1	General

The initiation of Ad hoc group standalone FD using media plane results in ad hoc group participants exchanging SDS data.

##### 7.17.5.3.2	Procedure

The procedure in figure 7.17.5.3.2-1 describes the case where an MCData user is initiating ad hoc group standalone data communication for sending file to multiple MCData users, with or without download completed report request.

Pre-conditions:

1.	As defined in the Common Ad hoc group call procedures of subclause 7.17.3.1.1.

Figure 7.17.5.3.2-1: Ad hoc group SDS data communication session

1.	User at MCData client 1 initiate an ad hoc group call as specified in the subclause 7.17.3.1.1.

2.	MCData client 1 and MCData server have successfully established media plane for file transmission and the MCData client 1 transmits the file data. MCData server distributes the file received from MCData client 1 to MCData clients 2 to n over the established media plane. Distribution of file can be via unicast or via MBMS bearer(s).

3.	The dispositions notifications are handled as specified in the subclause 7.5.2.7.

4.	Based on the MCData user action or conditions to release, the established media plane for FD communication is released as specified in the subclause 7.17.3.1.2.

* * * End of Changes * * * *