3GPP TSG-SA WG6 Meeting #53	S6-230714

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0116 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MSGin5G message deferred delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposed to fulfill the stage 1 requirement Specified in 3GPP TS22.262: [R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available. This feature applies to the MSGin5G Server in the terminating side based on the registration status of the recipient and the service policy. It is a different feature with the Store and Forward specified in the current TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the deferred delivery procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement [R-5.1.2-005] in TS22.262 cannot be fully fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.x	MSGin5G deferred delivery

This procedure applies when an MSGin5G outbound message is received at the MSGin5G Server in the terminating side and the recipient UE is not registered, the MSGin5G Servicer in the terminating side may perform the procedure for deferring messages and pushing the deferred messages once the target recipient becomes registered.

Figure 8.3.x-1 shows the procedure for the MSGin5G deferred delivery.

Pre-conditions:

1.	The MSGin5G Client or Application Server has registered to the MSGin5G Server at least once and the MSGin5G Client Profile or Application Server Profile is still stored on the MSGin5G Server (these information has not been deleted due to the registration expiration).

2.	The recpient MSGin5G Client is not registered and deferred delivery is supported based on the service policy.

Figure 8.3.x-1: Store and forward procedure

1.	The MSGin5G outbound message procedure is started on the MSGin5G Server in the terminating side. The MSGin5G Server checks the registration status of the recpient and service policy, and then decides that the deferred delivery is needed. The MSGin5G Server stores the message.

2.	Before the message expiration, the recipient MSGin5G UE registers on the MSGin5G Server as specified in clause 8.2.1.

3.	MSGin5G Server in the terminating side delivers the stored MSGin5G message to the recipient MSGin5G UE as specified in clause 8.3.3.