3GPP TSG-SA WG6 Meeting #53	S6-230751

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0183 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS service area in pre-conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The pre-condition in cl. 14.3.4A.6.2 is not fully correct as the procedures covers normal multicast MBS case which is not restricted by the service area. - VAL UE 2 to n are within the MBS service area where the MBS session is configured. In clause 14.3.4A.6.2 step 1, the MBS session announcement is also being restricted “within the service area” which is also not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Modify the pre-condition to “VAL UE 2 to n are within the MBS service area (if the session is limited to an area) where the MBS session is configured.” 2) update the clause 14.3.4A.6.2 step 1 description on the session announcement to remove the service area restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusions will be caused to readers on how the MC service server use the normal multicast MBS session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.4A.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** 1st change **********************/

##### 14.3.4A.6.2	Procedure

The procedure in figure 14.3.4A.6.2-1 describes how media related to a specific group communication can be distributed over a configured MBS session which consist of multiple QoS flows, i.e. addressing different service requirements.

Pre-conditions:

-	VAL UE 1 to n are attached to the 5GS, registered and belong to the same VAL service group X.

-	The VAL server has decided to use an MBS session for VAL service group communications associated to VAL service group X.

-	VAL UE 2 to n are within the MBS service area (if the session is limited to an area) where the MBS session is configured.

Figure 14.3.4A.6.2-1: VAL service group media transmission over MBS sessions

1.	The VAL server creates a multicast or a broadcast MBS session targeting group communications associated to VAL service group X via the NRM server, as being specified in clause 14.3.4A.2. Therefore, the VAL server can provide default service requirements to be addressed by the MBS session.

The MBS session is announced by NRM server and received by NRM client 2 to n. The NRM server has identified that VAL UE 2 to n can receive media over the MBS sessions, e.g. based on a notification from NRM clients indicating the successful join of the multicast MBS session or a monitoring report of the broadcast MBS session (similar to the listening status report used for MBMS).

2.	A new VAL group communication is established for the VAL service group X consisting of a specific required service requirements, e.g. a VAL service emergency group communication. The group communication setup can be done over unicast.

2a. For broadcast MBS sessions, the session is established as described in 3GPP TS 23.247 [39].

2b. For multicast MBS session, the session is established upon the acceptance of the first UE session join request initiated from the VAL UE towards the 5GS, as described in 3GPP TS 23.247 [39]. The multicast session can then have either an active or an inactive state.

3. 	The VAL server may initiate the multicast MBS activation towards the NRM as described in clause 14.3.4A.5 in order to activate the multicast MBS session in case the session has an inactive state.

4.	Considering that the established group communication requires a specific QoS, e.g. an VALemergency group communication which requires higher priority (i.e. better ARP), the VAL server initiates an MBS session update to the NRM to provide the new required service requirements, if not done during the MBS session creation in step 1, as described in clause 14.3.4A.2. The MBS session should then be updated and an additional QoS flow may be configured.

5.	The NRM server sends a MapGroupToSessionStream to NRM clients 2 to n over the configured MBS session providing the required stream information to receive the media related to the specific established VAL group communication within the MBS session.

6.	NRM clients process the MapGroupToSessionStream information to receive the related media over the specific MBS session stream.

7.	VAL client 1 sends media to the VAL server over unicast to be distributed for the established group communication.

8.	The VAL server distributes the media to VAL clients 2 to n over the indicated stream within the established MBS session.

9.	The VAL server may initiate the multicast MBS session deactivation towards the NRM as described in clause 14.3.4A.5, in order to deactivate the multicast MBS session.

10.	The NRM server may further trigger the UE to leave multicast MBS session.

/******************** end of 1st change **********************/