3GPP TSG-SA WG6 Meeting #53	S6-230765

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0042 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to Realtime UAVs status for DAA support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To provide support for DAA applications, it was agreed to enhance the procedure for Realtime UAVs status as per clause 7.7.2.2 in the TR 23.700-55. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhanced the procedure and information flow for Real-time UAV status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | USS (UASS) may not be able to receive Real-time UAV status with list of UAVs for handling DAA aspects. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.2, 7.5.2.3, 7.5.3.2, 7.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.5.2.2	Subscription for real-time UAV status information

Figure 7.5.2.2-1 describes the procedure for subscription for real-time UAV status information.

Pre-condition:

-	UAS application specific server has been provisioned with UAE server information.

Figure 7.5.2.2-1: Subscription for real-time UAV status information

1.	The UAE application specific server sends subscribe real-time UAV status information request to the UAE server. The request includes the list of UAV IDs.

2.	The UAE server stores the subscription information.

3.	The UAE server sends subscription response to the UAS application specific server.

#### 7.5.2.3	Notification of real-time UAV status information

Pre-conditions:

-	UAS application specific server has performed subscription as per procedure in clause 7.5.2.2 with UAE server and the procedure for processing real-time UAV status as specified in clause 7.5.2.1 has performed.

Figure 7.5.2.3-1: Notification for real-time UAV status information

1.	When real-time UAV status information is available at the UAE as per the subscription then, the UAE server sends notification of real-time UAV(s) status information to the UAS application specific server.

* * * Next Change * * * *

#### 7.5.3.2	Subscribe real-time UAV status information request

Table 7.5.3.2-1 describes the information flow for a UAS application specific server to subscribe to real-time UAV status information at the UAE server.

Table 7.5.3.2-1: Subscribe real-time UAV status information request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | The identifier of the UAS application specific server which initiated this request. |
| List of UAVs | M | The list of UAVs for which real-time UAVs status is requested. |
| > UAV ID | M | The identifier of the UAV (e.g. 3GPP UE ID or CAA level UAV ID) for which the real-time UAV status is requested. |
| Area of Interest | M | Geographic area location information where the UASS server wishes to monitor the UAS's location adherence. |
| Notification Target URI | M | Target URI where the UAS application specific server wishes to receive the notifications about real-time UAV status information. |

* * * Next Change * * * *

#### 7.5.3.4	Notify real-time UAV status information

Table 7.5.3.4-1 describes the information flow for a UAS application specific server to receive notification about real-time UAV status information from the UAE server.

Table 7.5.3.4-1: Notify real-time UAV status information

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the subscription for this notification. |
| List of Real-time UAV status information | M | The list of real-time UAV status information |
| >UAV ID | M | The identifier of the UAV (e.g. 3GPP UE ID or CAA level UAV ID) for which the real-time UAV status information is notified. |
| >UAV status information | M | The UAV status information includes the UAV network connection status information, location information and timestamp. |

* * * Next Change * * * *