3GPP TSG-SA WG6 Meeting #53	S6-230767

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0076 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding information flows and editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Information flow for clause 9.20.2 are to be developed. Some editorial issues exist which needs to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information flows are proposed Editorial corrections are proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature cannot be implemented by stage 3 without information flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.19.2.1, 9.20.2, 9.20.2.1 (new) to 9.20.2.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.19.2.1	Session-oriented service trigger request

Table 9.19.2.1-1 describes the information flow session-oriented service trigger request from the V2X application specific server to the VAE server.

Table 9.19.2.1-1: Session-oriented service trigger request

| Information element | Status | Description |
| --- | --- | --- |
| Remote V2X UE ID | M | Identity of the V2X UE (VAE client) which is the remote vehicle. |
| V2X service ID | M | The V2X service ID for which application requirement corresponds to. |
| V2X application specific server identity information | M | Identity information of the V2X application specific server. |
| Session ID | O | The session identifier to be used for the session-oriented service. |
| V2X application QoS requirements for the session | O | The application QoS requirements (reliability, delay, jitter) for the session-oriented service. |

* * * Next Change * * * *

### 9.20.2	Information flows

#### 9.20.2.1	Service adaptation and QoS control subscription request

Table 9.20.2.1-1 describes the information flow Service adaptation and QoS control subscription request from the V2X application specific server to the VAE server.

Table 9.20.2.1-1: Service adaptation and QoS control subscription request

| Information element | Status | Description |
| --- | --- | --- |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the service adaptation and QoS control subscription is initiated. |
| V2X service ID | M (NOTE) | The V2X service ID for which subscription corresponds to. |
| V2X UE ID | M (NOTE) | The V2X UE ID for which subscription corresponds to. |
| NOTE: Either of the information element should be present. |  |  |

#### 9.20.2.2	Service adaptation and QoS control subscription response

Table 9.20.2.2-1 describes the information flow for VAE server to respond for Service adaptation and QoS control subscription request from the V2X application specific server.

Table 9.20.2.2-1: Service adaptation and QoS control subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAE server in response to subscription request indicating success or failure |
| Subscription ID (NOTE) | O | Identifier of a successful subscription. |
| NOTE: This IE is included when the Result indicates success |  |  |

#### 9.20.2.3	Service requirement adaptation notification request

Table 9.20.2.3-1 describes the information flow for service requirement adaptation notification request from VAE server to the V2X application specific server.

Table 9.20.2.3-1: Service requirement adaptation notification request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the subscription for this notification. |
| List of V2X UEs information | O | The information of the V2X UEs which are affected by this service requirement adaptation notification |
| > V2X UE ID | M | The identifier of V2X UE |
| QoS change information | M | The actual or expected QoS change information |

#### 9.20.2.4	Service requirement adaptation notification response

Table 9.20.2.4-1 describes the information flow for service requirement adaptation notification response from V2X application specific server to the VAE server.

Table 9.20.2.4-1: Service requirement adaptation notification response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the subscription for this notification. |
| Acknowledgement Result (NOTE) | M | Indicates positive or negative acknowledgement. |
| List of V2X UEs information (NOTE) | O | The information of the V2X UEs which are affected by the service requirement adaptation and requires QoS change to be applied. |
| > V2X UE ID | M | The identifier of V2X UE |
| NOTE: If Acknowledgement Result IE indicates positive, then List of V2X UEs information IE is included. |  |  |