3GPP TSG-SA WG6 Meeting #53	S6-230796

Athens, Greece, 27th Ferbruary – 3rd March 2023	(revision of S6-230629)

Title:	LS on SA6 V2X service support

Response to:

Release:	Release 18

Work Item:	V2XAPP, ADAES, SNAAPP

Source:	3GPP TSG SA WG6

To:	SA

Cc:	SA1, SA2

Contact person:	Name: Emmanouil Pateromichelakis

E-mail Address:	epateromiche@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: TS 23.286, TS 23.436, TR 23.700-95

# 1	Overall description

In SA6#52-bis-e, SA6 received (in cc) a Reply LS on QoS Sustainability analytics and V2X service adaptations (SP-221320) which provided a consolidated answers from SA1 and SA2 on the 5GAA LS on QoS Sustainability analytics and V2X service adaptations (S-220077). The 5GAA LS was pointing to a technical report (5GAA_T-220058_WI-PRESA-Technical-Report), which included some requirements and assumptions on the current 3GPP support to support these requirements.

After careful review of the technical report and the questions, we found that some concepts described in the technical report and some requirements for 3GPP can be applicable to 3GPP SA6 work. Therefore, we would like to provide the latest updates of SA6 related work.

At first to note, that SA6 provides enablement capabilities related to end to end QoS (end to end QoS: QoS between the application entities in the UE and the application server), whereas SA2 handles the network QoS (from UE to network) aspects. The current “QoS prediction” in the provided TR discusses the service level requirements and QoS predictions which can be also applicable to SA6 notion of QoS.

SA6 has ongoing normative work on some topics with relevance to the 5GAA work:

- V2XAPP (TS 23.286, TR 23.700-64): The R18 study also takes into consideration the V2X application specific services and recommendations specified by automotive industry bodies (5GAA, ETSI ITS and AECC) related to advanced V2X services for VRU, Haptics/ToD and edge deployments for V2X services; and provides capabilities (among others) related to high risk VRU zones, V2X service deployment in edge data network, usage of network analytics and enhanced network monitoring to support advanced V2X services

- ADAES (TS 23.436, TR 23.700-36): ADAES is a new enabler service which provides application layer analytics including end to end performance predictions and application QoS sustainability to be consumed by verticals. Such predictions do not only cover the UE to server sessions, but also the UE-to-UE application sessions.

- SNAAPP:, SNAAPP (TR 23.700-95, TS 23.222) studied the potential enhancements in CAPIF and application enablement frameworks (e.g. SEAL, EDGEAPP, vertical enabler layers) to support the subscriber-aware northbound API access (SNA), whose requirements are specified in TS 22.261. In particular, for the scenario related to QoS, the study discussed the use case where an end user (also a subscriber of the MNO) allows the AF to invoke the QoS API (offered by MNO) to modify the QoS of the end user. Such capability can allow the UE to directly consume NEF services (e.g. for getting NWDAF analytics as well as QoS notifications).

5GAA_T-220058_WI-PRESA-Technical-Report also lists in Table 33 some requirements and SA6 would like to provide the possible addressing of the requirements, since the assessment of 3GPP status seems not to include the SA6 viewpoint.

| Requirement No. | Requirements of QoS Prediction | 3GPP SA6 Status |
| --- | --- | --- |
| Req.1 | Increase granularity of QoS predictions (e.g. below cell level, more KPI reporting, richer information, aware of the specific UE context) also by means of more input data to the analytics enabler (e.g. from core network, RAN, etc.) | Complementary to NWDAF functionality in 5GC, QoS prediction is also supported in ADAES work item from application layer. Such prediction can be in finer granularities. Please see TS 23.436 clause 6.1 and 8.2. More specifically, the ADAES feature allows the application server to collect QoS related events or online analytics from the VAL UEs, which happen in real time, and allow the per session QoS analytics (for example prediction of RTT deviation for V2X session #1). Such granularity of reporting can be customized based on the VAL customer needs, without being constrained by the network layer granularities. |
| Req. 2 | Improve freshness of QoS predictions (e.g. notice period) | Complementary to NWDAF functionality in 5GC, which supports early notifications from 5GC side, in ADAES related feature (TS 23.436), the notice period can be tailored to vertical needs including also input from the V2X UEs, based on the VAL server requirements. |
| Req. 3 | Enable QoS predictions also of the PC5/Sidelink Interface | This is supported from application layer between two or more VAL UEs. More details can be found in TS 23.436 clause 6.3 and 8.4. |
| Req. 4 | Investigate QoS prediction in Multi-MNO/Cross-border environments | Even if SA6 doesn’t specifically support the QoS prediction for multi-operator scenarios, V2XAPP supports multi-MNO interactions, since the enabler servers (e.g. VAE server, ADAES) sit on top of the network and there are deployments where enabler layer communicates with more than one PLMNs. In V2XAPP, from R17 there is support for multi-MNO service provisioning for multi operator V2V services. Please review TS 23.286 and TS 23.436 and provide any additional feedback to satisfy this requirement. |
| Req. 5 | Investigate the need for coordinating notifications of QoS prediction among V2X Servers (e.g. ordering, prioritization) | V2XAPP layer and well as SEAL/ADAE layer could support such coordination since it allows multiple V2X / VAL servers to connect simultaneously to a VAE server via standardized APIs. Possible consideration of such scenario is not discussed in Release 18 time frame; however V2XAPP layer can be seen as a placeholder for coordinating notifications among V2X servers since VAE layer may be collocated with V2X server or may connect to multiple V2X servers simultaneously. |

# 2	Actions

To SA

ACTION:

SA6 kindly requests SA to take into account SA6 status related to supporting 5GAA requirements and forward this LS on behalf of 3GPP to 5GAA.

# 3	Dates of next TSG SA WG 6 meetings

SA6#54-e              17th April – 26th April 2023 	e-meeting

SA6#55                 22nd May – 26th May 2023 	Berlin, DE