3GPP TSG-SA WG6 Meeting #53	S6-230798

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230760)

Title:	LS on addressing packet loss during multicast MBS delivery

Response to:

Release:	Rel-17

Work Item:	MCOver5MBS, MBS_Ph2

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG2

Cc:	3GPP TSG CT WG3

Contact person:	Yanmei Yang

yangyanmei@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

SA6 identifies that packet loss could happen in multicast MBS delivery due to AF being not aware of the multicast MBS session state in some cases, e.g.:

-	A talker starts to speak to the group after a silent period with no DL media. The media packet from the talker needs to be delivered via the multicast MBS session.

-	The multicast MBS session is configured and the group members have joined in advance. After a while, the application layer signalling, e.g., group call request, MapGroupToSessionStream, floor taken needs to be delivered via the multicast MBS session.

During the silent period without DL media, the multicast MBS session used for this group communication might become inactive and the joined UEs become IDLE. The new DL packets trigger the multicast MBS session being activated and the IDLE UEs being paged. However, not all the UEs become CONNECTED at the same time. The late CONNECTED UEs will lose the packet as the NG-RAN will start to transmit the packet when the first UE becomes CONNECTED which triggers the shared delivery restoring again.

SA6 would like to request SA2 to provide their feedback on how to address the above scenario.

# 2	Actions

To SA2

ACTION: 	SA6 kindly asks SA2 to provide their feedback on the above scenario.

# 3	Dates of next TSG SA WG 6 meetings

SA6#54                  17th April – 26th April 2023 	e-meeting

SA6#55                 22nd May – 26th May 2023 	Berlin, Germany