3GPP TSG-SA WG6 Meeting #53	S6-230801

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230762)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0100 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for usage of term MCX to MC service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The usage of the term "MCX" should be replaced by the term "MC service" in TS 23.289. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting all appearances of MCX to MC service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsistency in the specification for the usage of the term related to MC service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 4.7.2, 4.7.3.1, 4.7.3.2, 4.7.3.3, 4.7.4, 4.7.5, 7.3.1, 7.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.7.1	General

The main purpose of 5G Multicast-Broadcast Service (MBS) use by mission critical services is to provide efficient downlink delivery of user traffic in group calls and communications. The architectural figures in this clause are aligned  with the 5GS architecture for MBS shown in Figure 5.1-2 of 3GPP TS 23.247 [15], which identifies both mandatory and optional functional entities and interfaces, in reference point representation, available for use by the MC services.

Multicast and broadcast services in 5G for MC group communications rely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the MC service server to multiple MC users (i.e., users affiliated to a certain MC group) is supported either as point-to-point or point-to-multipoint over the radio. The MBS sessions can either be of broadcast or multicast type and consist of one or multiple QoS flows for different service requirements.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

Within this arrangement, the MC service server decides whether to create broadcast or multicast MBS sessions to be associated with certain MC groups. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the MC service UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED state.

The following capabilities (non-exhaustive list) provided by MBS could be used by MC service servers:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Transparent MBS Data forwarding;

-	Dynamic PCC control for MBS session.

### 4.7.2	General on-network architecture for use of MBS by MC services

Figure 4.7.2-1 presents a high-level architectural view of mission critical services when using MBS. The shown architecture is consistent with 3GPP TS 23.501 [7] and 3GPP TS 23.247 [15].

MC services use MBS control plane capabilities by initiating access via Nmb13, Nmb10 or N33. MBS user plane capabilities can be accessed via N6mb or Nmb8. MC service servers can initiate access to MBS PCC capabilities supported by PCF via N5 or N33. If the MC service server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS control plane capabilities and the PCC capabilities.

The 5G-GC1 reference point, which exists between the MC service client and the MC service server, is used for application layer signalling for the control of mission critical service delivery over MBS session. The functions of this reference point are defined in clause 7.3.

Figure 4.7.2-1: Architectural view of a mission critical system when using MBS

NOTE 1:	Support of interfaces associated to 5GS optional entities (e.g. MBSF, MBSTF, NEF) is necessary only if features enabled by these entities are supported.

NOTE 2:	When the MC service server uses MBS, the N5 reference point is used as described in the present document.

* * * Next Change * * * *

#### 4.7.3.1	Instantiation without optional entities and associated interfaces

Figure 4.7.3.1-1 presents a high-level architectural view of mission critical services when using MBS without the presence or use of the optional entities (MBSF, MBSTF and NEF) and their associated interfaces. The shown architecture is a particularization of the general architecture shown in figure 4.7.3.1-1.

MC services use MBS control plane capabilities by initiating access via Nmb13. MBS user plane capabilities can be accessed via N6mb. MC service servers can initiate access to MBS PCC capabilities supported by PCF via N5.

Figure 4.7.3.1-1: Architectural view of a mission critical system when using MBS without optional MBS interfaces

#### 4.7.3.2	Instantiation without MBSF / MBSTF and associated interfaces

Figure 4.7.3.2-1 presents a high-level architectural view of mission critical services when using MBS without the presence or use of the optional entities MBSF and MBSTF and their associated interfaces. The shown architecture is a particularization of the general architecture shown in figure 4.7.2-1.

MC services use MBS control plane capabilities by initiating access via Nmb13 or N33. MBS user plane capabilities can be accessed via N6mb. MC service servers can initiate access to MBS PCC capabilities supported by PCF via N5 or N33. If the MC service server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS control plane capabilities and the PCC capabilities.

Figure 4.7.3.2-1: Architectural view of a mission critical system when using MBS without optional MBSF/MBSTF entities and their associated interfaces

#### 4.7.3.3	Instantiation without NEF and associated interfaces

Figure 4.7.3.3-1 presents a high-level architectural view of mission critical services when using MBS without the presence or use of the optional entity NEF and its associated interfaces. The shown architecture is a particularization of the general architecture shown in figure 4.7.2-1.

MC services use MBS control plane capabilities by initiating access via Nmb13 or Nmb10. MBS user plane capabilities can be accessed via N6mb or Nmb8. MC service servers can initiate access to MBS PCC capabililities supported by PCF via N5.

Figure 4.7.3.3-1: Architectural view of a mission critical system when using MBS without optional NEF entity and its associated interfaces

### 4.7.4	Service layer-based interworking between eMBMS and MBS

Figure 4.7.4-1 presents a high-level architectural view of mission critical services interworking between eMBMS and MBS at the service layer. The shown architecture is consistent with 3GPP TS 23.247 [15], subclauses 5.2, 6.8 and configurations 2 and 3 in Annex A.

The interworking between eMBMS and MBS for mission critical operation is enabled by the Joint BM-SC, MBSF and MBSTF functional entity. MC services can use control plane capabilities by accessing the Joint entity directly via MB2-C or Nmb10 or indirectly (using NEF) via N33+Nmb5. User plane traffic delivery is supported via MB2-U or Nmb8. If the MC service server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS PCC capabilities.

Figure 4.7.4-1: Service layer-based mission critical interworking between eMBMS and MBS

NOTE:	Support of interfaces associated to 5GS optional entities (e.g., NEF) is necessary only if features enabled by these entities are supported.

### 4.7.5	Application layer based interworking between eMBMS and MBS

Figure 4.7.5-1 presents a high-level architectural view of mission critical services interworking between eMBMS and MBS at the application layer. The shown architecture does not use the MBSF/MBSTF entities defined in 3GPP TS 23.247 [15] and is inclusive of configuration 1 in Annex A of 3GPP TS 23.247 [15].

MC services initiate access to control plane capabilities via MB2-C (for eMBMS) and via Nmb13 or N33 (for MBS). User plane capabilities can be accessed via MB2-U (for eMBMS) and via N6mb (for MBS). MC service servers can initiate access to PCC capabilities via the Rx interface (for the PCRF in the EPS) and via the N5 or N33 interfaces (for the PCF in the 5GS). If the MC service server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS control plane capabilities and the PCC capabilities.

Figure 4.7.5-1: Application layer-based mission critical interworking between eMBMS and MBS

NOTE:	Support of interfaces associated to 5GS optional entities (e.g., NEF) is necessary only if features enabled by these entities are supported.

* * * Next Change * * * *

### 7.3.1	General

This subclause defines information flows and procedures for 5G MBS usage that applies to MC services. 5G MBS session can be used by any MC service for any MC service group.

The following subclauses specify the procedures and information flows for the usage of 5G MBS transmission that are utilized by the following MC services:

-	MCPTT (as specified in 3GPP TS 23.379 [6]);

-	MCVideo (as specified in 3GPP TS 23.281 [4]); and

-	MCData (as specified in 3GPP TS 23.282 [5]).

MC service specific pre-requisites and resultant behaviour by functional entities in performing these procedures are specified in the respective MC service TSs as listed above.

The first phase to utilize MBS sessions for MC service media transmission is to have the sessions created hence the network resources are reserved. The MC service server may consider the UE's capabilities and service related information e.g., UE's MBS capabilities, location, MBS listening status report sent by group members when it decides to create or use MBS sessions. The MC service server needs to interact with the 5GC for this matter. During the interaction, the necessary information related to the requested session is determined, e.g., MBS session mode (either a broadcast or a multicast session) and the required QoS profile. This interaction depends on the configuration option under consideration, i.e., whether the MC service server is in trusted domain (limited operations), and whether the session creation is done with or without a dynamic PCC rule.

NOTE 1:	It is implementation specific whether the MC service server decides to use multicast or broadcast MBS sessions.

NOTE 2:	It is implementation specific whether the MC service server decides to create (one or multiple) MBS sessions for MC media for MC group communications associated to a certain MC group or create (one or multiple) dynamic MBS sessions once the need has emerged, e.g., dynamic MBS sessions to be associated for an ad hoc group.

NOTE 3:	It is implementation specific whether an MBS session is associated to one or multiple MC groups, and whether it is re-assigned to other MC groups.

NOTE 4:	How the MC service server uses the UE's capabilities and service related information in order to create and use the MBS session is implementation specific.

The information elements describing the MBS session under consideration is then sent to the MC service clients via MBS session announcement, where the latter need to react according to the announced session mode.

If eMBMS and 5G MBS co-exist for MC services, the MC service server may decide to trigger the establishment of an eMBMS bearer to deliver the MC media associated to the MC service group communications, if the target MC service group(s) consists of members with MBMS capable UE. As a result, the MC service server subsequently needs to send an eMBMS bearer announcement towards the clients camping on LTE.

NOTE 4:	It is implementation specific whether the MC service server triggers an eMBMS bearer or a unicast bearer to serve MC service clients camping on LTE.

* * * Next Change * * * *

#### 7.3.2.1	MBS session announcement

Table 7.3.2.1-1 provides the information elements during MBS session announcement, which are sent by the MC service server to the clients. The MBS session announcement includes information elements related to the announced MBS session. Optionally, it includes eMBMS related information elements, if eMBMS and 5G MBS co-exist.

Table 7.3.2.1-1: MBS session announcement

| Information element | Status | Description |
| --- | --- | --- |
| 5G MBS session information | M | Providing the MBS session related information if MC service server decides to use 5G MBS session to deliver MC service group communication data |
| >MBS session ID | M | The identity of the MBS session used to deliver MC service group communication data. It is either TMGI for broadcast MBS and multicast MBS sessions, or source specific IP multicast address for multicast MBS session |
| >MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| >MC service group ID | O | Indicate the MC service group ID associated to the MBS session |
| >MBS related SDP information | M | SDP related to application-level control signalling or media to be transmitted over the MBS session (e.g., codec, protocol ID, FEC information, IP address and ports) |
| >List of MBS Service Area information (see NOTE 5) | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| >MBS session announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS session announcement |
| >Multicast MBS session related information (see NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session such as PLMN ID of the default PLMN service provider in case of source specific IP multicast address, DNN, and SNSSAI of the PDU session associated with the multicast MBS session |
| > UE session join notification (see NOTE 2) | O | Indicate if the MC service server requires a notification from the MC service client once it has joined the multicast MBS session |
| >Monitoring state | O | Indicate if the MC service client is required to actively monitor the MBS session quality and report it to the MC service server. This is applicable for both multicast and broadcast MBMS session. |
| >Frequency (see NOTE 3) | O | Identification of frequency associated with a broadcast MBS session, if multi carrier support is provided |
| >MBS Frequency Selection Area ID (MBS FSA ID) (see NOTE 3) | O | The frequency associated to a certain broadcast area, if multi carrier support is provided |
| eMBMS bearer information | O | Providing the 4G eMBMS bearer related information if MC service server decides to use 4G eMBMS additionally with 5G MBS session to deliver MC service group communication data |
| >TMGI (see NOTE 4) | M | TMGI information |
| >Alternative TMGI | O | A list of additional alternative TMGI may be included and used in roaming scenarios |
| >QCI | O | QCI information used by the ProSe UE-Network Relay to determine the ProSe Per-Packet Priority value to be applied for the multicast packets relayed to Remote UE over PC5 |
| >List of service area identifier | M | A list of service area identifier for the applicable MBMS broadcast area |
| >Frequency | O | Identification of frequency if multi carrier support is provided |
| >eMBMS related SDP information | M | SDP with media and floor control information applicable to groups that can use this eMBMS bearer (e.g., codec, protocol id, FEC information) |
| >Monitoring state | O | Indicate if the MC service client is required to actively monitor the eMBMS bearer quality and report it to the MC service server |
| >ROHC information | O | Indicate the usage of ROHC over the eMBMS bearer and provide the parameters of the ROHC channel to signal to the ROHC decoder |
| NOTE 1: Such information may be pre-configured in the MC service UE, or provided in any other implementation specific way NOTE 2: It is applicable for multicast MBS session NOTE 3: It is applicable for broadcast MBS session NOTE 4: TMGI for 4G eMBMS bearer can be the same or different with 5G MBS session ID. NOTE 5: Details of MBS service area information is defined in 3GPP TS 23.247 [15]. |  |  |