3GPP TSG-SA WG6 Meeting #53	S6-230804

Athens, Greece, 27th February– 3rd March 2023	(revision of S6-230573)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0360 | rev | 3 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to MC gateway UE and identities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, AT&T, Nokia, Nokia Shanghai Bell, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR provides an update to clause 11.3, including the underlying subclauses, to avoid confusion related to identities behind the MC gateway UE. The present text in clause 11.3.3 defines concepts that are similar to the case in clause 11.3.2. However, Figure 11.3.3-1 is not aligned with the text above. Whether there is a common IMS subscription or not, the same concepts apply. This is due to the fact that each MC service client has a unique set of identities, e.g., IMC (see clause 4.3.3 in 3GPP TS 23.228), and a unique IP address via Framed route (see clause 11.4). Hence, implicit (de-)registration described in 3GPP TS 23.228 does not apply in such scenarios. However, this is different from the case when MC GW UE provides access to user residing on non-3GPP devices which cannot host a client. For this case, as the 3GPP devices does not host a client, i.e., does not have IMS subscription, they rely on the IMS identities and other information for SIP signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating clauses 11.3.1, 11.3.2 and 11.3.3 to avoid confusion related to MC service clients behind the gateway UE. - Updating Figure 11.3.2-1 to include both cases when the MC service clients behind the MC GW UE have common or separate IMS subscription. - Removing Figure 11.3.3-1 since there is no identities behind the MC GW UE, and replacing it with a text defining this special case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion related to identities behind MC gateway UE, and aspects related to IMS subscriptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1, 11.3.2, and 11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 11.3	Using identities behind the MC gateway UE

11.3.1	General

The MC gateway UE enables access to the MC system for MC service clients either hosted on the MC gateway UE or hosted on non-3GPP devices using non-3GPP connectivity with the MC gateway UE. Simultaneous sharing of an MC gateway UE by multiple MC service clients requires a unique relationship between the MC service identities used by the MC service clients with corresponding media streams passing the MC gateway UE towards the MC system sharing 3GPP connectivity of the MC gateway UE.

An MC service user behind the MC gateway UE has no 3GPP transport access credentials (i.e., no UICC), whereas the MC system relies on private user identity associated with public user identities (i.e., IMPU/IMPI) needed for authentication purposes. Moreover, the one-to-one correspondence between MC service ID and IMPUs enables routing of signalling traffic between the MC service server and the MC service client.

NOTE:	Due to using Framed routing as described in clause 11.4, MC service clients behind the MC gateway UE have unique IP addresses, hence, implicit registration and de-registration approach described in clause 5.2.1a 3GPP TS 23.228 [9] is not applicable irrespective whether the MC service users behind the MC gateway UE and the MC service client hosted by the MC gateway UE both have common IMS subscription or not.

The use of the various mission critical user identities is determined by clause 8 of the present document.

# * * * * second change * * * *

### 11.3.2	When using separate IMS subscriptions

If the MC service clients are hosted by non-3GPP devices or the MC service clients are hosted by the MC gateway UE, an MC service user corresponding to an MC client uses a dedicated IMS/SIP subscription without any relation to the MC gateway UE subscription.

Figure 11.3.2-1 depicts the use of the IMS Credential (IMC) application in accordance with 3GPP TS 23.228 [9].

Figure 11.3.2-1: IMC per MC service user

The MC service user profiles are independent of the MC gateway MC service user profile.

# * * * * Third change * * * *

### 11.3.3	When sharing MC gateway UE's IMS subscription

For the case of non-3GPP devices which are unable to host an MC service client, the MC gateway UE acts as a proxy to transmit media towards the MC service users at the non-3GPP devices. As there are no MC service clients hosted by these non-3GPP devices, they rely on shared credentials at the MC gateway UE for MC services.

The interworking and signalling between the instantiation of MC service client at the MC gateway UE and the non-3GPP devices which cannot host a client is out of scope of 3GPP.

# * * * * End of changes * * * *