3GPP TSG-SA6 Meeting #53	S6-230805

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0067 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on optional use of Non-3GPP security message response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sepura Ltd, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCCI |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current text can read as if a direct end-to-end non-3GPP security message response in acknowledgement is required whenever a non-3GPP security message request has been issued. However, the acknowledgement only means the non-3GPP security message was successfully received at the IWF in clause 10.12.2.2, or at the MC service client in clause 10.12.2.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the use of an end to end non-3GPP security message response is asynchronous across the nodes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-operation of LMR KMMs between interworked systems & mis-alignment between Stage 2 & Stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-230622 |  |  |  |  |  |  |  |  |

* * * Begin Change * * * *

### 10.12.2	LMR key management messages

#### 10.12.2.1	General

This subclause defines end to end messaging to convey the non-3GPP, LMR security information opaquely (message contents are out of 3GPP's scope) across the MC system, between the IWF and the LMR aware MC service client. The end to end messasges are service independent, any MC service may support them.

#### 10.12.2.2	MC service client initiated

Figure 10.12.2.2-1 describes the case where an MC service client sends LMR security information to the IWF.

Pre-conditions:

1.	The MC service client is registered and the user is authenticated and authorized to use the MC service server.

Figure 10.12.2.2-1: Non-3GPP security messaging, MC service client to the IWF

1.	The MC service client sends a non-3GPP security message request to the MC service server. The contents of the message are opaque to the MC service and are out of scope of 3GPP.

2.	The MC service server forwards the contents of the non-3GPP security message request to the IWF.

3.	The IWF acknowledges with a non-3GPP security message response to the MC service server.

4.	The MC service server acknowledges with a non-3GPP security message response to the MC service client.

NOTE:	The non-3GPP security message response can be asynchronous across the MC service server.

#### 10.12.2.3	IWF initiated

Figure 10.12.2.3-1 describes the case where the IWF sends LMR security information to an MC service client.

Pre-conditions:

1.	The MC service client is registered and the user is authenticated and authorized to use the MC service server.

Figure 10.12.2.3-1: Non-3GPP security messaging, from the IWF to MC service client

1.	The IWF sends a non-3GPP security message request to the MC service server. The contents of the message are opaque to the MC service and are out of scope of 3GPP.

2.	The MC service server forwards the contents of non-3GPP security message request to the MC service client.

3.	The MC service client acknowledges with a non-3GPP security message response to the MC service server.

4. The MC service server acknowledges with a non-3GPP security message response to the IWF.

NOTE:	The non-3GPP security message response can be asynchronous across the MC service server.

* * * End Change * * * *