3GPP TSG-SA WG6 Meeting #53	S6-230806

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230513)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0358 | rev | 5 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Criteria to the Preconfigured User Regroup Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, Motorola Solutions, Ericsson, AT&T, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are instances when having the ability to include criteria in the user regroup request are needed, particularly in emergency situations when there is a need to location personel in a specific geographical area These requirements being addressed are in TS22.280: - [R-6.6.4.2-001] The MCX Service shall provide a means for combining a multiplicity of MCX Users into a new, temporary group (i.e., to perform a "User Regroup operation") - [R-6.6.4.2-002] The MCX Service shall provide a means for combining a multiplicity of MCX Users into a new, temporary group based on a parameter or a combination of parameters (e.g., particular geographic area, Participant type, initiation of urgent type communication such as MCX Emergency Alert or MCX Emergency Group Communication). - [R-6.6.4.2-002a] The MCX Service shall provide a means for automatically combining a multiplicity of MCX Users into a temporary MCX Service Group based on certain criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An additional IE service user criteria, to the Preconfigured regroup request for the MC service client-MC service server and the MC service server-MC service server messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MC servers for preconfigured user regroup would not support having selection criteria in determining regroup participants. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.2.1 10.15.2.2, 10.15.2.3, and 10.15.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S6-230080 Added new IE’s for Preconfigured regroup request messages in section 10.15.2.1, 10.15.2.2, and 10.15.2.3 to provide the capability to add criteria for regroup requests for user regroup and user regroup. Revision of S6-230314 Added additional clarification in the description in sections 10.15.2.1, 10.15.2.2, and 10.15.2.3 to indicate use of the MC user criteria is left to implementation. Revision S6-23512 and 0513 – Corrected cover page Revision S6-230806 – Added changes to correct procedure in 10.15.3.2.2, changes MC service group ID to optional, removed MC service group criteria |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.15.2.1	Preconfigured regroup request (MC service client – MC service server)

Table 10.15.2.1-1 describes the information flow preconfigured regroup request from the MC service client to the MC service server.

Table 10.15.2.1-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location, which is left to implementation. |
| NOTE: One and only one of these shall be present. |  |  |

* * * Second Change * * * *

#### 10.15.2.2	Preconfigured regroup request (MC service server – MC service client)

Table 10.15.2.2-1 describes the information flow preconfigured regroup request from the MC service server to the MC service client.

Table 10.15.2.2-1 Preconfigured regroup request information elements

| Information Element | Status | Description |  |
| --- | --- | --- | --- |
| MC service ID | M | The MC service ID of the target MC service group member |  |
| MC service group ID | O | MC service group ID of the regroup group |  |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |  |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |  |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |  |
| MC service user criteria | O (see NOTE) |  | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location, which is left to implementation. |
| NOTE: One and only one of these shall be present. |  |  |  |

* * * Third Change * * * *

#### 10.15.2.3	Preconfigured regroup request (MC service server – MC service server)

Table 10.15.2.3-1 describes the information flow preconfigured regroup request from the MC service server to the MC service server.

Table 10.15.2.3-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| MC service user criteria | O (see NOTE) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location, which is left to implementation. |
| NOTE: One and only one of these shall be present. |  |  |

* * * Fourth Change * * * *

##### 10.15.3.2.2	Preconfigured User Regroup formation in a single MC system

Figure 10.15.3.2.2-1 illustrates the procedure to initiate a user regroup procedure using a preconfigured MC service group. The procedure takes place prior to the establishment of a group call to the MC service regroup group.

Pre-conditions:

-	MC service clients 2 and 3 are registered with the MC service service.

-	The MC service group identity and group configuration for the preconfigured MC service group have been preconfigured in MC service clients 2 and 3, and MC service clients 2 and 3 have received the relevant security related information to allow them to communicate in the MC service regroup group.

-	MC service client 1 is authorized to initiate a preconfigured user regroup procedure.

-	MC service client 1 is aware of a suitable preconfigured group whose configuration has been preconfigured in the MC service UEs of the MC service users who will be regrouped.

Figure 10.15.3.2.2-1: User regroup procedure using preconfigured group in a single MC system

1.	The authorized user of MC service client 1 initiates the user regroup procedure, specifying the list of MC service users to be regrouped (MC service clients 2 and 3), the MC service group ID of the regroup group, and the MC service group ID of the group from which configuration information for the regroup group is to be taken. Alternatively, the MC service client 1 can provide the criteria which allows the MC service server to determine the list of MC service users to be regrouped.

2.	MC service client 1 sends the preconfigured regroup request to the MC service server.

3.	The MC service server checks that MC service client 1 is authorized to initiate a preconfigured user regroup procedure.

NOTE 1:	MC service clients can be involved in multiple user and group regroups simultaneously.

4.	The MC service server sends the preconfigured regroup requests to MC service clients 2 and 3 in steps 4a and 4b respectively.

NOTE 2:	When using multicast, the MC service server can periodically rebroadcast the preconfigured regroup request.

5.	MC service clients 2 and 3 notify their users of the regrouping in steps 5a and 5b respectively.

6.	MC service clients 2 and 3 may send the preconfigured regroup response to the MC service server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request.

7.	The MC service server affiliates the regrouped MC service clients to the regroup group.

8.	The MC service server sends a preconfigured regroup response to MC service client 1.

NOTE 3:	After the user regrouping procedure, the regrouping remains in effect until explicitly cancelled by the procedure in 10.15.3.2.3.

* * * End of Changes * * * *