3GPP TSG-SA WG6 Meeting #53	S6-230831

Athens, Greece 27th Feb. – 3rd Mar. 2023	(revision of S6-230567, S6-230462, S6-230324, S6-230038)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0155 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New AC-EEC procedure to invoke UE ID request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new procedure is needed to support the retrieval of the UE ID for AC’s (and hence EAS’s) dealing with IPv4 NATed IP address issue. That is when EAS’s direct invocation of UE Identifier API procedure (i.e. Eees_UEIdentifier API) due to NATed IP address fails to identify the UE then the EAS may use the AC to ask the EEC for the UE ID. Therefore, this procedure is provided to enable the AC interact with the EEC to obtain the UE ID and behind the scene pass on the UE ID to the EAS. In turn the EEC would rely on its interaction with the EES to obtain the UE ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clause 8.14.1 & 8.14.2.1 to list the new UE ID retrieval procedure. Also, adding the details of the new procedure (Edge UE ID request) in the new clause 8.14.2.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retrival of the UE ID corresponding to the UE’s NATed IP address is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.1, 8.14.2.1, 8.14.2.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: It adds information flow clauses related to the the new procedure (i.e. 8.14.3.x & 8.14.3.y). It also adds an EN to clause 8.14.1 & 8.14.2.1 & 8.14.x. while making some editorial changes. Rev 2: removed “Registration ID” IE from the Edge UE ID request throughout the CR. Changed EN re Edge UE ID and added a EN re preserving privacy of the UE Rev 3: - As per S6-230564 recommendation (Discussion on “UE ID” vs “Edge UE ID”), it removes the following EN “Editor's note: The definition of EDGE UE ID is FFS” from clause 8.14.2.1 and accordingly removes the word “Edge” from the term “Edge UE ID” throughout the CR in order to keep 23.558 consistent in using the already defined term “UE ID”. - Also taking care of a typo in an EN in clause 8.14.3.x. Rev 4: - Adds the IE “UE ID Type” to the UE ID response in clause 8.14.3.y to make it consistent with the response EES provide EEC in the response to the UE Identifire API. |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

## 8.14	EDGE-5 APIs

### 8.14.1	General

EEC exposes EDGE-5 APIs corresponding to EEC's capabilities, for the AC to request EEC's services for edge enablement. Using these APIs, ACs request the EEC for EEL services. EDGE-5 APIs include one-time request/response operations for EAS discovery, retrieval of UE ID and ACR operations. Additionally, the AC can request for an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required.

NOTE 1:	Details on how the AC and EEC communicate with each other is out of scope.

NOTE 2:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

Editor's note:	[SA3] How to ensure of user's authorization/consent as well as AC’s authorization in invoking functions exposed by EEC (to AC) which in turn relies on functions exposed by the network (e.g. Location) via EES/NEF is SA3’s responsibility.

### 8.14.2	Procedures

#### 8.14.2.1	General

Following procedures are specified for EDGE-5:

-	Registration;

-	EAS discovery;

-	ACR request;

-	EEC services subscription; and

-	UE ID request.

# * * * Change 2 * * * *

#### 8.14.2.x	UE ID request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 8.14.2.x-1: UE ID request procedure

1.	The AC sends an UE ID request to the EEC. The request includes AC's security credentials and the EAS ID list for which the AC needs to know the associated Edge UE ID or AF-specific UE ID(s).

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC process the request from the AC. The EEC uses the UE Identifier API procedure (see clause 8.6.5) to query the EES.

NOTE 1:	This procedure may be used to support the retrieval of the UE ID for AC’s (and EAS’s) dealing with IPv4 NATed IP address issue (i.e. EAS’s direct invocation of UE Identifier API procedure due to NATed IP address fails to identify the UE). Under such a scenario then it is understood that, the AC through application signalling which is outside the scope of this document, would pass on the UE ID (received in step 3) to the EAS so that it can be used to invoke capability APIs over EDGE-3 or EDGE-7.

Editor's note:	The use of this procedure for other use cases such as preserving the privacy of the UE is FFS.

# * * * Change 3 * * * *

#### 8.14.3.x	UE ID request

Table 8.14.3.x-1 describes information elements of the UE ID request sent by the AC to the EEC.

Table 8.14.3.x-1: UE ID request

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID list | M | List of EAS ID(s) the AC requires to know the associated AF-specific UEID(s) for. |
| Security credentials | M | Security credentials of the AC sending the UE ID request. |

Editor's note:	[SA3] How to ensure EAS ID list IE only contains the EAS IDs that are indeed associated with the AC requesting the UE ID(s), and hence preventing UE ID (i.e. AF-specific UE ID) harvesting by a rogue AC  is FFS.

#### 8.14.3.y	UE ID response

Table 8.14.3.y-1 describes information elements of the UE ID response sent by the EEC to the AC.

Table 8.14.3.y-1: UE ID response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the UE ID request was successful. |
| > UE ID list | M | List of all the Edge UE IDs or AF-specific UE IDs the AC asked for in the request and is authorized to use as per EAS ID(s). |
| >> UE ID | M | Edge UE ID or AF-specific UE ID |
| >> UE ID type | M | Indication whether the UE ID is CN assigned AF-specific UE ID or Edge UE ID. |
| >> EAS ID | M | Associated EAS ID |
| Failure response | O | Indicates that the UE ID request failed. |
| > Cause | M | Provides the cause for UE ID request failure. |

# * * * End of changes * * * *