3GPP TSG-SA WG6 Meeting #53	S6-230857

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230674,

S6-230340, S6-230228)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0161 | rev | 4 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL server provisioning for Location Management Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specfieid in clause 9.3.13.1, the VAL service areas are configured by the VAL server to the location management server. Further, there is no need to specify API for each configuration (like VAL service area Id list). So, it is better design to have single provisioning API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a procedures and information flows for VAL server to provision requried configuration to the SEAL LMS. Removed / Made void – clauses related to VAL service area ID request/response/API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL LMS will not be aware of the default triggers and also VAL service areas. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.21, 9.3.2.22, 9.3.2.xx (NEW), 9.3.2.yy (NEW), 9.3.zz(NEW), 9.4.1, 9.4.8, 9.4.a (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.21	Void

#### 9.3.2.22	Void

* * * Next Change * * * *

#### 9.3.2.xx	VAL server provisioning request

Table 9.3.2.xx-1 describes the information flow from the VAL server to the location management server for providing provisioning configuration.

Table 9.3.2.xx-1: VAL server provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Endpoint information | O | Information of the endpoint of the VAL server. |
| Security credentials | M | Security cradentials to authorize and authenticate VAL server |
| List of VAL service specific information | M | Provides the list of VAL service specific information |
| > VAL service ID | M | Identify of the VAL service for which the configuration information is provided. |
| > VAL Service Area ID | M | VAL Service area Identity |
| >> List of VAL Service Areas | O | List of VAL Service Areas identified distinctly through geographical area (e.g., co-ordinates) |

#### 9.3.2.yy	VAL server provisioning response

Table 9.3.2.yy-1 describes the information flow from the location management server to the VAL server as a response for providing provisioning configuration.

Table 9.3.2.yy-1: VAL server provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * Next Change * * * *

#### 9.3.2.A	VAL server obtain provisioning parameters request

Table 9.3.2.A-1 describes the information flow from a VAL server to a location management server for obtain VAL service area provisioning parameters.

Table 9.3.2.A-1: VAL server obtain provisioning parameter request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| VAL service ID | M | Identify of the VAL service for which the configuration information is required. |
| Security credentials | M | Security cradentials to authorize and authenticate VAL server |

* * * Next Change * * * *

#### 9.3.2.B	VAL server obtain provisioning parameters response

Table 9.3.2. -1 describes the information flow from a location management server to the VAL server for provide provisioning paramters for the VAL service.

Table 9.3.2.B-1: VAL server obtain provisioning parameter response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| > VAL Service Area ID (NOTE) | M | VAL Service area Identity |
| >> List of VAL Service Areas (NOTE) | O | List of VAL Service Areas identified distinctly through geographical area (e.g., co-ordinates) |
| NOTE: This IE is added only for successful case. |  |  |

* * * Next Change * * * *

#### 9.3.2.C	VAL server provisioning parameters delete request

Table 9.3.2.C-1 describes the information flow from a VAL server to a location management for delete provisioning parameters.

Table 9.3.2.C-1: VAL service provisioning parameters delete request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| Security credentials | M | Security cradentials to authorize and authenticate VAL server |
| List of VAL service IDs | M | List of one or more VAL service area identifiers that shall be deleted. |

* * * Next Change * * * *

#### 9.3.2.D	VAL server provisioning parameters delete response

Table 9.3.2.D-1 describes the information flow from a location management server to the VAL server to indicate results of the delete VAL service provisioning parameters request.

Table 9.3.2.D-1: VAL service provisioning parameters delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |

* * * Next Change * * * *

#### 9.3.2.E	VAL service area subscription request

Table 9.3.2.E-1 describes the information flow from a VAL server to a location management to subscribe VAL service area identifier update events at the LM server.

Table 9.3.2.E-1: VAL service area subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| List of the subscribed events | M | Indicates the list of the requested events. |
| > Event type | M | Indicates the subscribed event (e.g., creation/update/deletion of the VAL service area ID) |
| > VAL service area IDs | O | Indicates the list of the VAL service area identifiers. This information element shall be provided when the requested event type is update or delete VAL service area ID. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL service area update events. |
| Subscription duration | O | It indicates the subscription duration. If absent, 1 hour is used as default setting. |

Editor's note:	The subscribed event types within the VAL service area subscription request are FFS.

* * * Next Change * * * *

#### 9.3.2.F	VAL service area subscription response

Table 9.3.2.F-1 describes the information flow from a location management server to a VAL server for VAL service area subscription response.

Table 9.3.2.F-1: VAL service area subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |

* * * Next Change * * * *

#### 9.3.2.G	VAL service area notification

Table 9.3.2.G-1 describes the information flow for VAL service area notification from a location management server to a VAL server.

Table 9.3.2.G-1: VAL service area notification

| Information element | Status | Description |
| --- | --- | --- |
| List of events | M | Indicates the list of the occurred events. |
| > Event type | M | Indicates the subscribed event (e.g., creation/update/deletion of the VAL service area ID). |
| > VAL service area ID | M | The VAL service area identifier. |
| >> List of VAL service area information | M | List of one or more VAL service area identifiers. |

Editor's note:	The subscribed event types within the VAL service area subscription request are FFS.

* * * Next Change * * * *

### 9.3.zz	VAL server provisioning for location management service

#### 9.3.zz.1	General

The high level procedure for VAL server to provision required information to SEAL location management server is described in the following subclause.

Editor’s note: The subscribe-notify model for VAL server to receive provisioining parameter is FFS.

#### 9.3.zz.2	Providing provisioning parameters

The procedure for VAL server to provision required information to SEAL location management server is illustrated in figure 9.3.zz.2-1.

Figure 9.3.zz.2-1: VAL Server provisioning to SEAL Location Management Server

1.	The VAL server sends a request message to location management server to provision required information. The request message includes identity of the VAL server, endpoint information of the VAL server, security credentials of the VAL server, and service provider specific information like list of VAL service areas.

2.	Upon receiving the request, the location management server authorizes the request based on the security credentials provided in the request and considering the service level agreement between VAL service provider and SEAL service provider. If VAL server is authorized to use the SEAL service, then the location management server stores the details about the VAL server including list of VAL service areas. The location management server sends the response message to the VAL server.

NOTE: The same procedure can be used to update the provisioning parameters.

Editor’s note:	Whether the VAL server registers with location Management Server is FFS.

#### 9.3.zz.3	Obtaining provisioning parameters

Figure 9.3.zz.3-1 illustrates the high level procedure of the VAL service area retrieval.

Figure 9.3.zz.3-1: Obtain VAL service provisioning parameters procedure

1.	The VAL server sends the VAL server obtain provisioning parameters request to the location management server to retrieve the provisioning parameters. In the request message, the VAL server includes the information as specified in table 9.3.2.A-1.

2.	The location management server shall check if the VAL server is authorized to initiate VAL server obtain provisioning parameters request.

3.	If the VAL server is authorized to obtain the VAL service provisioning parameters, the location management server sends the VAL service area obtain response with the information as specified in table 9.3.2.B-1.

#### 9.3.zz.4	Deleting provisioning parameters

Figure 9.3.zz.4-1 illustrates the high level procedure of the VAL service area delete.

Figure 9.3.zz.4-1: Delete VAL service provisioning parameters procedure

1.	The VAL server sends the VAL service area delete request to the location management server to delete the VAL service area identifier(s) configured by the VAL server. In the request message, the VAL server includes the information as specified in table 9.3.2.C-1.

2.	The location management server shall check if the VAL server is authorized to initiate VAL service area delete request.

3.	The location management server checks whether the VAL service area IDs exist and then deletes the VAL service area identifier(s).

NOTE:	An deletion of the VAL service area ID(s) does not affect the existing subscriptions in the SEAL layer.

4.	The location management server sends the VAL service area delete response with the information as specified in table 9.3.2.D-1.

#### 9.3.zz.5	VAL service area identifier subscribe procedure

Figure 9.3.13.5-1 illustrates the high level procedure of the subscribe VAL service area update events.

Figure 9.3.zz.5-1: Subscribe VAL service area procedure

1.	The VAL server sends the VAL service area subscription request to the location management server to subscribe for the VAL service area identifier(s) update events. In the request message, the VAL server includes the information as specified in table 9.3.2.27-1.

2.	The location management server shall check if the VAL server is authorized to susbcribe for the VAL service area identifier(s) update event(s).

3.	Tf the VAL server is authorized to subscribe for the VAL service area identifier(s) update events, the location management server creates the subscription based on the provided parameters in the request.

4.	The location management server sends the VAL service area subscription response with the information as specified in table 9.3.2.28-1.

#### 9.3.zz.6	VAL service area identifier notify procedure

Figure 9.3.13.6-1 illustrates the high level procedure for notification of a VAL server about the VAL service area identifier update events at the location management server.

Pre-conditions:

-	the VAL server has an active VAL service area subscription at the location management server; and

-	the subscribed event has occurred.

Figure 9.3.zz.6-1: Notify VAL service area procedure

1.	The location management server identifies the list of the occurred events for which the VAL server subscribed.

2.	The location management server sends a VAL service area notification to the VAL server. In the notification message, the location management server includes the information as specified in table 9.3.2.G-1.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_LmsVALServerProvisioning | Provide_Configuration | VAL server | Request /Response |
|  | Obtain_VAL_Server_provisioning_parameter | VAL server | Request /Response |
|  | Update_VAL_Server_provisioning_parameter | VAL server | Request /Response |
|  | Delete_VAL_Server_provisioning_parameter | VAL server | Request /Response |
|  | Subscribe_VAL_Service_Area_Change_Event | VAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | VAL server |  |

* * * Next Change * * * *

### 9.4.8	Void

* * * Next Change * * * *

### 9.4.a	SS_LmsVALServerProvisioning API

#### 9.4.a.1	General

API description: This API enables the VAL server to provision configuration for the VAL service to the SEAL LM-S.

#### 9.4.a.2	Provide_Configuration operation

API operation name: Provide_Configuration

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.xx

Outputs: See subclause 9.3.2.yy

See subclause 9.3.zz.2 for the details of usage of this API operation.

#### 9.4.a.3	Obtain_VAL_Server_provisioning_parameter

API operation name: Obtain_VAL_Server_provisioning_parameter

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.A

Outputs: See subclause 9.3.2.B

See subclause 9.3.zz.3 for the details of usage of this API operation.

#### 9.4.a.4	Update_VAL_Server_provisioning_parameter

API operation name: Update_VAL_Server_provisioning_parameter

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.xx

Outputs: See subclause 9.3.2.yy

See subclause 9.3.zz.2 for the details of usage of this API operation.

#### 9.4.a.5	Delete_VAL_Server_provisioning_parameter

API operation name: Delete_VAL_Server_provisioning_parameter

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.C

Outputs: See subclause 9.3.2.D

See subclause 9.3.zz.4 for the details of usage of this API operation.

#### 9.4.a.6	Subscribe_VAL_Service_Area_Change_Event

API operation name: Subscribe_VAL_Service_Area_Change_Event

Description: Subscription to the VAL service area identifiers change events.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.E

Outputs: See subclause 9.3.2.F

See subclause 9.3.zz.5 for the details of usage of this API operation.

#### 9.4.a.7	Notify_VAL_Service_Area_Change_Event

API operation name: Notify_VAL_Service_Area_Change_Event

Description: Notifying the VAL server about the VAL service area identifier change events.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.G

Outputs: None.

See subclause 9.3.zz.6 for the details of usage of this API operation.

* * * End of Change * * * *