3GPP TSG-SA WG6 Meeting #53	S6-230859

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230751)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0183 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS service area in pre-conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The pre-condition in cl. 14.3.4A.6.2 is not fully correct as the procedures covers normal multicast MBS case which is not restricted by the service area. - VAL UE 2 to n are within the MBS service area where the MBS session is configured. Multicast MBS session: An MBS session to deliver the multicast communication service. A multicast MBS session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a geographical area where to distribute it. Broadcast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e. all UEs in the broadcast coverage area are authorized to receive the data). “A Local MBS service is an MBS service provided in one MBS service area.” – clause 6.2 “NOTE: Broadcast communication service is the service provided simultaneously to all UEs in a geographical area, therefore for broadcast it is naturally a local MBS service.” – Clause 6.2.2 In clause 14.3.4A.6.2 step 1, the MBS session announcement is also being restricted “within the service area” which is also not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) remove the last pre-condition in clause 14.3.4A.6.2 2) update the clause 14.3.4A.1 to capture the relationship between MBS service area and MBS session. 3) update the clause 14.3.4A.6.2 step 1 description on the session announcement to remove the service area restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusions will be caused to readers on how the MC service server use the normal multicast MBS session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.4A.1, 14.3.4A.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** 1st change **********************/

#### 14.3.4A.1	General

This subclause defines information flows and procedures for 5G MBS usage that applies to VAL services. 5G MBS session can be used by any VAL service for any VAL service group.

The main purpose of using 5G Multicast-Broadcast Service (MBS) by verticals is to provide efficient downlink delivery of user traffic in VAL service group communications or in a certain area.

Multicast and broadcast communication services in 5G for vertical applicationsrely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the VAL server to multiple VAL users is supported either as point-to-point or point-to-multipoint over the radio. The MBS session which is consist of one or multiple QoS flows for different service requirements is either broadcast or multicast type. For the broadcast MBS session or local MBS session, the MBS service area is configured with the MBS session.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

Within this arrangement, the VAL server decides whether to create broadcast or multicast MBS sessions to be associated with certain VAL service groups or area. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the VAL UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED state.

The following capabilities (non-exhaustive list) provided by MBS could be used by NRM server:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Dynamic PCC control for MBS session.

The first phase to utilize MBS sessions for VAL media transmission is to reserve the network resource by creating a MBS sessions. The MBS session creation is initiated by the VAL server towards the NRM server, and the NRM server further interacts with the 5GS to complete the whole process. The UE’s capabilities and service related information e.g., UE’s MBS capabilities, location, MBS listening status report sent by group members are considered when deciding to create or use MBS sessions. During the interaction with NRM server, the necessary information related to the requested session is determined, e.g., MBS session mode (either a broadcast or a multicast session) and the required service profile. This interaction between the NRM server and the 5GS depends on the configuration option under consideration, i.e., whether the NRM server is in trusted domain (limited operations), and whether the session creation is done with or without a dynamic PCC rule.

NOTE 1:	It is implementation specific whether the VAL server decides to use multicast or broadcast MBS sessions.

NOTE 2:	It is implementation specific whether the VAL decides to create (one or multiple) MBS sessions for VAL media for VAL group communications associated to a certain VAL group or create (one or multiple) dynamic MBS sessions once the need has emerged, e.g., dynamic MBS sessions to be associated for an ad hoc group.

NOTE 3:	It is implementation specific whether an MBS session is associated to one or multiple VAL service groups, and whether it is re-assigned to other VAL service groups.

NOTE 4:	How the NRM server uses the UE’s capabilities and service related information in order to create and use the MBS session is implementation specific.

The information elements describing the MBS session under consideration is then sent to the NRM clients via MBS session announcement, where the latter need to react according to the announced session mode.

If eMBMS and 5G MBS co-exist for VAL services, the NRM service server may decide to trigger the establishment of an eMBMS bearer to deliver the VAL media associated to the VAL service group communications, if the target VAL service group(s) consists of members with MBMS capable RAT. As a result, the NRM server subsequently needs to send an eMBMS bearer announcement towards the clients camping on LTE.

NOTE 5:	It is implementation specific whether the NRM server triggers an eMBMS bearer or a unicast bearer to serve VAL clients camping on LTE.

# /******************** 2nd change **********************/

##### 14.3.4A.6.2	Procedure

The procedure in figure 14.3.4A.6.2-1 describes how media related to a specific group communication can be distributed over a configured MBS session which consist of multiple QoS flows, i.e. addressing different service requirements.

Pre-conditions:

-	VAL UE 1 to n are attached to the 5GS, registered and belong to the same VAL service group X.

-	The VAL server has decided to use an MBS session for VAL service group communications associated to VAL service group X.

Figure 14.3.4A.6.2-1: VAL service group media transmission over MBS sessions

1.	The VAL server creates a multicast or a broadcast MBS session targeting group communications associated to VAL service group X via the NRM server, as being specified in clause 14.3.4A.2. Therefore, the VAL server can provide default service requirements to be addressed by the MBS session.

The MBS session is announced by NRM server and received by NRM client 2 to n. The NRM server has identified that VAL UE 2 to n can receive media over the MBS sessions, e.g. based on a notification from NRM clients indicating the successful join of the multicast MBS session or a monitoring report of the broadcast MBS session (similar to the listening status report used for MBMS).

2.	A new VAL group communication is established for the VAL service group X consisting of a specific required service requirements, e.g. a VAL service emergency group communication. The group communication setup can be done over unicast.

2a. For broadcast MBS sessions, the session is established as described in 3GPP TS 23.247 [39].

2b. For multicast MBS session, the session is established upon the acceptance of the first UE session join request initiated from the VAL UE towards the 5GS, as described in 3GPP TS 23.247 [39]. The multicast session can then have either an active or an inactive state.

3. 	The VAL server may initiate the multicast MBS activation towards the NRM as described in clause 14.3.4A.5 in order to activate the multicast MBS session in case the session has an inactive state.

4.	Considering that the established group communication requires a specific QoS, e.g. an VALemergency group communication which requires higher priority (i.e. better ARP), the VAL server initiates an MBS session update to the NRM to provide the new required service requirements, if not done during the MBS session creation in step 1, as described in clause 14.3.4A.2. The MBS session should then be updated and an additional QoS flow may be configured.

5.	The NRM server sends a MapGroupToSessionStream to NRM clients 2 to n over the configured MBS session providing the required stream information to receive the media related to the specific established VAL group communication within the MBS session.

6.	NRM clients process the MapGroupToSessionStream information to receive the related media over the specific MBS session stream.

7.	VAL client 1 sends media to the VAL server over unicast to be distributed for the established group communication.

8.	The VAL server distributes the media to VAL clients 2 to n over the indicated stream within the established MBS session.

9.	The VAL server may initiate the multicast MBS session deactivation towards the NRM as described in clause 14.3.4A.5, in order to deactivate the multicast MBS session.

10.	The NRM server may further trigger the UE to leave multicast MBS session.