3GPP TSG-SA WG6 Meeting #53	S6-230863

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230755)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0187 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applications on UE directly consuming SEAL services via SEAL-S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3, SNAAPP |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following stage 1 requirement in TS 22.261 specifies that: The 5G system shall be able to provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE. The SEAL architecture should be able to offer service APIs to the application on the UE as well apart from the application servers deployed on the network. The service APIs are offered over SEAL-S reference point. Thus, the VAL client should be able to access the SEAL service APIs directly over SEAL-S. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The general architecture of SEAL and the corresponding description of functional entities, reference points are modified to support this interaction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The service APIs offered by SEAL services over SEAL-S cannot be enabled to be directly consumed by the VAL client. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.4.2.2, 6.4.2.5, 6.5.2.7, 8.2.1, 8.2.2, 8.2.3, 15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.4.2.5	SEAL server

The SEAL server provides the server side functionalities corresponding to the specific SEAL service. The SEAL server supports interactions with the VAL layer entities. The SEAL server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The SEAL server also supports interactions with the corresponding SEAL server in distributed SEAL deployments.

NOTE:	It is up to each SEAL server to support the appropriate signalling plane entities.

* * * Next Change * * * *

#### 6.5.2.7	SEAL-S

The interactions between the VAL layer entities and the SEAL server are generically referred to as SEAL-S reference point. The specific SEAL service reference point corresponding to SEAL-S is specified in the specific SEAL service functional model.

* * * Next Change * * * *

## 15.2	Functional model representation

Figure 15.2-1 illustrates the service-based interface representation of the functional model for SEAL services.

Figure 15.2-1: SEAL generic functional model representation using service-based interfaces

The SEAL function(s) exhibit the service-based interfaces which are used for providing and consuming SEAL services. The service APIs are specified for each SEAL function enabled over the service-based interface. The service-based interfaces of specific SEAL services are specified in this document. All the interactions with SEAL are governed based on the reference point interactions of the functional models specified in subclause 6. VAL server function represents the functionalities of the VAL server. VAL client function in the VAL UE represents the functionalities of the VAL client. The VAL client function is a consumer of SEAL services using the service-based interfaces.

NOTE 1:	If the VAL client function consumes SEAL services directly from SEAL function(s), the VAL client does not communicate with the SEAL client for consuming such SEAL services.

NOTE 2:	The service-based interface Sval for the VAL server function is out of scope of the present document.

The service APIs offered by the SEAL function(s) are published and discovered on the CAPIF core function as specified in 3GPP TS 23.222 [8]. The secure access to the SEAL functions' service APIs by VAL client function and/or VAL server function complies with the authentication and authorization procedures of CAPIF as specified in 3GPP TS 23.222 [8].

Editor's note:	Refinement of the SEAL services in service-based interface representation is FFS.