3GPP TSG-SA WG6 Meeting #53	S6-230911

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230526)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0107 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification when Store and forward applies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The pre-condition 2 for Store and forward services to apply is that the recipient UE is unavailable, but the text in the note states that reachability for message delivery is the deciding factor if store and forward services are applicable where availability and reachability are the same thing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Availability and reachability are different concepts and both availability and reachability are necessary conditions for the message to be delivered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how Store and forward services need to be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.3.6	MSGin5G Store and Forward

Figure 8.3.6-1 shows the procedure for providing store and forward services for MSGin5G message requests.

This procedure applies when an MSGin5G message is received at the MSGin5G Server and the message cannot be delivered to the recipient UE; otherwise, the procedure detailed in clause 8.3.2 applies.

Pre-conditions:

1.	The MSGin5G Client or Application Server has registered to the MSGin5G Server.

2.	The MSGin5G Server has determined that the recipient UE is not available.

NOTE:	In addition to UE registration status, the MSGin5G Server can use e.g, UE reachability status monitoring specified in clause 8.9.2 or the recipient's Communication Availability information specified in clause 8.2 to determine whether the recipient is available and reachable for message delivery.

Figure 8.3.6-1: Store and forward procedure

1.	Inbound MSGin5G message handling, see steps 1-3 in clause 8.3.2. The value of the Store and forward flag IE (see Table 8.3.2-1) in the MSGin5G message indicates that store and forward services are requested by the sender.

2.	The MSGin5G Server checks the registration information of the recipient UE. If the Store and forward option IE (see Table 8.2.1-3) indicates that the recipient UE opts out of store and forward services, the message is discarded and the procedure ends. If the Store and forward flag IE (see Table 8.3.2-1) in the received message indicates that store and forward services are not requested by the sender, the message is discarded and the procedure ends.

If store and forward processing is required, the MSGin5G Server uses the Application specific store and forward information IE (see Table 8.3.2-2) to determine storage and forwarding.

3.	Before the Message expiration time is expired, the MSGin5G Server may trigger the Recipient UE based on the MSGin5G device triggering procedure in clause 8.9.3.

4.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the delivery had been deferred.

5.	The recipient UE becomes available.

6.	When the recipient UE becomes available, the MSGin5G Server attempts delivery of the request using the procedure specified in clause 8.3.3.

If the UE does not become available prior to the expiration time, the MSGin5G Server attempts delivery of the request at the message expiration time and the stored message is discarded afterwards.

7.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the message was discarded.