3GPP TSG-SA WG6 Meeting #53	S6-230930

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230715)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0339 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Notifying authorized user about adhoc group participants list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently only an originator of the ad hoc group call is notified about all the MCPTT users who acknowledged the ad hoc group call request and joined the call, and when MCPTT users joins or leaves the MCPTT ad hoc group call. If the authorized user (e.g., Dispatcher) is part of the ad hoc group call then authorized user also should be notified about the participant’s information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex A.3: configuration is added to the user profile to authorize an user to receive the participants information of ad hoc group call. 10.19.3.1.3: Procedure is updated to include the notification of participant information to authorized user based on the configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An authorized user (e.g., Dispatcher) will not know who all are participated in the ad hoc group call. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.10, 10.19.3.1.3, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Corrected the Authrozied spelling - 10.19.2.10 – The information element is updated to include additional condition to send the notification of participant list - when user join or leave the MCPTT ad hoc group call. Other corrections to the information element. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.19.2.10	Ad hoc group call notify (MCPTT server – MCPTT client)

Table 10.19.2.10-1 describes the information flow ad hoc group call notify from MCPTT server to MCPTT client.

Table 10.19.2.10-1: Ad hoc group call notify

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of ad hoc group call participant |
| Functional alias | O | The associated functional alias of the MCPTT user of the ad hoc group call participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID associated with the ad hoc group call |
| MCPTT ID list | O | The list of the invited MCPTT users who acknowledge the ad hoc group call request, when MCPTT users join or leave the MCPTT ad hoc group call. |

* * * Next Change * * * *

##### 10.19.3.1.3	Ad hoc group call setup with MCPTT server determining the participants lists

Figure 10.19.3.1.3-1 below illustrates the ad hoc group call setup procedure initiated by the MCPTT user and MCPTT client 1 wherein the list of participants is determined by the MCPTT server based on the citeria received from the MCPTT client.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initate ad hoc group call.

2.	The MCPTT user at MCPTT client 1 wants to invite MCPTT users who are satisying certain criteria for the ad hoc group call.

Figure 10.19.3.1.3-1: Ad hoc group call participants determined by MCPTT server

1.	User at MCPTT client 1 would like to initiate an ad hoc group call in-order to invite the participants satisfying specific criteria. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing the details of the criteria to be applied by the MCPTT server for determining the participants list. If end-to-end encryption is supported, the Encryption supported information element shall be set to true and pre-configured MCPTT group whose configuration is to be applied is included. An SDP offer containing the MCPTT client media parameters is included. If there is a floor request to transmit, then the ad hoc group call request contains an indication of an implicit floor request. If the MCPTT user of MCPTT client 1 has selected a functional alias, then the ad hoc group call request contains that functional alias. If the ad hoc group call request contains an implicit floor request it may also include location information.

If the MCPTT user at MCPTT client 1 initiates an MCPTT emergency ad hoc group call or the MCPTT emergency state is already set for the MCPTT client 1 (due to a previously triggered MCPTT emergency alert):

i.	the MCPTT ad hoc group call request shall contain an emergency indicator;

ii.	if the MCPTT emergency state is not set already, MCPTT client 1 sets its MCPTT emergency state. The MCPTT emergency state of MCPTT client 1 is retained until explicitly cancelled by the user of MCPTT client 1.

2.	The MCPTT server accepts the ad hoc group call request if the ad hoc group call is supported and authorized. Otherwise reject the ad hoc group call request and do not continue with the rest of the steps.

If functional alias is present, the MCPTT server checks whether the provided functional alias is allowed to be used and has been activated for the user.

If location information was included in the ad hoc group call request, the MCPTT server checks the privacy policy of the MCPTT user to decide if the location information of MCPTT client 1 can be provided to other users on the call (refer to Annex A.3 "Authorisation to provide location information to other MCPTT users on a call when talking").

If an emergency indicator is present in the received MCPTT ad hoc group call request, the MCPTT ad hoc group is considered to be in the in-progress emergency state until this ad hoc group call is terminated; and

If an imminent peril indicator is present in the received MCPTT ad hoc group call request, the MCPTT ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group call is terminated.

3.	The MCPTT server shall send the ad hoc group call request return message to MCPTT client 1 containing the below:

i.	The MCPTT ad hoc group ID generated by the MCPTT server; and

ii.	Result of whether the ad hoc group call is authorized or not

If the ad hoc group call request is not authorized, MCPTT server and client 1 shall not proceed with the rest of the steps.

4.	The MCPTT server determines the list of participants to be invited for the ad hoc group call based on the information present in the information element Criteria for determining the participants. This information element could carry either criteria or indicator identifying pre-defined criteria or a combination of both.

NOTE 1:	The content of the Criteria information element, the details of the pre-defined criteria, and the way how their MCPTT server determines the list of participants are left to implementation.

5.	The MCPTT server sends the ad hoc group call requests towards the MCPTT clients 2 and 3. While sending the ad hoc group call requests, the MCPTT server shall remove the information elements that are not required to be conveyed to the target MCPTT clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. The MCPTT server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

6.	The receiving MCPTT clients are notified about the incoming ad hoc group call.

7.	The receiving MCPTT clients accept the ad hoc group call requests and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server.

8.	The MCPTT server sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

9.	The MCPTT server may notify the initiating MCPTT user of all MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the MCPTT ad hoc group call. The authorized users (not shown in figure) who are configured to receive the participants information of ad hoc group call, the MCPTT server notify about all MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call, and when MCPTT users join or leave the MCPTT ad hoc group call.

NOTE 2:	The authorized user can learn who is currently affiliated to the current adhoc group call.

Editor's note: How an authorized user determine the currently affiliated status of adhoc group is FFS.

10.	MCPTT client 1, MCPTT client 2 and MCPTT client 3 establish media plane and floor control resources.

* * * Next Change * * * *

# A.3	MCPTT user profile configuration data

The general aspects of MC service user profile configuration data are specified in 3GPP TS 23.280 [16]. The MCPTT user profile configuration data is stored in the MCPTT user database. The MCPTT server obtains the MCPTT user profile configuration data from the MCPTT user database (MCPTT-2).

Tables A.3-1 and A.3-2 contain the MCPTT user profile configuration required to support the use of on-network MCPTT service. Tables A.3-1 and A.3-3 contain the MCPTT user profile configuration required to support the use of off-network MCPTT service. Data in table A.3-1 and A.3-3 can be configured offline using the CSC-11 reference point.

Table A.3-1: MCPTT user profile data (on and off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server | MCPTT user database |
| --- | --- | --- | --- | --- | --- |
| Subclause 8.1.2 of 3GPP TS 23.280 [16] | MCPTT user identity (MCPTT ID) | Y | Y | Y | Y |
| 3GPP TS 33.180 [19] | KMSUri for security domain of MCPTT ID (see NOTE 4) | Y | Y | Y | Y |
| Subclause 5.2.4 of 3GPP TS 23.280 [16] | Pre-selected MCPTT user profile indication (see NOTE 3) | Y | Y | Y | Y |
| Subclause 5.2.4 of 3GPP TS 23.280 [16] | MCPTT user profile index | Y | Y | Y | Y |
| Subclause 5.2.4 of 3GPP TS 23.280 [16] | MCPTT user profile name | Y | Y | Y | Y |
| [R-5.19-007], [R-6.13.4-002] of 3GPP TS 22.280 [17] | User profile status (enabled/disabled) |  | Y | Y | Y |
| [R-5.8-001], [R-6.9-003] of 3GPP TS 22.280 [17] | Authorised to create and delete aliases of an MCPTT User and its associated user profiles. |  |  | Y | Y |
| [R-5.8-002], [R-6.9-003] of 3GPP TS 22.280 [17] | Alphanumeric aliases of user | Y | Y | Y | Y |
| [R-5.10-001] of 3GPP TS 22.280 [17] | Participant type of the user | Y | Y | Y | Y |
| [R-5.3-002], [R-5.10-001] of 3GPP TS 22.280 [17] | User's Mission Critical Organization (i.e. which organization a user belongs to) | Y | Y | Y | Y |
| [R-5.4.2-003] of 3GPP TS 22.280 [17] | Maximum number of simultaneously received group calls (Nc5) |  | Y | Y | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Authorised to make a private call | Y | Y | Y | Y |
| [R-5.6.5-001] of 3GPP TS 22.179 [2] | Authorised to make a private call with manual commencement | Y | Y | Y | Y |
| [R-5.6.5-003] of 3GPP TS 22.179 [2] [R-6.7.3-007] of 3GPP TS 22.280 [17] | List of user(s) who can be called in private call |  |  |  |  |
|  | > MCPTT ID | Y | Y | Y | Y |
|  | > User info ID | Y | N | Y | Y |
|  | > ProSe discovery group ID | Y | N | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCPTT ID (see NOTE 4) | Y | Y | Y | Y |
| [R-6.7.4-004] of 3GPP TS 22.280 [17] | > Presentation priority relative to other users and groups (see NOTE 2) | Y | Y | Y | Y |
| [R-5.6.5-003] of 3GPP TS 22.179 [2] | Authorised to make a private call to users not included in "list of user(s) who can be called in private call" | Y | Y | Y | Y |
| [R-5.6.5-002] of 3GPP TS 22.179 [2] | Authorised to make a private call with automatic commencement | Y | Y | Y | Y |
| [R-5.6.3-011], [R-6.7.4-010] of 3GPP TS 22.179 [2] | Authorisation of user to force automatic answer for a private call | Y | Y | Y | Y |
| [R-5.6.5-006], [R-6.7.5-002] of 3GPP TS 22.179 [2] | Authorised to restrict the provision of a notification of call failure reason for private call | Y | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Authorisation to protect confidentiality and integrity of media in a private call (see NOTE 1) | Y | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Authorisation to protect confidentiality and integrity of floor control signalling in a private call (see NOTE 1) | Y | Y | Y | Y |
| [R-5.6.2.2.1-001] of 3GPP TS 22.280 [17] | Authorisation to make an MCPTT emergency group call functionality enabled for user | Y | Y | Y | Y |
| [R-5.6.2.4.1-001] of 3GPP TS 22.280 [17] | Group used on initiation of an MCPTT emergency group call (see NOTE 7) | Y | Y | Y | Y |
| [R-5.6.2.4.1-001] of 3GPP TS 22.280 [17] | Recipient for an emergency private MCPTT call (see NOTE 7) |  |  |  |  |
|  | > MCPTT ID | Y | Y | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCPTT ID (see NOTE 4) | Y | Y | Y | Y |
| [R-5.6.2.2.2-005] of 3GPP TS 22.280 [17] | Authorisation to cancel an in progress emergency associated with a group | Y | Y | Y | Y |
| [R-5.6.2.2.3-001] of 3GPP TS 22.280 [17] | Authorised to make an Imminent Peril group call | Y | Y | Y | Y |
| [R-5.6.2.2.3-009] of 3GPP TS 22.280 [17] | Group used on initiation of an MCPTT imminent peril group call (see NOTE 8) | Y | Y | Y | Y |
| [R-5.6.2.2.2-002] of 3GPP TS 22.280 [17] | Authorised for imminent in- peril cancelation | Y | Y | Y | Y |
| [R-5.6.2.3.1-001] of 3GPP TS 22.179 [2] | Authorised to make an emergency private call | Y | Y | Y | Y |
| [R-5.6.2.3.2-001] of 3GPP TS 22.179 [2] | Authorised to cancel emergency priority in a private emergency call by an authorized user | Y | Y | Y | Y |
| [R-5.6.2.4.1-002] of 3GPP TS 22.280 [17] | Authorised to activate emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-013] of 3GPP TS 22.280 [17] | Automatically trigger a MCPTT emergency communication after initiating the MCPTT emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.2-002] of 3GPP TS 22.280 [17] | Authorisation to cancel an MCPTT emergency alert | Y | Y | Y | Y |
| [R-5.1.7-002] and [R-6.8.7.2-007] and [R-6.8.7.2-008] of 3GPP TS 22.280 [17] | Priority of the user (see NOTE 9) |  | Y | Y | Y |
| [R-5.2.2-003] of 3GPP TS 22.280 [17] | Authorisation to create a group-broadcast group |  |  | Y | Y |
| [R-5.2.2-003] of 3GPP TS 22.280 [17] | Authorisation to create a user-broadcast group |  |  | Y | Y |
| [R-5.3-003], [R-6.12-001], [R-7.2-005] of 3GPP TS 22.280 [17] | Authorisation to provide location information to other MCPTT users on a call when talking |  | Y | Y | Y |
| 3GPP TS 23.283 [20] | Authorised to use LMR E2EE for interworking | Y | Y | Y | Y |
| 3GPP TS 23.283 [20] | > List of supported LMR technology types |  |  |  |  |
| 3GPP TS 23.283 [20] | >> LMR technology type (P25, TETRA etc.) | Y | N | Y | Y |
| 3GPP TS 23.283 [20] | >> URI of LMR key management functional entity (see NOTE 6) | Y | N | Y | Y |
| 3GPP TS 23.283 [20] | >> LMR specific identity (RSI for P25 or ITSI for TETRA) (see NOTE 5) | Y | N | Y | Y |
| 3GPP TS 23.283 [20] | >>LMR specific security information (see NOTE 5) | Y | N | Y | Y |
| [R-6.12-003] of 3GPP TS 22.280 [17] | Authorised to restrict the dissemination of the location information | Y | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [11]. NOTE 2: The use of this parameter by the MCPTT UE is outside the scope of the present document. NOTE 3: As specified in 3GPP TS 23.280 [16], for each MCPTT user's set of MCPTT user profiles, only one MCPTT user profile shall be indicated as being the pre-selected MCPTT user profile. NOTE 4: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [16]. NOTE 5: This is an LMR specific parameter with no meaning within MC services. NOTE 6: The LMR key management functional entity is part of the LMR system and is outside the scope of the present document. NOTE 7: This parameter is used for the emergency communication and also used as a target of the emergency alert request. At most one of them is configured; i.e. emergency communication will go to either a group or a user. If both are not configured the MCPTT user's currently selected group will be used. NOTE 8: This group, if configured, will be used for imminent peril communication. If not configured the MCPTT user's currently selected group will be used. NOTE 9: The use of the parameter is left to implementation. |  |  |  |  |  |

Table A.3-2: MCPTT user profile data (on network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server | MCPTT user database |
| --- | --- | --- | --- | --- | --- |
| [R-5.1.5-001], [R-5.1.5-002], [R-5.10-001], [R-6.4.7-002], [R-6.8.1-008] of 3GPP TS 22.280 [17] | List of on-network MCPTT groups for use by an MCPTT user |  |  |  |  |
|  | > MCPTT Group ID | Y | Y | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of group (see NOTE 3) | Y | Y | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | Y | Y | Y |
| [R-6.2.3.7.2-006] of 3GPP TS 22.179 [2] | > Authorisation of an MCPTT user to change the maximum number of simultaneous talkers | Y | Y | Y | Y |
| Subclause 5.2.5 of 3GPP TS 23.280 [16] | List of groups user implicitly affiliates to after MCPTT service authorization for the user |  |  |  |  |
|  | > MCPTT Group IDs | Y | Y | Y | Y |
| [R-6.4.2-006] of 3GPP TS 22.280 [17] | Authorisation of an MCPTT user to request a list of which groups an MCPTT user has affiliated to |  | Y | Y | Y |
| [R-6.4.6.1-002], [R-6.4.6.1-003] of 3GPP TS 22.280 [17] | Authorisation to change affiliated groups of other specified user(s) |  | Y | Y | Y |
| [R-6.4.6.2-001], [R-6.4.6.2-002] of 3GPP TS 22.280 [17] | Authorisation to recommend to specified user(s) to affiliate to specific group(s) |  | Y | Y | Y |
| [R-6.6.1-004] of 3GPP TS 22.280 [17] | Authorisation to perform regrouping | Y | Y | Y | Y |
| [R-6.7.2-001] of 3GPP TS 22.280 [17] | Presence status is available/not available to other users | Y | Y | Y | Y |
| [R-6.7.1-002], [R-6.7.2-002] of 3GPP TS 22.280 [17] | List of MCPTT users that an MCPTT user is authorised to obtain presence of |  |  |  |  |
|  | > MCPTT IDs | Y | Y | Y | Y |
| [R-6.7.2-003] of 3GPP TS 22.280 [17] | User is able/ unable to participate in private calls | Y | Y | Y | Y |
| [R-6.7.1-004], [R-6.7.2-003], [R-6.7.2-004] of 3GPP TS 22.280 [17] | Authorisation to query whether MCPTT User is available for private calls |  | Y | Y | Y |
| [R-6.7.1-010] of 3GPP TS 22.179 [2] | Authorisation to override transmission in a private call | Y | Y | Y | Y |
| [R-6.7.1-013] of 3GPP TS 22.179 [2] | Authorisation to restrict provision of private call set-up failure cause to the caller |  | Y | Y | Y |
| [R-6.7.6-001] of 3GPP TS 22.179 [2] | Authorized to make a private call-back request | Y | Y | Y | Y |
| [R-6.7.6-004] of 3GPP TS 22.179 [2] | Authorized to cancel a private call-back request | Y | Y | Y | Y |
| [R-6.8.7.4.2-001], [R-6.8.7.4.2-002] of 3GPP TS 22.280 [17] | Authorisation of an MCPTT user to cancel an emergency alert on any MCPTT UE of any MCPTT user |  | Y | Y | Y |
| [R-6.13.4-001] of 3GPP TS 22.280 [17] | Authorisation for a MCPTT user to enable/disable an MCPTT user |  | Y | Y | Y |
| [R-6.13.4-003], [R-6.13.4-005], [R-6.13.4-006], [R-6.13.4-007] of 3GPP TS 22.280 [17] | Authorisation for an MCPTT user to (permanently /temporarily) enable/disable a UE |  | Y | Y | Y |
| [R-6.2.3.4-001] of 3GPP TS 22.179 [2] | Authorisation to revoke permission to transmit |  | Y | Y | Y |
| [R-7.14-002], [R-7.14-003] of 3GPP TS 22.280 [17] | Authorization for manual switch to off-network while in on-network | Y | Y | Y | Y |
| [R-5.1.5-004] of 3GPP TS 22.280 [17] | Limitation of number of affiliations per user (N2) | N | Y | Y | Y |
| [R-5.5.2-009] of 3GPP TS 22.179 [2] | Maximum number of simultaneous transmissions received in one group call for override (N7) |  | Y | Y | Y |
| [R-6.4.6.1-001], [R-6.4.6.1-004] of 3GPP TS 22.280 [17] | List of MCPTT users whose selected groups are authorized to be remotely changed |  |  |  |  |
|  | > MCPTT IDs | Y | Y | Y | Y |
| Subclause 10.15.3 | Authorization to make a first-to-answer call | Y | Y | Y | Y |
| [R-6.15.2.2.2-001] of 3GPP TS 22.280 [17] | Authorization to make a remotely initiated ambient listening private call | Y | Y | Y | Y |
| [R-6.15.2.2.3-001] of 3GPP TS 22.280 [17] | Authorization to make a locally initiated ambient listening private call | Y | Y | Y | Y |
| [R-6.15.3.2-001] of 3GPP TS 22.280 [17] | Authorization to make a remotely initiated private call | Y | Y | Y | Y |
| [R-6.15.3.2-003] of 3GPP TS 22.280 [17] | Authorization to make a remotely initiated group call | Y | Y | Y | Y |
| [R-5.9a-013] of 3GPP TS 22.280 [17] | Authorised to request association between active functional alias(es) and MCPTT ID(s) |  | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [17] | Authorised to take over a functional alias from another MCPTT user |  | Y | Y | Y |
|  | List of functional alias(es) of the MCPTT user |  |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Functional alias | Y | Y | Y | Y |
| [R-5.4.2-007a] of 3GPP TS 22.280 [17] | >> Maximum number of parallel emergency group calls | Y |  | Y | Y |
| [R-5.9a-018] of 3GPP TS 22.280 [17] | >> Criteria for automatic activation by the MCPTT server (see NOTE 6) | N | Y | Y | Y |
| [R-5.9a-017], [R-5.9a-018] of 3GPP TS 22.280 [17] | >> Criteria for automatic de-activation by the MCPTT server (see NOTE 6) | N | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [17] | >> Location criteria for activation | Y |  | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [17] | >> Location criteria for de-activation | Y |  | Y | Y |
|  | >> Manual de-activation is not allowed if the location criteria are met | Y |  | Y | Y |
| [R-5.9a-020] of 3GPP TS 22.280 [17] | List of functional aliases to which first-to-answer calls and private calls are allowed when using a certain functional alias |  |  |  |  |
|  | > Used functional alias | Y | Y | Y | Y |
|  | >> List of functional aliases which can be called |  |  |  |  |
|  | >>> Functional alias | Y | Y | Y | Y |
| [R-5.9a-021] of 3GPP TS 22.280 [17] | List of functional aliases from which first-to-answer calls and private calls can be received when using a certain functional alias |  |  |  |  |
|  | > Used functional alias | N | Y | Y | Y |
|  | >> List of functional aliases from which calls can be received |  |  |  |  |
|  | >>> Functional alias | N | Y | Y | Y |
| [R-6.7.3-007a] of 3GPP TS 22.280 [17] | List of user(s) from which private calls can be received |  |  |  |  |
|  | > MCPTT ID | Y | Y | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCPTT ID | Y | Y | Y | Y |
| [R-6.7.4-004] of 3GPP TS 22.280 [17] | > Presentation priority relative to other users and groups | Y | Y | Y | Y |
|  | Authorised to receive private calls from any other MCPTT ID (see NOTE 8) | Y | Y | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | List of partner MCPTT systems in which this profile is valid for use during migration |  |  |  |  |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCPTT system | Y | Y | Y | Y |
| Subclause 10.1.1 of 3GPP TS 23.280 [16] | > Access information for partner MCPTT system (see NOTE 4) | Y |  | Y | Y |
| Subclause 10.6.2.9 | Authorized to initiate or cancel group regrouping using a preconfigured regroup group | Y | Y | Y | Y |
| [R-6.6.4.2-002a] and [R-6.6.4.2-002b] of 3GPP TS 22.280 [17] | List of groups the client affiliates/de-affiliates when one or multiple criteria are met |  |  |  |  |
|  | > MCPTT Group ID | Y | Y | Y | Y |
|  | >> Criteria for affiliation (see NOTE 5) | Y | Y | Y | Y |
|  | >> Criteria for de-affiliation (see NOTE 5) | Y | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y | Y | Y | Y |
| [R-6.6.4.2-002] of 3GPP TS 22.280 [17] | List of groups the client affiliates after receiving an emergency alert |  |  |  |  |
|  | > MCPTT Group ID | Y | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y | Y | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Allow private call forwarding |  | Y | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Call Forwarding NoAnswer Timeout |  | Y | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Call forwarding turned on |  | Y | Y | Y |
| R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Target of the MCPTT private call forwarding |  |  |  |  |
| R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | > Target MCPTT ID (see NOTE 10) |  | Y | Y | Y |
| R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | > Target functional alias (see NOTE 10) |  | Y | Y | Y |
| R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Condition |  | Y | Y | Y |
| [R-5.6.3-014], [R-6.7.4-015] of 3GPP TS 22.179 [2] | Allow private call transfer (see NOTE 7) | Y | Y | Y | Y |
| [R-5.6.3-014], [R-6.7.4-015] of 3GPP TS 22.179 [2] | List of MCPTT users that the MCPTT user is authorised to use as targets for call transfer |  |  |  |  |
| [R-5.6.3-014], [R-6.7.4-015] of 3GPP TS 22.179 [2] | > MCPTT ID |  | Y | Y | Y |
| [R-5.6.3-014], [R-6.7.4-015] of 3GPP TS 22.179 [2] | List of functional aliases that the MCPTT user is authorised to use as targets for call transfer |  |  |  |  |
| [R-5.6.3-014], [R-6.7.4-015] of 3GPP TS 22.179 [2] | > Functional alias |  | Y | Y | Y |
| ] [R-5.6.3-014], [R-6.7.4-015] of 3GPP TS 22.179 [2] | Authorised to transfer private calls to any MCPTT user | Y | Y | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Authorised to forward private calls based on manual input to any MCPTT user (see NOTE 9) | Y | Y | Y | Y |
| [R-5.10-001b] of 3GPP TS 22.280 [17] | Maximum number of successful simultaneous MCPTT service authorizations for this user (see NOTE 11) | N | Y | Y | Y |
| [R-6.15.5.3-001] of 3GPP TS 22.280 [17] | Authorised to initiate ad hoc group call | Y | Y | Y | Y |
| R-6.15.5.3-003] of 3GPP TS 22.280 [17] | Authorised to participate in ad hoc group call | Y | Y | Y | Y |
|  | Authorised to initiate emergency ad hoc group call | Y | Y | Y | Y |
|  | Authorised to initiate imminent peril ad hoc group call | Y | Y | Y | Y |
|  | Authorised to receive the participants information of an ad hoc group call | Y | Y | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [16]. NOTE 2: The use of this parameter by the MCPTT UE is outside the scope of the present document. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [16]. NOTE 4: Access information for each partner MCPTT system comprises the list of information required for initial UE configuration to access an MCPTT system, as defined in table A.6-1 of 3GPP TS 23.280 [16] NOTE 5: The criteria may consist of conditions such as the MCPTT user location or the active functional alias of the MCPTT user. NOTE 6: The criteria may consist of conditions such MCPTT user location or time. NOTE 7: Defines the right to perform a call transfer. For call transfer the MCPTT server does not check if the initial originating MCPTT user has the right to make a private MCPTT call to the final destination MCPTT user. NOTE 8: This parameter only applies to MCPTT users which are in the same security domain. NOTE 9: Defines the right to perform a call forwarding based on manual user input. For call forwarding based on manual user input the MCPTT server does not check if the initial originating MCPTT user has the right to make a private MCPTT call to the final destination MCPTT user. NOTE 10: Either the Target MCPTT ID or the Target functional alias may be present (but not both). NOTE 11: If configured, this value has precedence over the system level parameter "maximum number of successful simultaneous service authorisations" in table A.5-2. If not configured, the corresponding parameter from table A.5-2 shall be used. |  |  |  |  |  |

Table A.3-3: MCPTT user profile data (off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server | MCPTT user database |
| --- | --- | --- | --- | --- | --- |
| [R-7.2-003], [R-7.6-004] of 3GPP TS 22.280 [17] | List of off-network MCPTT groups for use by an MCPTT user | Y | N | Y | Y |
|  | > MCPTT Group ID | Y | N | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of group (see NOTE 3) | Y | N | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | N | Y | Y |
| [R-7.3.3-008] of 3GPP TS 22.179 [2] | Allowed listening of both overriding and overridden | Y | N | Y | Y |
| [R-7.3.3-006] of 3GPP TS 22.179 [2] | Allowed transmission for override (overriding and/or overridden) | Y | N | Y | Y |
| [R-7.8.1-001] of 3GPP TS 22.280 [17] | Authorization for participant to change an off-network group call in-progress to off-network emergency group call | Y | N | Y | Y |
| [R-7.8.3.1-003] of 3GPP TS 22.280 [17] | Authorization for participant to change an off-network group call in-progress to off-network imminent peril group call | Y | N | Y | Y |
| [R-7.12-002], [R-7.12-003] of 3GPP TS 22.280 [17] | Authorization for off-network services | Y | N | Y | Y |
| Subclauses 10.6.3, 10.7.3 | User info id (as specified in 3GPP TS 23.303 [7]) | Y | N | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [16]. NOTE 2: The use of this parameter by the MCPTT UE is outside the scope of the present document. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [16] |  |  |  |  |  |

* * * End of Changes * * * *