3GPP TSG-SA WG6 Meeting #53	S6-230955

Athens, Greece, 27th February – 3rd March 2023	(revision of S6-230541, 230027)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0040 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of API for DAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | API related with DAA is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | API for messages related with DAA is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | API related with DAA will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.x (new), 8.2.x.1 (new), 8.2.x.2 (new), 8.2.x.3 (new), 8.2.x.4 (new), 8.2.x.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## * * *   First Change   * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Manage_C2OperationMode | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| UAE_DAASupport API | Manage_DAAManagement | UAS application specific server | Request/ Response |
|  | Notify_DAAManagementComplete | UAS application specific server | Subscribe/notify |
|  | Notify_DAAClientSupportEvent | UAS application specific server | Subscribe/notify |
|  | Manage_DAAServerSupportEvent | UAS application specific server | Request/ Response |
| NOTE: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Configure_C2OperationMode |  |  |  |

## * * *   First Change   * * *

### 8.2.x	UAE_DAASupport API

#### 8.2.x.1	General

API description: This API enables the UAS application specific server to communicate with the UAE server for the DAA application policy and send and receive notifications related with DAA support aspects between the USS and the UAE client.

#### 8.2.x.2	Manage_DAAManagement operation

API operation name: Manage_DAAManagement

Description: Manage (initiate, change or delete) configuration of the DAA application policy.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.7.3.1.

Outputs: Refer clause 7.7.3.2.

See clause 7.7.2.1.1 for the details of usage of this API operation.

#### 8.2.x.3	Notify_DAAManagementComplete operation

API operation name: Notify_DAAManagementComplete

Description: Notification about the DAA management completion by UAE server.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.7.3.3.

Outputs: None.

See clause 7.7.2.1.1 for the details of usage of this API operation.

#### 8.2.x.4	Notify_DAAClientSupportEvent operation

API operation name: Notify_DAAClientSupportEvent

Description: Notification about possible UAVs in proximity, identified by the client.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.7.3.6.

Outputs: Refer clause 7.7.3.7.

See clause 7.7.2.2.1 for the details of usage of this API operation.

#### 8.2.x.5	Manage_DAAServerSupportEvent operation

API operation name: Manage_DAAServerSupport

Description: Notification about possible UAVs in proximity, identified by the server.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.7.3.8.

Outputs: Refer clause 7.7.3.9.

See clause 7.7.2.2.2 for the details of usage of this API operation.

## * * *   End of Change   * * *