3GPP TSG-SA WG6 Meeting #53	S6-230966

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230646)

Source:	Huawei

Title:	Add procedures and information flows of communication management

Spec:	3GPP TS 23.435 v0.5.0

Agenda item:	8.10

Document for:	Approval

Contact:	Xiaoqian JIA, jiaxiaoqian@huawei.com

1. Introduction

The communication service lifecycle management exposure is approved in NSCE study phase. The NSCE server supports the communication service lifecycle management exposure to VAL server. The NSCE server acquires the communication services related requirements which are from the vertical industry perspective, evaluates these requirements and then determines the network slice by pre-configured industry profiles or by KQI-KPI translation algorithms. After the network slice is determined the NSCE server allocate proper network slice resources to support the communication services. The NSCE server also enables the VAL server to update or terminate the communication services if the communication service is downgrade/upgrade or needed to be released.

2. Reason for Change

To add the functionality of communication servcie lifecycle management exposure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v0.5.0.

* * * First Change * * * *

9.x	Communication service lifecycle management exposure

9.x.1	General

The NSCE server supports the communication service lifecycle management exposure to VAL server. The NSCE server acquires the application services related requirements for a specific VAL service from the vertical industry perspective, evaluates these requirements and then determines the network slice by pre-configured industry mapping relations or by KQI-KPI translation algorithms. After the network slice requirements are determined the NSCE server allocate proper network slice resources to support the application services. The NSCE server also enables the VAL server to  reconfigure or disengage the communication services for the application. The procedures in clause 9.x.2 are for the creation, reconfiguration and disengagement of communication service respectively.

9.x.2	Procedure

9.x.2.1	Procedures on communication service lifecycle management exposure

9.x.2.1.1	Communication Service Creation

Figure 9.x.2.1.1-1 illustrates the procedure of communication service creation. The NSCE helps to allocate network slice resources to support the application service required by the verticals.

Pre-conditions:

1.	The VAL server has registered to receive NSCE services.

Figure 9.x.2.1.1-1: Communication service creation

1.	The VAL server sends a request to NSCE server to create a communication service to support a specific application service, e.g., the VAL server wants to create a video streaming service in a future factory, the communication service creation request carries the identifier of the video streaming service and the corresponding service attributes.

2.	NSCE server translates the application service requirements (e.g., for a video streaming service, the service location and the resolution of the video) and then perform slice determination (e.g., dLtThptPerSlice, uLtThptPerSlice, latency as defined in serviceProfile TS 28.541[14]). The NSCE may perform the translation by pre-configured industry profiles or by KQI-KPI translation algorithms which are out scope of standard. The procedures of APIs translation defined in clause 9.3 is referred to.

3.	NSCE server initiates the Slice Service subscription procedures by utilizing the management service of network slice creation as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5. The slice creation request may fail due to the shortage of network resources or other causes.

4.	NCSE server sends the communication service creation response to VAL server. If the slice creation is succeeded in step3, then the response from NSCE server should include the attributes and the values of network slice determined by NSCE server. If the slice creation failed in step3, the NSCE server shall indicate the cause of the creation failure, e.g. the shortage of network slice resources.

9.x.2.1.2	Communication Service Reconfiguration

Figure 9.x.2.1.2-1 illustrates the procedure of communication service reconfiguration to support the application service. The NSCE server provides the APIs to enable the VAL server to reconfigure the application service related requirements in case the current application service is not satisfied or application service is adjusted.

Pre-conditions:

- The VAL server has registered to receive NSCE services.

- The VAL server has requested the NSCE server to create a communication service to support the application service.

Figure 9.x.2.1.2-1: Communication service reconfiguration

1.	VAL server sends the communication service reconfiguration request to the NSCE server to reconfigure the properties of the application service (e.g., In case there is a downgrade/upgrade of the application service where the application service profile is changed).

2.	NSCE server translates the application requirements according to the reconfigured application properties then updates the requested network slice.

3.	NSCE server initiates the Slice Service update procedures by utilize the management service of network slice modification as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5.

4.	NCSE server sends the communication service modification response to VAL server.

9.x.2.1.3	Communication Service disengagement

Figure 9.x.2.1.3-1 illustrates the procedure of communication service disengagement. The NSCE server provides the APIs to enable the VAL server to terminate an application service.

Pre-conditions:

- The VAL server has registered to receive NSCE services.

- The VAL server has requested the NSCE server to create a communication service to support the application service.

Figure 9.x.2.1.3-1: Communication service disengagement

1.	The VAL server sends a request to NSCE server to disengage the communication service when the application service is to be terminated.

2.	NSCE server initiates the Slice Service de-allocation procedures by utilizing the management service of network slice de-allocation as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5.

3.	NCSE server sends the communication service disengagement response to VAL server.

9.x.3	Information flows

9.x.3.1	General

The following information flows are specified for communication service management:

-	communication service creation

-	communication service reconfiguration

-	communication service disengagement

9.x.3.2	Communication service creation

Table 9.x.3.2-1 and Table 9.x.3.2-2 describe information elements for communication service creation request and response between the VAL server and the NSCE server.

Table 9.x.3.2-1: Communication service creation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service name | M | The name of the application service to be supported by the created communication service, the value can be as followings: V2X service; Video streaming service; Remote control service; … |
| VAL service ID | M | Identifier of the application service |
| Area of interest | M | The geographical or service area for which the application service profile applies. |
| Application service profile | M | The list of the requirements of the corresponding application service |
| > ReqInfo | M | The element containing the reqName and reqValue |
| >>ReqName | M | The name of the application service requirement, the value of this IE can be as followings: the resolution of a video service, the end user numbers, the latency, … |
| >>ReqValue | M | The corresponding value of the application service requirement |

Table 9.x.3.2-2: Communication service creation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID |  | M |  | Identifier of the application service to be supported by the created communication service. |  |
| Result | M | Indicates the success or failure of the communication service creation |
| Network slice info List | O see NOTE 1) | The list of the network slice info determined by NSCE |
| > Network slice info | O (see NOTE 1) | The network slice info which includes the attributes and the corresponding values of network slice |
| >>S-NSSAI | O (see NOTE 1) | The identifier of network slice |
| >>attributes of network slice | O (see NOTE 1) | The list of attributes of the serviceProfile e.g, dLtThptPerSlice or latencies of network slice as defined in serviceProfile TS 28.541[10] |
| >>AttributeValues | O (see NOTE 1) | The corresponding values of the attributes of the service profiles that determined by the NSCE server |
| Cause | O (see NOTE 2) | Indicates the cause of creation failure |
| NOTE 1: Shall be present if the result is success and shall not be present otherwise NOTE 2: Shall be present if the result is failure and shall not be present otherwise |  |  |

9.x.3.3	Communication service reconfiguration

Table 9.x.3.3-1 and Table 9.x.3.3-2 describe information elements for communication service reconfiguration request and response between the VAL server and the NSCE server.

Table 9.x.3.3-1: Communication service reconfiguration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID |  | M |  | The identifier of the VAL server |  |
| VAL service name | M | The name of the application service to be upgrade/downgrade which requires the reconfiguration of the communication service, the value can be as followings: V2X service; Video streaming service; Remote control service; … |
| VAL service ID | M | Identifier of the application service |
| Area of interest | M | The geographical or service area for which the application profile applies. |
| Application service profile | M | The list of the requirements of the corresponding application service to be changed |
| > ReqInfo | M | The element containing the reqName and reqValue |
| >>ReqName | M | The name of the application service requirement, the value of this IE can be as followings: the resolution of a video service, the end user numbers, the latency, … |
| >>ReqValue | M | The corresponding updated value of the application service requirement |

Table 9.x.3.3-2: Communication service reconfiguration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | Identifier of the application service |
| Result | M | Indicates the success or failure of the communication service reconfiguration |
| Network slice info List | O (see NOTE 1) | The list of the network slice info updated by NSCE |
| > Network slice info | O (see NOTE 1) | The network slice info which includes the attributes and the corresponding values of network slice |
| >>S-NSSAI | O (see NOTE 1) | The identifier of network slice |
| >>Attributes of network slice | O (see NOTE 1) | The list of attributes of the serviceProfile e.g., dLtThptPerSlice or latencies of network slice as defined in serviceProfile TS 28.541[10] |
| >>AttributeValues | O (see NOTE 1) | The corresponding values of the attributes of the service profiles that updated by the NSCE server |
| Cause | O(see NOTE 2) | Indicates the cause of reconfiguration failure |
| NOTE 1: Shall be present if the result is success and shall not be present otherwise NOTE 2: Shall be present if the result is failure and shall not be present otherwise |  |  |

9.x.3.4	Communication service disengagement

Table 9.x.3.4-1: Communication service disengagement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service name | M | The name of the application service to be terminated which requires the disengagement of the communication service, the value can be as followings: V2X service; Video streaming service; Remote control service; … |
| VAL service ID | M | Identifier of the application service |

Table 9.x.3.4-2: Communication service disengagement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the communication service disengagement request |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | Identifier of the application service |
| Cause | O(see NOTE) | Indicates the cause of disengagement failure |
| NOTE: Shall be present if the result is failure and shall not be present otherwise |  |  |

9.x.4	APIs

9.x.4.1	General

Table 9.x.4.1-1 and 9.x.4.1-2 illustrate the API for communication service management exposure.

Table 9.x.4.1-1: SS_NSCE_CommServiceCreation

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_CommService_Creation | CommService_Creation_Request | Request /Response | VAL server |

Table 9.x.4.1-2 SS_NSCE_ CommServiceReconfiguration

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_CommService_Reconfiguration | CommService_Reconfiguration_Request | Request /Response | VAL Server |

Table 9.x.4.1-3: SS_NSCE_ CommServiceDisengagement

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_CommService_Disengagement | CommService_Disengagement_Request | Request /Response | VAL Server |

9.x.4.2	SS_NSCE_CommService_Creation API

API operation name: CommService_Creation

Description: The consumer requests to create the communication service

Known Consumers: VAL server.

Inputs: See table 9.x.3.2-1.

Outputs: See table 9.x.3.2-2.

See clause 9.x.2.1.1 for details of usage of this operation.

9.x.4.3	SS_NSCE_CommService_Reconfiguration API

API operation name: CommService_Reconfiguration

Description: The consumer requests to report reconfigure the communication service.

Known Consumers: VAL server.

Inputs: See table 9.x.3.3-1.

Outputs: See table 9.x.3.3-2.

See clause 9.x.2.1.2 for details of usage of this operation.

9.x.4.4	SS_NSCE_CommService_Disengagement API

API operation name: CommService_Disengagement

Description: The consumer requests to disengagement the communication service.

Known Consumers: VAL server.

Inputs: See table 9.x.3.4-1.

Outputs: See table 9.x.3.4-2.

See clause 9.x.2.1.3 for details of usage of this operation.