3GPP TSG-SA WG6 Meeting #53	S6-230976

Athens, Greece 27th February – 3rd March		(revision of S6-230412, S6-230548)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0148 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add information flow for location reporting configuration update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion of KI#3 in TR 23.700-96, the solution#5 will be considered in the normative phase. In the step 4 of sol#5, the LMS will inform the LMC for the LCS profile mapping configuration. However there is no related information flow between LMS and LMC for above procedure in the TS 23.434, so the corresponding information flow should be added. According to the conclusion of KI#2 in TR 23.700-96, the solution#7 will be considered in the normative phase. Based on the sol#7, the LMS enhanced with FLF could select one or more access types and/or location methods for the target UE based on the requested location QoS, and send the LMC and/or 3GPP CN with the selected access types and location methods. So the “Location reporting configuration notification”(which has been documented in S6-230412 that has been approved in SA6#52-bis-e meeting) and “Location reporting configuration response” from LMS to LMC should add the optional access types and location methods paramaters to align with the conclusion of KI#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add information flow for location reporting configuration update. The version 2 just to add the access types and location methods paramaters in “Location reporting configuration response” and “Location reporting configuration notification” information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LMS will not inform the LMC for the LCS profile mapping configuration during the procedure of location profiling for fused location service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1,9.3.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2	Information flows for location information

9.3.2.0	Location reporting configuration request

Table 9.3.2.0-1 describes the information flow from the location management client to the location management server for requesting the location reporting configuration.

Table 9.3.2.0-1: Location reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested. |

9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE) | Identifies what location information is requested |
| Requested location access type | O | Identity of the location access type for which the location information is requested, e.g. as described in clause 6.1.6.3.6 and 6.1.6.3.17 of TS 29.572[x]. |
| Requested positioning method | O | Identity of the positioning method for which the location information is requested, e.g. as described in clause 6.1.6.3.6 of TS 29.572[x]. |
| Triggering criteria | O (see NOTE) | Identifies when the location management client will send the location report |
| Minimum time between consecutive reports | O (see NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

9.3.2.x	Location reporting configuration notification

Table 9.3.2.x-1 describes the information flow from the location management server to the location management client for the location reporting configuration notification. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.x-1: Location reporting configuration notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| VAL service ID | O (see NOTE) | Identity of the VAL service for which the location reporting configuration is notified. |
| Requested location access type | O | Identity of the location access type for which the location information is requested, e.g. as described in clause 6.1.6.3.6 and 6.1.6.3.17 of TS 29.572[x]. |
| Requested positioning method | O | Identity of the positioning method for which the location information is requested, e.g. as described in clause 6.1.6.3.6 of TS 29.572[x]. |
| Triggering criteria | O (see NOTE) | Identifies when the location management client will send the location report |
| Minimum time between consecutive reports | O (see NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

* * * End of Changes * * * *