3GPP TSG-SA WG6 Meeting #53	S6-230979

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230772, S6-230273)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0220 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to dynamic instantiation solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Dynamic EAS instantiation related changes that were agreed in SA6#52 meeting required corrections to the IE structure. Further, IE Instantiable EAS information is incomplete without the information on applicability of that IE. Lastly, EAS instantiation status is incomplete without the inclusion of uninstantiable EAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed changes include: 1. Definition for Instantiable EAS added 2. “instantiable but not be instantiated yet” status information is not sufficient to mitigate the wastage of EDN resources. It is necessary to also know when the EAS can instantiated, for mitigating wastage of EDN resources. So, instantiable criteria is included. This change is proposed to EES Profile, EDN configuration information, EAS discovery response and EAS discovery notification. 3. EAS instantiation status information is not complete unless the list includes “uninstantiable EAS” information as well. If not included, mitigating wastage of EDN resources is not efficient. This change is proposed to EES Profile and EDN configuration information. 4. Service provisioning response includes EDN configuration information and not EES profiles. Hence, corrected the same in second change. 5. Usage of instantiable criteria and uninstantiable EAS is clarified in service provisioning and EAS discovery procedures. 6. Clarification added that the Dynamic EAS instantiation must be only for the instantiable EASs. 7. Selected EAS ID must be either from the instantiated or instantiable in the EAS information provisioning request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dynamic EAS instantiation solution is not efficient to mitigate wastage of EDN resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 8.2.6, 8.3.3.2.2, 8.3.3.3.3, 8.5.2.2, 8.5.3.3, 8.5.3.6, 8.12.1, 8.15.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Key Issue#9 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Context: A set of data about the Application Client that resides in the Edge Application Server.

Application Context Relocation: Refers to the end-to-end service continuity procedure described in clause 8.8.

Application Context Transfer: Refers to the transfer of the Application Context between the source Edge Application Server and the target Edge Application Server, which is a part of the service continuity procedure described in clause 8.8.

Application Server: Application software resident in the cloud performing the server function.

Edge Computing: A concept, as described in 3GPP TS 23.501 [2], that enables operator and 3rd party services to be hosted close to the UE's access point of attachment, to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network.

Edge Computing Service Provider: A mobile network operator or a 3rd party service provider offering Edge Computing service.

Edge Data Network: A local Data Network that supports the architecture for enabling edge applications.

EEC Context: A set of data about the Edge Enabler Client that resides in the Edge Enabler Server.

Edge Enabler Layer: Refers to the overall functionality provided by the entities such as Edge Enabler Client, Edge Enabler Server and Edge Configuration Server, in support of applications as per the architecture defined in clause 6.

Edge Hosting Environment: An environment providing support required for Edge Application Server's execution.

Instantiable EAS: EAS type for which the instantiation trigger from the Edge Enabler Layer is considered by the EAS management system for instantiating EAS.

* * * Second Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (NOTE) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| EES Provider Identifier | O | The identifier of the ECSP that provides the EES Provider. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| NOTE: " Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

* * * Third Change * * * *

##### 8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

If the ECS is not provisioned with any EDN configuration information or is unable to determine the EES information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

When the received EDN configuration information includes instantiable EAS information, the EEC can select an EES corresponding to EAS that can be instantiated or instantiable. For EAS discovery to mitigate the waste of EDN resources EEC considers the instantiable EAS information and the associated instantiation criteria, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated.

NOTE 4:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 5:	Even after the EEC establishes a connection to the EES using information received in step 3, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

NOTE 6:	If the EAS instantiation fails based on the selected EES, the EEC may retry the EAS discovery request to another EES.8.3.3.2.3	Subscribe-notify model

* * * Fourth Change * * * *

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (NOTE 2) | O | List of EASID registered or expected to be registered with the EES. |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (NOTE 3) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| > EES Provider identifier | O | The identifier of the EES Provider (such as ECSP) |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: " Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

* * * Fifth Change * * * *

#### 8.5.3.3	EAS discovery response

Table 8.5.3.3-1 describes information elements for the EAS discovery response from the EES to the EEC.

Table 8.5.3.3-1: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list | O | List of discovered EAS(s). Each element includes the information described below. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (NOTE 2) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |
| NOTE 2: " Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

* * * Sixth Change * * * *

#### 8.5.3.6	EAS discovery notification

Table 8.5.3.6-1 describes the information elements for EAS discovery notification from the EES to the EEC.

Table 8.5.3.6-1: EAS discovery notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Event ID | M | Either EAS discovery notification or EAS dynamic information notification |
| EAS list | O | List of discovered EAS profiles. Each element includes the information described below. |
| > EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| > Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (NOTE 1) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| NOTE 1: " Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

* * * Eighth Change * * * *

### 8.12.1	General

The EES may trigger the EAS instantiation dynamically due to e.g., EAS discovery request, UE mobility, upon receiving EEC Registration request containing AC profile.

Upon receiving the EAS discovery request with EAS discovery filter from the EEC or the S-EES during the procedures for EAS discovery or ACR, the EES may fail to discover and select the EAS that matches the UE location and the requesting application characteristics specified in table 8.5.3.2-2 due to no EAS is available or instantiated. The EES may trigger the EAS management system to instantiate the EAS serving the AC in the EDN corresponding to the EAS that can be instantiable before returning the EAS information to the EEC or S-EES, based on the pre-configured dynamic EAS instantiation information about which EAS can be dynamically instantiated at the associated EDN.

NOTE 1:	The EAS management system is responsible for the authorization of the dynamic EAS instantiation.

NOTE 2:	The EAS management system can provide the pre-configured dynamic EAS instantiation information to the EES. Such a mechanism is out of scope of this release of the present document.

NOTE 3:	Dynamic EAS instantiation information at the EES is left implementation specific in this version of the specification.

* * * Ninth Change * * * *

#### 8.15.3.2	EAS information provisioning request

Table 8.15.3.2-1 describes the information elements for EAS information provisioning request from the EEC to the EES.

Table 8.15.3.2-1: EAS information provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | The identifier of the EEC. |
| ACID | M | The identifier of the AC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Selected EAS ID | O | The identifier of the selected EAS, which is either instantiated or instantiable. |
| Selected EAS Endpoint | O | The endpoint of the selected EAS |
| Request type | O | Request types: - ACR scenario selection announcement - ACR scenario selection request |
| Selected ACR scenario list (see NOTE 1) | O | The list of ACR scenarios selected by the EEC |
| AC Profile (see NOTE 2, NOTE 3) | O | AC Profile as described in Table 8.2.2-1 |
| EEC Service Continuity Support (NOTE 2) | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| NOTE 1: The IE may be present only if Selected EASID and Selected EAS Endpoint are present and Request type is “ACR scenario selection announcement” NOTE 2: The IEs are present only if request type is “ACR scenario selection request” NOTE 3: The IE is present if AC Profile is not shared to EES previously |  |  |

* * * End of Changes * * * *