3GPP TSG-SA6 Meeting #53	S6-230991

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023							(was S6-230640)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0238 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of service provisioning overview |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone Italia SpA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 01-Mar-2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Service provisioning overview description has a missing step and figure errors |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of initial trigger for service provisioning added to text and figure. Trigger for initial provisioning shown as arrow and not box Text description added for the UE requests service provisioning step. EEC registration to the EES changed to optional in the figure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of service provisioning is missing step of UE requests service provisioning. Figure incorrectly shows EEC registration to EES as mandatory. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** change 1 ***

## 8.3	ECS Discovery and Service provisioning

### 8.3.1	General

Service provisioning allows configuring the EEC with information about available Edge Computing services, based on the hosting UEs location, service requirements, service preferences and connectivity. This configuration includes the necessary address information for the EEC to establish connection with the EES(s).

If the ECS deployed by MNO is contracted with one or more ECSP(s), the ECS provides EES configuration information of MNO owned and ECSP owned EESs via MNO ECS as described in clause 8.3.3.3.3.

If the ECS is deployed by a non-MNO ECSP, the ECS endpoint address may be configured with the EEC. An EEC that is aware of multiple ECSP's ECS endpoint addresses may perform the service provisioning procedure per ECS of each ECSP multiple times.

Figure 8.3.1-1 illustrates an overview of service provisioning. Service provisioning procedures support the following models:

-	Request/Response model; and

-	Subscribe/Notify model.

initial trigger for service provisioning e.g. start of an edge application

UE requests service provisioning

UE configured to connect to the EDN

ECS requests service provisioning

EEC registers to EES (controlled by EES profile, needed for functions that require an EEC context)

UE is given ECS connection configuration

UE consuming edge services

service provisioning triggered at ECS e.g. new EAS installed

service provisioning triggered at UE e.g. new application installed, UE changes location

SMS trigger

Figure 8.3.1-1: Overview of Service provisioning

The UE is given configuration information enabling it to connect to the ECS. The UE can then request initial provisioning from the ECS to obtain configuration required to connect to the EDN. Once provisioned, the EEC of the UE registers with the selected EES(s) from the list of provisioned EES(s) it received from the ECS(s), if the EEC registration configuration in EES profile indicates that EEC registration is required. The UE further consumes the edge computing services and performs various operations such as EAS discovery, Edge application communications, ACR, etc. While the UE is consuming the edge computing services, the UE or ECS may be triggeredto re-request service provisioning.

The triggers for service provisioning are classified as:

a.	Triggers at UE - Some examples are:

-	AC related updates available at the EEC due to AC installation/re-installation, AC requesting application server access (e.g. via internet browser);

-	EEC supporting one or more ACs may be updated due to EEC re-installation; and

-	Lifetime of EDN Configuration Information is expired or the EEC detects that the UE moves out of EDN Service Area in the EDN Configuration Information.

b.	Triggers at ECS – Some examples are:

-	EES updates received due to EAS installation/re-installation/re-location; and

-	ECS receives the EDN/DNAI change notification of the UE from 5GC when the ECS subscribes to the user plane path management events as specified in clause 8.10.2.