3GPP TSG-SA WG6 Meeting #53	S6-231023

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230758, 800)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0098 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the unicast delivery stop after MBS delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Hytera |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After the UE has received the DL media via the MBS session, the AS can stop the unicast delivery. however this is missing in the spec. In addition, the group communication can be initiated before the MBS session and delivered via unicast. However, this is not aligned in clause 7.3.3.5.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Introduce new NOTE in clause 7.3.3.5.2 to capture that the unicast deliver can be stopped after starting the MBS delivery. 2) add a new text to clarify that the group communication may be initiated before the MBS sesison and delivered over unicast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Waste of network resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** 1st change **********************/

##### 7.3.3.5.2	Procedure

The procedure in figure 7.3.3.5.2-1 describes how media related to a specific group communication can be distributed over a configured MBS session which consist of multiple QoS flows, i.e. addressing different service requirements. The procedure is applicable for both the pre-created MBS session case as described in 7.3.3.1.2 and the dynamic MBS session case as described in 7.3.3.1.3. For simplicity, the figure 7.3.3.5.2-1 shows that the MBS session is pre-created prior to the group communication establishment.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server has decided to use an MBS session for MC service group communications associated to MC service group X.

-	MC service clients 2 to n are within the MBS service area where the MBS session is configured.

Figure 7.3.3.5.2-1: MC service group media transmission over MBS sessions

1.	The MC service server creates a multicast or a broadcast MBS session targeting group communications associated to MC service group X, as being specified in 3GPP TS 23.247 [15]. Therefore, the MC service server can provide default service requirements to be addressed by the MBS session, e.g. associated to MC 5QIs and specific allocation and retention priority (ARP) to transmit the media associated to MC service group communications.

The MBS session is announced and received by MC service client 2 to n, which are within the MBS service area. The MC service server has identified that MC service clients 2 to n can receive media over the MBS sessions, e.g. based on a notification from the MC service clients indicating the successful join of the multicast MBS session or a monitoring report of the broadcast MBS session (similar to the listening status report used for MBMS).

2.	A new MC service group communication is established for the MC service group X consisting of a specific required QoS, e.g. an MC service emergency group communication. The group communication setup can be done over unicast.

2a. For broadcast MBS sessions, the session is established upon sending a session start request as part of the MBS session creation procedure, which is described in 3GPP TS 23.247 [15].

2b. For multicast MBS session, the session is established upon the acceptance of the first UE session join request initiated from the MC service UE towards the 5GS, as described in 3GPP TS 23.247 [15]. The multicast session can then have either an active or an inactive state.

3. 	The MC service server may send a multicast MBS session activation request towards the 5GC in order to activate the multicast MBS session in case the session has an inactive state. For this purpose, the MC service server indicates the MBS session ID to be activated.

4.	Considering that the established group communication requires a specific QoS, e.g. an MC service emergency group communication which requires higher priority (i.e. better ARP), the MC service server requests an MBS session update to the 5GS to provide the new required QoS, if not done during the MBS session creation in step 1. The MBS session should then be updated and an additional QoS flow may be configured.

5.	The MC service server sends a MapGroupToSessionStream to MC service clients 2 to n over the configured MBS session providing the required stream information to receive the media related to the specific established MC service group communication within the MBS session.

6.	MC service clients process the MapGroupToSessionStream information to receive the related media over the specific MBS session stream.

7.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

8.	The MC service server distributes the media to MC service clients 2 to n over the indicated stream within the established MBS session.

NOTE:	The MC service server can stop the unicast delivery (if ongoing) towards the MC service clients considering the UE session join notification or the MBS listening status report.

9.	The MC service server may send a multicast MBS session deactivation request towards the 5GC in order to deactivate the multicast MBS session. For this purpose, the MC service server indicates the MBS session ID to be deactivated.

10.	The MC service server may further trigger the UE to leave multicast MBS session.

/******************** end of  change **********************/