3GPP TSG-SA6 Meeting #53	S6-231032

(revision of S6-230892, S6-230562)

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0229 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS-specific UE ID (Edge UE ID) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V., Qualcomm, Intel, AT&T, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the FS_eEDGEAPP study we have studied the use of Edge UE ID which is maintained in the EEL and which can be used for the interaction in EDGE-3. As AF specific UE ID can be used for an interaction with the NEF, Edge UE ID can be used for interaction in EDGE-3. While AF specific UE ID is for AFs (doesn’t differentiate between the EAS and the EES) the Edge UE ID is generated for and used by the EAS. Since the Edge UE ID could be temporary and generated, it could be used as an access token for an interaction between the EAS and the EES for specific UE. This also allows the EES to pass on its own AF ID in the NEF’s Nnef_UEId and generate the Edge UE ID for the EAS. This Edge UE ID can then be used in the EDGE-3 intractions. Furthermore, previously agreed CRs S6-230462 and S6-230494 use Edge UE ID and the defintion of Edge UE ID is still unresolved. This CR adds the definition of Edge UE ID to TS 23.558 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of Edge UE ID is added to TS 23.558 clause 7.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Edge UE ID will remain undefined and the use of Edge UE ID in previously agreed CRs S6-230462 and S6-230494 will remain to be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 7.2.x	Edge UE ID

The Edge UE ID is an identifier that is associated with the UE ID (GPSI as per clause 7.2.6) and managed by EES. The EES can generate Edge UE ID as needed (e.g. as per ECSP policy). Edge UE ID can be shared with the EAS directly (using UE Identifier API as per clause 8.6.5) and/or indirectly via AC (using UE ID request as per clause 8.14.2.x) in order for it to be used by the EAS over EDGE-3 interactions when it is not desired to share the UE ID (GPSI as per clause 7.2.6) with the EAS. The Edge UE ID can be temporary to limit the access of the EAS when needed.

NOTE:		The Edge UE ID is not applicable for EDGE-7 interactions.