3GPP TSG-SA WG6 Meeting #53	S6-231065

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230857, S6-230674,

S6-230340, S6-230228)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0161 | rev | 4 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL server provisioning for Location Management Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specfieid in clause 9.3.13.1, the VAL service areas are configured by the VAL server to the location management server. Further, there is no need to specify API for each configuration (like VAL service area Id list). So, it is better design to have single provisioning API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a procedures and information flows for VAL server to provision requried configuration to the SEAL LMS. Removed / Made void – clauses related to VAL service area ID request/response/API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL LMS will not be aware of the default triggers and also VAL service areas. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.21, 9.3.2.22, 9.3.2.xx (NEW), 9.3.2.yy (NEW), 9.3.2.A (NEW), 9.3.2.B (NEW), 9.3.2.C (NEW), 9.3.2.D (NEW), 9.3.zz(NEW), 9.3.zz.1 (NEW), 9.3.zz.2 (NEW), 9.3.zz.3 (NEW), 9.3.zz.4 (NEW), 9.4.1, 9.4.8, 9.4.a (NEW), 9.4.a.1 (NEW), 9.4.a.2 (NEW), 9.4.a.3 (NEW), 9.4.a.4 (NEW), 9.4.a.5 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.21	Void

#### 9.3.2.22	Void

* * * Next Change * * * *

#### 9.3.2.xx	VAL server provisioning request

Table 9.3.2.xx-1 describes the information flow from the VAL server to the location management server for providing provisioning configuration.

Table 9.3.2.xx-1: VAL server provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Endpoint information | O | Information of the endpoint of the VAL server. |
| Security credentials | M | Security cradentials to authorize and authenticate VAL server |
| List of VAL service specific information | M | Provides the list of VAL service specific information |
| > VAL service ID | M | Identify of the VAL service for which the configuration information is provided. |
| > VAL Service Area ID | M | VAL Service area Identity |
| >> List of VAL Service Areas | O | List of VAL Service Areas identified distinctly through geographical area (e.g., co-ordinates) |

#### 9.3.2.yy	VAL server provisioning response

Table 9.3.2.yy-1 describes the information flow from the location management server to the VAL server as a response for providing provisioning configuration.

Table 9.3.2.yy-1: VAL server provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * Next Change * * * *

#### 9.3.2.A	VAL server obtain provisioning parameters request

Table 9.3.2.A-1 describes the information flow from a VAL server to a location management server for obtain VAL service area provisioning parameters.

Table 9.3.2.A-1: VAL server obtain provisioning parameter request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| VAL service ID | M | Identify of the VAL service for which the configuration information is required. |
| Security credentials | M | Security cradentials to authorize and authenticate VAL server |

* * * Next Change * * * *

#### 9.3.2.B	VAL server obtain provisioning parameters response

Table 9.3.2. -1 describes the information flow from a location management server to the VAL server for provide provisioning paramters for the VAL service.

Table 9.3.2.B-1: VAL server obtain provisioning parameter response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| > VAL Service Area ID (NOTE) | M | VAL Service area Identity |
| >> List of VAL Service Areas (NOTE) | O | List of VAL Service Areas identified distinctly through geographical area (e.g., co-ordinates) |
| NOTE: This IE is added only for successful case. |  |  |

* * * Next Change * * * *

#### 9.3.2.C	VAL server provisioning parameters delete request

Table 9.3.2.C-1 describes the information flow from a VAL server to a location management for delete provisioning parameters.

Table 9.3.2.C-1: VAL service provisioning parameters delete request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| Security credentials | M | Security cradentials to authorize and authenticate VAL server |
| List of VAL service IDs | M | List of one or more VAL service area identifiers that shall be deleted. |

* * * Next Change * * * *

#### 9.3.2.D	VAL server provisioning parameters delete response

Table 9.3.2.D-1 describes the information flow from a location management server to the VAL server to indicate results of the delete VAL service provisioning parameters request.

Table 9.3.2.D-1: VAL service provisioning parameters delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |

* * * Next Change * * * *

### 9.3.zz	VAL server provisioning for location management service

#### 9.3.zz.1	General

The high level procedure for VAL server to provision required information to SEAL location management server is described in the following subclause.

Editor’s note: The subscribe-notify model is FFS.

#### 9.3.zz.2	Providing provisioning parameters

The procedure for VAL server to provision required information to SEAL location management server is illustrated in figure 9.3.zz.2-1.

Figure 9.3.zz.2-1: VAL Server provisioning to SEAL Location Management Server

1.	The VAL server sends a request message to location management server to provision required information. The request message includes identity of the VAL server, endpoint information of the VAL server, security credentials of the VAL server, and service provider specific information like list of VAL service areas.

2.	Upon receiving the request, the location management server authorizes the request based on the security credentials provided in the request and considering the service level agreement between VAL service provider and SEAL service provider. If VAL server is authorized to use the SEAL service, then the location management server stores the details about the VAL server including list of VAL service areas. The location management server sends the response message to the VAL server.

NOTE: The same procedure can be used to update the provisioning parameters.

Editor’s note:	Whether the VAL server registers with location Management Server is FFS.

#### 9.3.zz.3	Obtaining provisioning parameters

Figure 9.3.zz.3-1 illustrates the high level procedure of the VAL service area retrieval.

Figure 9.3.zz.3-1: Obtain VAL service provisioning parameters procedure

1.	The VAL server sends the VAL server obtain provisioning parameters request to the location management server to retrieve the provisioning parameters. In the request message, the VAL server includes the information as specified in table 9.3.2.A-1.

2.	The location management server shall check if the VAL server is authorized to initiate VAL server obtain provisioning parameters request.

3.	If the VAL server is authorized to obtain the VAL service provisioning parameters, the location management server sends the VAL service area obtain response with the information as specified in table 9.3.2.B-1.

#### 9.3.zz.4	Deleting provisioning parameters

Figure 9.3.zz.4-1 illustrates the high level procedure of the VAL service area delete.

Figure 9.3.zz.4-1: Delete VAL service provisioning parameters procedure

1.	The VAL server sends the VAL service area delete request to the location management server to delete the VAL service area identifier(s) configured by the VAL server. In the request message, the VAL server includes the information as specified in table 9.3.2.C-1.

2.	The location management server shall check if the VAL server is authorized to initiate VAL service area delete request.

3.	The location management server checks whether the VAL service area IDs exist and then deletes the VAL service area identifier(s).

NOTE:	An deletion of the VAL service area ID(s) does not affect the existing subscriptions in the SEAL layer.

4.	The location management server sends the VAL service area delete response with the information as specified in table 9.3.2.D-1.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_LmsVALServerProvisioning | Provide_Configuration | VAL server | Request /Response |
|  | Obtain_VAL_Server_provisioning_parameter | VAL server | Request /Response |
|  | Update_VAL_Server_provisioning_parameter | VAL server | Request /Response |
|  | Delete_VAL_Server_provisioning_parameter | VAL server | Request /Response |

* * * Next Change * * * *

### 9.4.8	Void

* * * Next Change * * * *

### 9.4.a	SS_LmsVALServerProvisioning API

#### 9.4.a.1	General

API description: This API enables the VAL server to provision configuration for the VAL service to the SEAL LM-S.

#### 9.4.a.2	Provide_Configuration operation

API operation name: Provide_Configuration

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.xx

Outputs: See subclause 9.3.2.yy

See subclause 9.3.zz.2 for the details of usage of this API operation.

#### 9.4.a.3	Obtain_VAL_Server_provisioning_parameter

API operation name: Obtain_VAL_Server_provisioning_parameter

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.A

Outputs: See subclause 9.3.2.B

See subclause 9.3.zz.3 for the details of usage of this API operation.

#### 9.4.a.4	Update_VAL_Server_provisioning_parameter

API operation name: Update_VAL_Server_provisioning_parameter

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.xx

Outputs: See subclause 9.3.2.yy

See subclause 9.3.zz.2 for the details of usage of this API operation.

#### 9.4.a.5	Delete_VAL_Server_provisioning_parameter

API operation name: Delete_VAL_Server_provisioning_parameter

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.C

Outputs: See subclause 9.3.2.D

See subclause 9.3.zz.4 for the details of usage of this API operation.

* * * End of Change * * * *