3GPP TSG-SA WG6 Meeting #54-e	S6-231129

17th – 26th April 2023	(revision of S6-23xxxx)

Source:	BDBOS, Netherlands Police

Title:	Modification of solution #5

Spec:	3GPP TR 23.700-38 v1.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	dania.azem@bdbos.bund.de

1. Introduction

This pCR modifies solution #5.

2. Reason for Change

SA6#53 approved pCRs on common ACM procedure (Tdoc S6-230816) and handling the functional alias (Tdoc S6-230818). The agreed changes should be considered and implemented in other relevant solutions. This pCR applies the required changes to solution #5.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-38 v1.2.0.

* * * First Change * * * *

## 7.7	Solution #5: Request to add users for migration to a partner MC system

### 7.7.1	Solution description

#### 7.7.1.1	General

This solution addresses the following aspects:

-	Key issue 3 - change user configuration

#### 7.7.1.2	Information flows

##### 7.7.1.2.1	User configuration request

Table 7.7.1.2.1-1 describes the information flow of the user configuration request sent from the primary ACMC to the partner ACMS(s) and, if required, the relevant partner ACMC(s).

Table 7.7.1.2.1-1: User configuration request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the primary MC system who triggers the information update. |
| Functional alias | O | The functional alias of the MC service user at the primary MC system who triggers the information update. |
| MC service ID | O (see NOTE 1) | The identity of the MC service user at the partner MC system towards which the information update is sent |
| Functional alias | O (see NOTE 1) | The functional alias of the MC service user at the partner MC system towards which the information update is sent |
| MC service ID list | M | Set of identities of the migrating MC service user(s). |
| Request category | O | A set of request notification identifiers, e.g. high, medium, low for alerting partner MC system of specific urgencies. |
| Request duration | O | A set of time and date information for each MC service user to inform the partner MC system when the user is expected to attain service at the partner MC system and for how long that MC service user is expected to receive service(s). |
| Additional Information (see NOTE 2) | O | Additional set of MC service user or UE identification information, such as labels, hardware UE identifiers or other identifying information, which may include device descriptions and user capability information. This information is linked to a particular MC service user and/or UE. |
| NOTE 1: Either the MC service ID or the functional alias must be present. NOTE 2: Further details on what elements need to be included in this message are FFS |  |  |

##### 7.7.1.2.2	User configuration response

Table 7.7.1.2.2-1 describes the information flow of the user configuration response sent from a primary ACMS to a primary ACMC or a partner ACMC to a partner ACMS and primary ACMS, ACMC.

Table 7.7.1.2.2-1: User configuration response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the partner MC system which handled the received update. |
| Functional alias | O | The functional alias of the MC service user at the partner MC system which handled the received update. |
| MC service ID list | M | Set of MC service IDs used in the primary MC system of the migrating MC service user(s) |
| MC service ID list (see NOTE) | M | Set of MC service IDs of the migrating MC service user(s) created by the partner MC system of the migrating MC service user, corresponding to the MC service IDs above. |
| MC service user profile | M | Set of MC service user profiles that corresponds to each MC service ID in the list above. The user profile contains the initial UE configuration to access the partner MC system as defined in table A.6-1 of 3GPP TS 23.280 [5], and limited information for migration for each MC service user. |
| Response | M | Result information of the user configuration request. When sent within the primary MC system, this IE contains the result of the authorization check. |
| NOTE: The primary MC system of the migrated MC service user keeps a mapping of the MC service IDs used by MC service user(s) in the partner MC system(s). |  |  |

#### 7.7.1.3	Procedure

The procedure for an authorized MC service user in a primary MC system to request a partner MC system to authorize a list of MC user IDs from its MC System to migrate to that partner MC System is shown in figure 7.7.1.3-1.

Pre-conditions

-	The primary and partner MC system(s) are configured to accept connections from relevant ACMCs via the respective ACMSs in each of the connected MC service system(s) and have been configured and authorized successfully to allow exchange of administrative configuration information

-	The relevant ACMC(s) and ACMS connection authorizations have been established successfully

-	The primary ACMC(s) have access to the MC service user database in the primary MC system – not shown in the procedure below for simplicity reasons

-	When a functional alias is used as the target of the request by an ACM client in the primary MC system, it is required that the primary ACM server has subscribed to the MC service functional alias controlling server within the primary MC system, and that the partner ACM server has subscribed to the MC service functional alias controlling server within the partner MC system.

Figure 7.7.1.3-1: User configuration request in partner MC system

1.	The primary ACMC sends user configuration request to the primary ACMS, requesting to authorize a list of MC users from the primary MC System to migrate to the partner MC System.

2.	The primary ACMS performs an authorization check to verify that the MC service user is authorized to perform this action. A successful authorization check results in the user configuration request being forwarded to the relevant partner ACMS.

3.	The primary ACMS sends the user configuration request to the partner ACMS.

NOTE 1:	Step 3 is not followed if the authorization was unsuccessful.

4. 	The partner ACMS sends a process indication to the primary MC system, and stores, verifies and assesses the incoming request.

4a.	Based on local policy, the partner ACMS may automatically handle the request as described in clause 7.1.2 and send a response to the primary ACMS (i.e. perform step 8).

4b.	Otherwise, the ACM server requests verification by an ACM client, following the pending request procedure in clause 7.1.1 if the user is not logged on, and steps 5, 6 and 7. If the request targets a functional alias the ACM server resolves the functional alias (based on the current state of the functional alias) and proceeds as follows:

-	if the functional alias is active for one ACM client, the request is sent to the ACM client;

-	if the functional alias is active for more than one ACM client, the ACM server sends the request to one of these ACM clients based on the local policy;

-	if the functional alias is not currently active, the ACM server stores the request.

5.	The partner ACMS sends the stored user configuration request to the partner ACMC.

6.	The authorized MC service user of partner MC system checks the content of the user configuration request and decides whether to approve it or not. The authorized user in the partner MC system takes whatever actions are needed to apply the configurations. The actions within the partner MC system to apply the configurations are outside the scope of the 3GPP specifications.

7. The partner ACMC sends user configuration response to the partner ACMS.

8. The partner ACMS sends user configuration response to the primary ACMS.

9.	The primary ACMS stores the user configuration response. If the primary ACMC is not logged on the primary ACMS will follow the pending request procedure as described in clause 7.1.1.

10.	The primary ACMS sends the user configuration response to the primary ACMC.

### 7.7.2	Solution evaluation

This solution introduces a solution to request to add users for migration to a partner MC system. The solution returns to the primary system the status of the request, and all necessary information that is needed in the primary MC system to configure the migrating users.

* * * End of Change * * * *