3GPP TSG-SA WG6 Meeting #54-e	S6-231134

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.379 | CR | 0341 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Criteria to Preconfigured User Regroup Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, AT&T, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is to align TS23.379 with TS23.280. During SA6 meeting 52 S6-230806 was agreed that added criteria to the preconfigured regroup request and associated procedure in TS23.280. This CR makes the same changes to TS23.379 so both TS’s are aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An additional IE service user criteria, to the Preconfigured regroup request for the MC service client-MC service server and the MC service server-MC service server messages and updates the message flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT will not have the same capability as MCData and MCVideo |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.34 and 10.6.2.9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

##### 10.6.2.2.34	Preconfigured regroup request (MCPTT client – MCPTT server)

Table 10.6.2.2.34-1 describes the information flow preconfigured regroup request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.34-1 Preconfigured regroup request information elements

| Information Element | Status | Description |  |
| --- | --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |  |
| MCPTT group ID | O | MCPTT group ID of the regroup group |  |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |  |
| MCPTT group ID list | O (see NOTE) | List of MCPTT groups to be regrouped into the group regroup group |  |
| MCPTT user ID list | O (see NOTE) | List of MCPTT users to be regrouped into the user regroup group |  |
| Requested priority | O | Priority level requested for the call. |  |
| MC service user criteria | O (see NOTE) |  | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location, which is left to implementation. |
| NOTE: One and only one of these shall be present. |  |  |  |

* * * 2nd Change * * * *

###### 10.6.2.9.2.1	Regroup formation using preconfigured group in single MCPTT system

Figure 10.6.2.9.2.1-1 illustrates the procedure to initiate a group regroup procedure using a preconfigured MCPTT group. The procedure takes place prior to the establishment of a group call to the regroup group.

Pre-conditions:

-	MCPTT client 2 is an affiliated member of MCPTT group 1 and MCPTT client 3 is an affiliated member of MCPTT group 2.

-	The MCPTT group identity and group configuration for the regroup MCPTT group have been preconfigured in MCPTT clients 2 and 3, and MCPTT clients 2 and 3 have received the relevant security related information to allow them to communicate in the regroup MCPTT group.

-	MCPTT client 1 is authorized to initiated a preconfigured regroup procedure.

-	MCPTT client 1 is aware of a suitable preconfigured regroup group whose configuration has been preconfigured in the MCPTT UEs of the group members who will be regrouped.

-	In order to be aware whether the group is regrouped, the MCPTT server is subscribed to the group configuration in GMS.

-	The GMS has subscribed group dynamic data as specified in subclause 10.1.5.5.1 from the MCPTT server using the procedures defined in subclause 10.1.5.6 in TS 23.280 [16].

Figure 10.6.2.9.2.1-1: Regroup procedure using preconfigured group in single MCPTT system

1.	The authorized user of MCPTT client 1 initiates the regroup procedure, specifying the list of MCPTT groups to be regrouped (MCPTT groups 1 and 2), the MCPTT group ID of the regroup group (if available) and the MCPTT group ID of the group from which configuration information for the regroup group is to be taken. Alternatively, the MC service client 1 can provide the criteria which allows the MC service server to determine the list of MC service users to be regrouped.

2.	MCPTT client 1 sends the preconfigured regroup request to the MCPTT server.

3.	The MCPTT server checks that MCPTT client 1 is authorized to initiate a preconfigured regroup procedure, and resolves the group identities of the MCPTT groups requested in step 1. The MCPTT server also checks which group members are affiliated to MCPTT groups 1 and 2. The MCPTT server may retrieve the configuration for the regroup group from the GMS if that configuration information is not already known to the MCPTT server. The MCPTT server also checks that none of the MCPTT groups that are requested for regrouping are already regrouped by any mechanism. If the preconfigured regroup request is authorized, the MCPTT server assigns a MCPTT group ID for this regroup group call if:

i. one is not provided in the request or

ii. the one provided in the request is not accepted.

NOTE 1:	This procedure does not require that the authorized user of MCPTT client 1 is a group member of MCPTT groups 1 or 2, or that the authorized user of MCPTT client 1 is an affiliated group member of MCPTT groups 1 or 2.

NOTE 2:	The list of groups included in the regroup is held in dynamic data in the MCPTT server, and is not used to update group configuration information in the group management server.

4a.	If the MCPTT server determines that any of the groups requested for regrouping, including the regroup group, have been regrouped by other group regrouping procedures, the MCPTT server then sends a preconfigured regroup reject back to MCPTT client 1 with a reject reason indicating that one of the groups has already been regrouped, and this procedure terminates.

4b. The MCPTT server shall send the preconfigured regroup request return message to MCPTT client 1 containing the below:

i.	the MCPTT group ID is generated by the MCPTT server; and

ii.	result of whether the ad hoc group call is authorized or not

If the preconfigured regroup request is not authorized, MCPTT client 1 shall not proceed with the rest of the steps.

5.	The MCPTT server sends the preconfigured regroup requests to MCPTT clients 2 and 3 in steps 5a and 5b respectively.

NOTE 3:	Only group members that are affiliated to the MCPTT groups that are to be regrouped are sent a preconfigured regroup request.

6.	MCPTT clients 2 and 3 notify their users of the regrouping in steps 6a and 6b respectively.

7.	MCPTT clients 2 and 3 may send the preconfigured regroup response to the MCPTT server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request.

8.	The MCPTT server affiliates the regrouped MCPTT clients to the regroup group.

9.	The MCPTT server sends a preconfigured regroup response to MCPTT client 1.

After the group regrouping procedure, the regrouping remains in effect until explicitly cancelled by the procedure in 10.6.2.9.2.2.

MCPTT client participation in the ongoing regroup persists until the MCPTT client is no longer affiliated to any of the regrouped groups (group 1 or 2 in this procedure).

MCPTT client affiliation to the regroup group may cease when the UE's MCPTT service ceases, e.g. when the UE is powered down, or by performing a log-off operation.

Editor's note:	Data persistence in the MCPTT client following a user log-off or power down needs further study.