3GPP TSG-SA WG6 Meeting #54e	S6-231137

e-meeting, 17th – 26th April 2023

Source:	Deutsche Telekom

Title:	Retrieving slice specific application performance statistics data

Spec:	3GPP TS 23.436 v.1.0.0

Agenda item:	8.16

Document for:	Approval

Contact:	Vasil Aleksiev, vasil.aleksiev@magenta.at

1. Introduction

As the ADAES is part of application enabler/SEAL and has a database to store specific slice events, a VAL server or other SEAL servers can need information on certain slice events or statistics from the past.

2. Reason for Change

The ADAES architecture stores historical data in A-ADRDF, but there is lack of procedure and service flows to extract such data if specific statistics are needed for a certain time window in the past for network slice statistics. Once ADAES service consumer (can be NSCE server) subscribes for slice specific application performance analytics, it can receive notifications according to its subscription. The ADAES service consumer may not have a data storage (for example NSCE does not have dedicated data storage) for storing the specific notifications.

4. Proposal

Clarifying in the general clause of slice specific application performance analytics (8.3.1) that the procedure introduced in 8.7.3 with S6-231136 can be used for retreiving slice specific application performance statistics data.

* * * First Change * * * *

## 8.3	Procedure on support for slice-specific application performance analytics

### 8.3.1	General

This clause describes the procedure for supporting slice-tailored application performance analytics. The ADAES service consumer may not have a data storage (for example NSCE does not have dedicated data storage) for storing the specific notifications. Procedure in 8.7.3 can be used for retreiving of slice -specific application performance data.

* * * End of Changes * * * *