3GPP TSG-SA WG6 Meeting #54-e	S6-231145

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0265 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting a NOTE related to UE Identifier API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NOTE 1 in clause 8.6.5 reads as follows: “To overcome CN UE's assigned private IP address reuse issue (e.g UE's IPv4 reuse by 5GC), the EES would either need to be perconfigured with the public IP address range (used by the NAT function over N6) and its associated IP domain or the UPF would need to support NAT function.” The bold part of the statement above (i.e. “or the UPF would need to support NAT function”) is not related to how to overcome the private IP address reuse issue which is the intended context of NOTE 1 and hence it needs to be removed in order to make NOTE 1 overall statement coherent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed “or the UPF would need to support NAT function” statement from NOTE 1. - Made other small editorial changes for better read in clause 8.6.5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will be confusing to the reader. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5, 8.6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 8.6.5	UE Identifier API

#### 8.6.5.1	General

EES exposes UE Identifier API to the EAS and EEC in order to provide an identifier uniquely identifying a UE. This API is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. hasn't already cached). This identifier, called UE ID and defined in clause 7.2.6, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 and/or EDGE-7 depending on the UE ID type.

The EAS's direct invocation of the UE Identifier API of the EES may result in UE ID not found response (e.g. if the NATed UE's public IPv4 address can’t be resolved by the core network). Under such circumstances, the EAS may choose to signal its AC to trigger the UE ID query onto the EEC over EDGE-5 (see clause 8.14.2.6). In turn, the EEC would invoke the EES's UE Identifier API using the UE's CN assigned IP addresses (i.e. IPv4 and/or IPv6) which should result in return of the UE ID to the EEC and from thereon to the AC and the EAS.

NOTE 1:	To overcome CN UE's assigned private IP address reuse issue (e.g UE's private IPv4 reuse by 5GC), the EES would need to be perconfigured with the public IP address range (used by the NAT function over N6) and its associated IP domain.

NOTE 2:	EEC retrieval of the UE's IP address from the device is out of scope.

# * * * Change 2 * * * *

#### 8.6.5.2	Procedure

Figure 8.6.5.2-1 illustrates the interactions between the EES and the EAS.

Pre-conditions:

1.	The EAS or EEC is authorized to discover and to use UE Identifier API provided by the EES.

2.	When the EEC is used to invoke the UE Identifier API with the UE IPv6 address as the input parameter, the UE IPv6 address may or may not be NATed. If NATed however, the IPv6 may not be reused (i.e. assigned to more than one UE simultaneously). If the EEC already has the UE ID (GPSI as per clause 7.2.6), and it needs the Edge UE ID to share with an AC/EAS, this procedure can still be used to retrieve Edge UE ID.

3.	EAS is considered an AF behind EES (as another AF) and EES is authorized to pass EAS ID instead of its own AF ID when it needs to interact with the NEF’s Nnef_UEId_Get (as per TS 23.502 clause 4.15.10 "AF specific UE ID retrieval").

Editor's note:	[SA3] [SA2] Check pre-condition #3 to ensure EES is Ok to pass on EAS ID in NEF’s Nnef_UEId as opposed to its own AF ID.

Figure 8.6.5.2-1: UE Identifier API

1.	The EAS or EEC invokes UE Identifier API exposed by the EES. If it is the EAS invoking the API and it recognizes that the UE’s IP address is a public IP address, i.e., the UE is behind a NAT, the Port Number and associated IP address should be included in user information.

2.	The EES uses the received user information in the step 1 (e.g. IP address) and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3].

3.	The EES provides the UE identifier to the EAS or to EEC (i.e. whichever invoked the API). The UE identifier returned in the response which is referred to as UE ID may be the 3GPP Core Network assigned UE ID (aka AF-sepecific UE ID; see TS 23.502 clause 4.15.10) or the EES-generated Edge UE ID as defined in clause 7.2.9. If UE ID (GPSI as per clause 7.2.6) is included in the request received from EEC, the EES can provide the Edge UE ID based on the received UE ID and step 2 can be skipped.

Editor's note:	[SA3] Whether and how user's consent is obtained to share the UE identifier with a particular EAS or EEC is SA3's responsibility.

4.	The EAS uses the UE ID received in step 3 to invoke capability exposure API(s) provided by the EES over EDGE-3 and/or EDGE-7 depending on the UE ID type.

NOTE:	UE ID of type CN-assigned can be used over EDGE-3 and EDGE-7 whereas the UE ID of type EES-generated Edge UE ID can be used over EDGE-3 only.

# * * * * End of Changes * * * *