3GPP TSG-SA WG6 Meeting #54-e	S6-231148

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0268 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on ECS discovery procedure description regarding DNN and slice information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed in the last meeting that the connection with the V-ECS can be established over an HR or LBO PDU Session. Accordingly, this CR proposes to correct the misaligned descriptions on DNN and S-NSSAI information for roaming UE to establish a PDU Session with the partner ECS in the ECS discovery procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on the description related to DNN and S-NSSAI information for roaming UE to establish a PDU Session with the partner ECS in the ECS discovery procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description in ECS discovery procedure remains incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2.3.2, 8.17.2.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to KI#10 and Sol#4/Sol#14 in TR 23.700-98. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

8.17.2.3.2	Request-response model

When required to find a suitable partner ECS, the ECS queries the ECS-ER by providing information such as the location of the UE, applications required by the UE etc. In response the ECS-ER provides ECS configuration information of partner ECS(s) providing the required application at the location indicated by the ECS. Figure 8.17.2.3.2-1 illustrates the procedure.

Pre-conditions:

1.	The ECS has the address (e.g. URI) of the ECS-ER.

Figure 8.17.2.3.2-1: ECS querying ECS-ER

1.	The ECS sends a ECS discovery request to the ECS-ER. The request contains security information of the ECS, Service Provisioning filters received from the EEC or the EES and UE’s current location. If the UE hosting the EEC is roaming in a V-PLMN (determined using the serving PLMN information in the received service provisioning request, or by interacting with the H-PLMN), then the ECS discovery request includes the information of the serving PLMN i.e. the V-PLMN.

2.	The ECS-ER authorizes the received request. If authorized, the ECS-ER processes the request and gathers the information of partner ECS(s) that can satisfy the query parameters received in step 1.

Editor's note: Whether and how ECS-ER uses a different procedure to collect information from another ECS-ER or ECS from a different domain is FFS.Related updates to EDGE-10 interface are FFS.

3.	The ECS-ER sends a ECS discovery response to the ECS. The response includes ECS configuration information of the Partner ECS(s) available in the H-PLMN and/or the V-PLMN, depending on the serving PLMN of the UE as indicated in step 1 and may include DNN and S-NSSAI information for roaming UEs to establish a PDU session with the Partner ECS as specified in 3GPP TS 23.548 [20]. The ECS caches the received information for further use.

NOTE:	The ECS sending the ECS discovery request can also be an ECS-ER.

* * * Next Change * * *

8.17.2.3.3.3	Notify

Figure 8.17.2.3.3.3-1 illustrates the procedure.

Pre-conditions:

1.	The ECS has subscribed for ECS discovery.

Figure 8.17.2.3.3.3-1: ECS discovery notification

1.	An event occurs at the ECS-ER that satisfies trigger conditions for ECS discovery notification, e.g. after new ECS registration.

2.	The ECS-ER sends a ECS discovery notification to the ECS. The notification includes the ECS configuration information of the Partner ECS(s) as requested in the subscription and may include information for roaming UEs to establish a PDU session with the Partner ECS as specified in 3GPP TS 23.548 [20]. The ECS caches the information for further use.

* * * End of Change * * *