3GPP TSG-SA WG6 Meeting #54-e	S6-231172

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0120 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | An Application Client is not a Constrained UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A UE in which the Application Client (AC) resides on another UE than the MSGin5G Client with which it communicates over MSGin5G-5 is a deployment option and is not an example of a Constrained UE. In both the case where the AC resides on the same UE as where the MSGin5G Client resides or the case where the AC resides on a different UE, there is no difference in any procedure; in both cases the communication is over MSGin5G-5 reference point. "If allowed by configuration UE-1 may act as a Gatewayy UE". The words "if allowed" seem unecessary, because "if not allowed" Constrained UEs cannot be supported and what would then be the purpose of specifying the support of Constrained UEs? If Constrained UEs are supported in the service is an implementation option, but the TS needs to specify the case where such UEs are supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of the architecture option in which a UE that only has an AC is considered a Constrained UE. If UE-2 is a Constrained UE, UE-1 is named a Gateway UE. The names Proxy UE and Relay UE have been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on how to specify two identical architecture options in clauses that are different under the definitions of a Constrained or Unconstrained UE. Three names for UE-1 (the "UE-in-the-middle"), where a single name is sufficient, is confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	Application Architecture

Figure 5.2-1 shows the application architecture of the MSGin5G Service.  The MSGin5G Service shall fulfil the service requirements which are enumerated in 3GPP TS 22.262 [2] and the architecture requirements enumerated in clause 4.

Figure 5.2-1: Application Architecture of the MSGin5G Service

The MSGin5G Client(s) interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service. The Legacy 3GPP Message Gateway and Non-3GPP Message Gateway may interact with SEAL clients over the SEAL-C reference point specified for each SEAL service. The MSGin5G Server(s) interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [5].

NOTE 1:	For simplicity, the SEAL clients' interaction with Legacy 3GPP Message Gateway and Non-3GPP Message Gateway, and the SEAL-UU interface between Legacy 3GPP Message Gateway and Non-3GPP Message Gateway and SEAL servers are not shown in Figure 5.2-1.

The MSGin5G UE-1 communicates with MSGin5G Server over MSGin5G-1 reference point. The MSGin5G UE-2 is a constrained device which does not have enough capability to communicate with MSGin5G Server directly. MSGin5G UE-1 acts as a UE Message Gateway to MSGin5G UE-2.

The Legacy 3GPP Message Gateway interacts with MSGin5G Server over MSGin5G-2 reference point on behalf of Legacy 3GPP UE (e.g., SMS, NIDD).

The Non-3GPP Message Gateway interacts with MSGin5G Server over MSGin5G-4 reference point on behalf of Non-3GPP UE.

The Broadcast Message Gateway interacts with MSGin5G Server over MSGin5G-7 reference point.

NOTE 2:	A SEAL Group Management Server and a SEAL Configuration Management Server (both specified in 3GPP TS 23.434 [5]) may be collocated in the MSGin5G Server. A SEAL Configuration Management Client specified in 3GPP TS 23.434 [5] may be collocated in the MSGin5G Client, Legacy 3GPP  Message Gateway and Non-3GPP Message Gateway. The implementation of such deployment option is out of scope of the present specification.

NOTE 3:	If an Application Client resides on the same UE as the MSGin5G Client with which it communicates over the MSGin5G-5 reference point or if it resides on a different UE is an implementation option.

NOTE 5:	Depending on the non-3GPP message service, the interaction between Non-3GPP message client and Non-3GPP Message Gateway may involve 3GPP Core Network.

NOTE 6:	3GPP Core Network may not be involved for the interaction between the Non-3GPP Message Gateway and the MSGin5G Server.

Figure 5.2-2 illustrates the functional model for interconnection between MSGin5G servers.

# Figure 5.2-2: Interconnection between MSGin5G Servers

To support distributed SEAL server deployment in one PLMN, and delivering MSGin5G messages between different PLMNs, the MSGin5G Server interacts with another MSGin5G Server over MSGin5G-8 reference point.