3GPP TSG-SA WG6 Meeting #54-e	S6-231189

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0275 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EES and ECS resource migration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reliable EC has been concluded in Rel-18 TR 23.700-98. This CR introduces procedures in the Edge Enablement Layer to support resource migration for the EES and the ECS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the following information - Feature description of EES and ECS resource migration - Requirements of EES and ECS resource migration - Procedures and information flows of EES and ECS resource migration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EES and ECS resource migration needed cannot be migrated in reliable edge computing scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x, 5.2.Y, 5.2.Y1, 5.2.Y2, 8.Z, 8.Z.1, 8.Z.2, 8.Z.2.1, 8.Z.2.2, 8.Z.2.2.1, 8.Z.2.2.2, 8.Z.2.2.3, 8.Z.2.3, 8.Z.2.3.1, 8.Z.2.3.2, 8.Z.2.3.3, 8.Z.2.4, 8.Z.2.4.1, 8.Z.2.4.2, 8.Z.2.4.3, 8.Z.2.4.4, 8.Z.2.4.5, 8.Z.2.5, 8.Z.2.5.1, 8.Z.2.5.2, 8.Z.3, 8.Z.3.1, 8.Z.3.2, 8.Z.3.3, 8.Z.3.4, 8.Z.3.5, 8.Z.3.6, 8.Z.3.7, 8.Z.3.8, 8.Z.3.9, 8.Z.3.10, 8.Z.3.11, 8.Z.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Reliable EC |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 4.X	Support for EES and ECS resource migration

The edge enablement layer provides server migration capabilities for EES and ECS relocation to provide reliable edge computing services to consumers. When an EES or ECS becomes unavailable (e.g., planned or unplanned unavailability), resources of the EES or ECS associated with consumers are relocated to an alternate EES or ECS where service can be resumed to minimize edge service interruptions and reconfiguration.

* * * Next Change * * * *

### 5.2.Y	EES and ECS resource migration

#### 5.2.Y.1	General

This clause specifies the requirements for EES and ECS resource migration.

#### 5.2.Y.2	Requirements

[AR-5.2.Y.2-a]	The application layer architecture shall provide mechanisms to support EES resource migration such that resources associated with an EES consumer can be relocated to an alternate EES.

[AR-5.2.Y.2-b]	The application layer architecture shall provide mechanisms to support ECS resource migration such that resources associated with an ECS consumer can be relocated to an alternate ECS.

[AR-5.2.Y.2-c]	The application layer architecture shall provide mechanisms to support EES or ECS resource migration to an alternate EES or ECS for scenarios of planned unavailability.

[AR-5.2.Y.2-d]	The application layer architecture shall provide mechanisms to support EES or ECS resource migration to an alternate EES or ECS for scenarios of unplanned unavailability.

* * * Next Change * * * *

## 8.Z	EES and ECS resource migration

### 8.Z.1	General

The EES or ECS resource migration capability allows to migrate EES/ECS consumer’s resources to an alternate EES or ECS to minimize service interruption and reconfiguration.

The migration capability supports planned and unplanned unavailability scenarios, and the consumer resources migrated may include consumer’s profile, consumer’s context, consumer’s subscription information or any information that allows a consumer to consume services from an alternate server with minimum disruption or reconfiguration.

### 8.Z.2	Procedure

#### 8.Z.2.1	General

The following clauses define procedures for planned and unplanned EES or ECS unavailability scenarios.

#### 8.Z.2.2	Planned unavailability procedures

##### 8.Z.2.2.1	General

The EES or ECS resource migration procedures allows for planned unavailability scenario such as a planned maintenance shutdown. In this scenario, the EES or ECS perform consumer resource migration to an alternate server identified using configured server set information before the EES or ECS becomes unavailable, then the EES or ECS inform the consumers of the alternate server availability.

Clause 8.Z.2.2.2 illustrate EES resource migration for planned unavailability.

Clause 8.Z.2.3.3 illustrates ECS resource migration for planned unavailability.

##### 8.Z.2.2.2	EES resource migration for planned unavailability

Figure 8.Z.2.2.2-1 illustrates the EES resource migration for planned unavailability procedure.

Pre-conditions:

- The EES has been pre-configured with EES server set information;

- The EES has detected a condition requiring consumer’s resource migration to an alternate EES;

- EEC or EAS consumers have subscribed to “EES resource migration information” with the EES;

Figure 8.Z.2.2.2-1: EES resource migration for planned unavailability

1.	Upon detecting a condition requiring consumer’s resources migration, the EES selects an alternate EES using pre-configured EES server set information considering the alternate EES priority. The EES server set information contains identifiers of alternate EES instance(s), endpoint information of the alternate EES, altenate EES instance(s) priority and may contain connectivity information to the alternate EES instance(s) as described in table 8.Z.3.6-3.

2.	The EES sends an EES resource migrate request to the selected alternate EES, the request contains security credentials, the identifier of the EES instance for planned unavailability and the EES consumer information as described in table 8.Z.3.2-1.

3.	Upon receiving the EES resource migrate request from the EES, the alternate EES verifies if the requesting EES is authorized to perform the operation using the security credentials and may verify that the requesting EES is a member of the server set. If the operation is authorized, the alternate EES fetches the resource’s data associated with each consumer identified in the request and stores the resources. The alternate EES creates any processes associated to migrated resources such as event detection for subscriptions, ACR detection or subscription to 3GPP core network, for example periodic UE location updates. The alternate EES may change identifier of migrated resources if required, for example a migrated subscription identifier may need to be updated at the alternate server; the alternate EES creates a mapping of migrated resources that needed their identifiers to be updated.

4.	The alternate EES sends an EES resource migrate response to the EES. The response indicates if the resource migration was successful including a list of migrated resource information which contains for each consumer, the consumer identifier and may contain for each consumer a mapping of migrated resources information and procedure(s) that the alternate EES requires consumers to perform, for example the alternate EES may require consumer(s) to perform a registration update to complete the migration. If the EES resource migration was not successful, the alternate EES indicates failure in the resource migration response and may include a reason.

5.	If EES resource migrate response is successful, the EES sends an EES resource migrate notify to each consumer including the subscription identifier, the event identifier (e.g., “EES resource migration information”), the alternate EES information, and migrated resources information as described in table 8.Z.3.6-1. Additionally, the EES may perform local resource management and may unregister from the ECS. If resource migration failed, the EES may retry the EES resource migrate request with the same or a different alternate EES from the server set considering the failure reason.

Upon receiving the EES resource migration notify, the consumer(s) (e.g., EEC, EAS) replace bindings associated to the EES with bindings to the alternate EES and update identifiers of migrated resources according to the resource mapping if included in the notification. The EEC or EAS may use the altenate EES endpoint and connectivity information to access the alternate EES and, if the notification included alternate EES procedures, shall perform the required procedures with the alternate EES.

##### 8.Z.2.2.3	ECS resource migration for planned unavailability

Editor’s Note: ECS resource migration for planned unavailability is FFS.

#### 8.Z.2.3	Unplanned unavailability procedures

##### 8.Z.2.3.1	General

The EES or ECS migration procedures allow for unplanned unavailability scenarios such as a power outage of an EES or ECS. In this scenario, the EES or ECS consumer(s) detect service unavailability, identify an alternate server using provisioned server set information and inform the alternate server to migrate resources associated to the consumer.

To support unplanned outage scenario, consumer’s resources of an EES or ECS must be available to an alternate EES or ECS after the EES or ECS is unavailable; this is possible if a shared storage between server set participants is used or if direct resource synchronization between server set participants is performed.

NOTE: Resource synchronization using shared storage is out of scope of this specification.

Editor’s Note: It is FFS whether direct resources synchronization is in scope of this specification.

Clause 8.Z.2.3.2 illustrate EES resource migration for unplanned unavailability.

Clause 8.Z.2.3.3 illustrates ECS resource migration for unplanned unavailability.

##### 8.Z.2.3.2	EES resource migration for unplanned unavailability

Figure 8.Z.2.3.2-1 illustrates the EES resource migration for unplanned unavailability procedure.

Pre-conditions:

- The EES has been pre-configured with EES server set information;

- The EES has established resource synchronization with alternate EES of the server set;

- The EEC and EAS have been provisioned with EES server set information as described in 8.Z.2.5;

- The EEC and EAS have detected a condition (e.g., failed request) requiring EES resource migration to an alternate EES;

Figure 8.Z.2.3.2-1: EES resource migration for unplanned unavailability

1.	Upon detecting a condition requiring EES resources migration, the EEC or EAS select an alternate EES using pre-configured EES server set information considering the altenate EES priority. The EES server set information contains identifiers of alternate EES instance(s), endpoint information of the alternate EES, altenate EES instance(s) priority and may contain connectivity information to the alternate EES instance(s) as described in table 8.Z.3.6-3.

2.	The EEC or EAS send an EES resource migrate request to the selected alternate EES, the request contains security credentials, the identifier EES instance for unplanned unavailability and the consumer information of the EEC or EAS as described in table 8.Z.3.2-1.

3.	Upon receiving the EES migrate request from the EEC or EAS, the alternate EES verifies if the requesting EEC or EAS is authorized to perform the operation using the security credentials and may verify that the unavailable EES is a member of the server set. If the operation is authorized, the alternate EES fetches the resource’s data associated with the consumer identified in the request and stores the resources. The alternate EES creates any processes associated to migrated resources such as event detection for subscriptions, ACR detection or subscription to the 3GPP core network, for example periodic UE location updates. The alternate EES may change identifier of migrated resources if required, for example a migrated subscription identifier may need to be updated at the alternate server; the alternate EES creates a mapping of migrated resources that needed their identifiers to be updated.

4.	The alternate EES sends an EES resource migrate response to the EEC or EAS. The response indicates if the resource migration was successful including migrated resource information which contains the consumer identifier and may contain a mapping of migrated resources information and procedure(s) that the alternate EES requires consumers to perform, for example the alternate EES may require the consumer to perform a registration update to complete the migration. If the EES resource migration was not successful, the alternate EES indicates failure in the resource migration response and may include a reason.

Upon receiving the EES resource migration response, the EEC or EAS replace bindings associated to the unavailable EES with bindings to the alternate EES and updates the identifiers of migrated resources according to the resource mapping if included in the response. The EEC or EAS use the altenate EES connectivity information to access the alternate EES and, if the notification included alternate EES procedures, shall perform the required procedures with the alternate EES.

##### 8.Z.2.3.3	ECS resource migration for unplanned unavailability

Editor’s Note: ECS resource migration for unplanned unavailability is FFS.

#### 8.Z.2.4	EES Resource migrate subscribe-notify model

##### 8.Z.2.4.1	General

Clause 8.Z.2.4.2 and clause 8.Z.2.4.3 illustrate the EES resource migrate subscribe and notify procedures.

Clause 8.Z.2.4.4 illustrates the EES resource migrate subscription update procedure.

Clause 8.Z.2.4.5 illustrates the EES resource migrate unsubscribe update procedure.

##### 8.Z.2.4.2	Subscribe

Figure 8.Z.2.4.2-1 illustrates the EES resource migrate subscribe procedure between the EEC or EAS and the EES. This subscription enables EES to provide resource migration information of interest to the EEC or EAS (e.g., alternate EES migration information and/or EES server set changes information).

Pre-conditions:

1.	The EEC or EAS have received information (e.g., URI, IP address) related to the EES;

2.	The EEC or EAS have received appropriate security credentials authorizing communication with the EES;

3.	The EEC has optionally acquired a Notification Target Address to be used in its subscriptions to notifications.

NOTE 1:	How the EEC acquires the notification target address or a notification channel URI to receive the notifications is out of scope of this release. The notification target address can terminate at the EEC (e.g., in an IoT device) if the deployment supports EEC reachability, or it can terminate at a push notification service. Details of the push notification service are out of scope of this release.

Figure 8.Z.2.4.2-1: EES resource migration subscribe

1.	The EEC or EAS send an EES resource migration subscribe request to the EES. The EES resource migration subscribe request includes the requestor instance identifier (e.g., EECID or EAS endpoint), the security credentials, the notification endpoint and an event identifier (e.g., EES resource migration information or EES server set change information).

2.	Upon receiving the request, the EES checks if the requestor is authorized to subscribe for resource migration. If the request is authorized, the EES creates and stores the subscription for resource migration.

3.	If the processing of the request is successful, the EES sends an EES resource migration subscription response to the requestor, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC or EAS shall send an EES resource migration subscription update request prior to the expiration time. If an EES resource migrtation subscription update request is not received prior to the expiration time, the EES shall treat the EEC or EAS as implicitly unsubscribed. If the processing of the request failed, the EES sends an EES resource migration subscription response to the requesor, which indicates failure of the request and may include a failure reason.

If the EES resource migration subscription request fails, the EEC or EAS may resend the EES resource migration subscription request again, considering the received failure cause.

##### 8.Z.2.4.3	Notifiy

Figure 8.Z.2.4.3-1 illustrates the EES resource migration notify procedure between the EEC or EAS and the EES.

Pre-conditions:

1.	The EEC or EAS has subscribed with the EES for the resource migration information.

Figure 8.Z.2.4.3-1: EES resource migration notify

1.	An event occurs at the EES that satisfies trigger conditions for notifying a subscribed EEC or EAS. If the trigger condition is for EES resource migration information, the information includes the alternate EES information and and information about migrated resources. If the trigger condition is for an EES server set change, the information includes the changed server set information.

2.	The EES sends a resource migration notify to the EEC or EAS with the information determined in step 1.

##### 8.Z.2.4.4	Update

Figure 8.Z.2.4.4-1 illustrates the EES resource migration subscription update procedure between the EEC or EAS and the EES.

Pre-conditions:

1.	The EEC or EAS has subscribed with the EES for the resource migration information.

Figure 8.Z.2.4.4-1: EES resource migration subscription update

1.	The EEC or EAS send an EES resource migration subscription update request to the EES. The EES resource migration subscription update request includes the security credentials, the subscription identifier and may include event identifiers and proposed expiration time.

2.	Upon receiving the request, the EES checks if the requestor is authorized to update the subscription information. If the request is authorized, the EES updates the updates the subscription information.

3.	The EES sends an EES resource migration subscription update response to the EEC or EAS, which may include the expiration time, indicating when the updated subscription will automatically expire. To maintain the subscription, the EEC or EAS shall send a resource migration subscription update request prior to the expiration time. If a resource migration subscription update request is not received prior to the expiration time, the EES shall treat the EEC or EAS as implicitly unsubscribed. If the processing of the request failed, the EES sends an EES resource migration subscription update response to the requestor, which indicates failure of the request and may include a failure reason.

If the EES resource migration subscription update fails, the EEC or EAS may resend the EES resource migration subscription update request again, considering the received failure cause.

##### 8.Z.2.4.5	Unsubscribe

Figure 8.Z.2.4.5-1 illustrates the EES resource migration unsubscribe procedure between the EEC or EAS and the EES.

Pre-conditions:

1.	The EEC or EAS has subscribed with the EES for EES resource migration information.

Figure 8.Z.2.4.5-1: EES resource migration unsubscribe

1.	The EEC or EAS send an EES resource migration unsubscribe request to the EES. The EES resource miration unsubscribe request includes the security credentials of the EEC or EAS along with the subscription identifier.

2.	Upon receiving the request, the EES performs an authorization check to verify whether the requestor has authorization to perform the operation. If authorized, the EES cancels the subscription.

3.	The EES responds with an EES resource migration unsubscribe response.

#### 8.Z.2.5	EES server set provisioning

##### 8.Z.2.5.1	General

EES server set provisioning allows an EEC or EAS to obtain EES server set information needed to support the unplanned unavailability use case.

##### 8.Z.2.5.1	EES server set provisioning

Figure 8.Z.2.5.1-1 illustrates the EES server set provisioning procedure between the EEC or EAS and the EES. This procedure enables the EES to provide server set information when requested by the EEC or EAS.

Pre-conditions:

1.	The EEC or EAS has connectivity information (e.g., URI, IP address) related to the EES;

2.	The EEC or EAS has security credentials authorizing it to communicate with the EES;

3.	The EES has been pre-configured with EES server set information;

Figure 8.Z.2.5.1-1: EES server set provisioning

1.	The EEC or EAS send an EES server set provisioning request to the EES. The EES server set provisioning request includes the requestor instance identifier (e.g., EECID or EAS endpoint) along with security credentials.

2.	Upon receiving the request, the EES checks if the requestor is authorized to request EES server set information. If the request is authorized and the EES has pre-provisioned server set information, the EES identifies the server set information to provide to the requestor.

3.	If the processing of the request was successful, the EES sends an EES server set provisioning response to the requestor indicating successful processing and includes information about the EES server set. The server set information includes alternate EES identifiers, alternate EES priorities and may include alternate EES connectivity information. If the request processing failed, the EES indicates failure and may provide a failure reason.

Upon receiving the EES server set provisioning response and if the response is successful, the EEC or EAS store the EES server set associated with the EES and may initiate EES unavailability detection. If the server set provisioning fails, the requestor may send the EES server set provisioning request again considering the received failure cause.

### 8.Z.3	Information flows

#### 8.Z.3.1	General

The following information flows are specified for EES and ECS resource migration:

-	EES resource migrate request and response;

-	EES resource migrate subscribe request and response;

-	EES resource migrate notification;

-	EES resource migrate subscription update request and response;

-	EES resource migrate unsubscribe request and response.

-	EES server set provisioning request and response;

#### 8.Z.3.2	EES resource migrate request

Table 8.Z.3.2-1 describes information elements for the EES resource migrate request from the EEC or EES or EAS to the EES.

Table 8.Z.3.2-1: EES resource migrate request

| Information element | Status | Description |
| --- | --- | --- |
| EES identifier | M | The identifier of the EES instance |
| List of EES consumer information | M | The information associated with EES consumer(s) |
| > consumer identifier | M | The instance identifier of an EES consumer (e.g., EECID or EAS endpoint) |
| > consumer type | M | The type of an EES consumer (e.g., EEC or EAS) |
| > consumer resource URI(s) | O | The URI(s) associated with an EES consumer instance |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |

#### 8.Z.3.3	EES resource migrate response

Table 8.Z.3.3-1 describes information elements for the EES resource migrate response from the EES to the EEC or EES or EAS.

Table 8.Z.3.3-2 describes information related to migrated EES resources.

Table 8.Z.3.3-1: EES resource migrate response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EES resource migrate request was successful. |
| > List of migrated resource information | M | The information associated with migrated EES resources as described in table 8.Z.3.3-2 |
| Failure response | O | Indicates that the EES resource migrate request failed. |
| > Cause | O | Indicates the cause of EES resource migrate request failure. |

Table 8.Z.3.3-2: EES migrated resource information

| Information element | Status | Description |
| --- | --- | --- |
| consumer identifier | M | The instance identifier of an EES consumer (e.g., EECID or EAS endpoint). |
| migrated resource map | O | The mapping information of resources migrated to the alternate EES. |
| alternate EES procedure(s) | O | The procedure(s) that a consumer shall perform with the alternate EES to complete the migration. |

#### 8.Z.3.4	EES resource migrate subscribe request

Table 8.Z.3.4-1 describes information elements for the EES resource migrate subscribe request from the EEC or EAS to the EES.

Table 8.Z.3.4-1: EES resource migrate subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor instance (e.g., EECID or EAS endpoint) |
| Event identifiers | M | Event identifiers: - EES resource migration information - EES server set information change |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address | O | The Notification target address (e.g., URL) where the notifications destined for the EEC or the EAS should be sent. |

#### 8.Z.3.5	EES resource migrate subscribe response

Table 8.Z.3.5-1 describes the information elements for EES resource migrate subscribe response from the EES to the EEC or EAS.

Table 8.Z.3.5-1: EES resource migrate subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

#### 8.Z.3.6	EES resource migrate notify

Table 8.Z.3.6-1 describes the information elements for EES resource migrate notify from the EES to the EEC or EAS.

Table 8.Z.3.6-2 describes information related to an alternate EES.

Table 8.Z.3.6-3 describes information related to an EES server set.

Table 8.Z.3.6-1: EAS discovery notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Event identifier | M | Either EES resource migration information or EES server set information change |
| EES resource migration information (NOTE 1) | O | The information about the alternate EES and migrated resources |
| > Alternate EES information | M | The information about the alternate EES as described in table 8.Z.3.6.2-2 |
| > List of migrated resource information | M | The information associated with migrated EES resources as described in table 8.Z.3.3-2 |
| Server set information (NOTE 2) | O | The information about the EES server set as described in table 8.Z.3.6-3 |
| NOTE 1: Mandatory if the event identifier is "EES resource migration information” NOTE 2: Mandatory if the event identifier is "EES server set information change” |  |  |

Table 8.Z.3.6-2: Alternate EES information

| Information element | Status | Description |
| --- | --- | --- |
| Alternate EES identifier | M | The EESID of the alternate EES instance |
| Alternate EES information | M | The endpoint information of the alternate EES instance (e.g., IP address, FQDN, URI) |
| Alternate EES connectivity information | O | The connectivity information to the alternate EES instance (e.g., DNN, S-NSSAI) |

Table 8.Z.3.6-3: EES server set information

| Information element | Status | Description |
| --- | --- | --- |
| EES identifier | M | The EESID associated with the EES that provided the server set. |
| List of alternate EES servers | M | The information about alternate EES servers |
| > Alternate EES priority | M | The priority of the alternate EES |
| > Altenate EES information | M | The information of an alternate EES as described in table 8.Z.3.6-2 |
| NOTE: Only if the alternate EES is in a different EDN than the EES |  |  |

#### 8.Z.3.7	EES resource migrate subscription update request

Table 8.Z.3.7-1 describes the information elements for EES resource migrate subscription update request from the EEC or EAS to the EES.

Table 8.Z.3.7-1: EES resource migrate subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| Event identifiers | O | Event identifiers: - EES resource migration information - EES server set information change |

#### 8.Z.3.8	EES resource migrate subscription update response

Table 8.Z.3.8-1 describes the information elements for EES resource migrate subscription update response from the EES to the EEC or EAS.

Table 8.Z.3.8-1: EES resource migrate subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

#### 8.Z.3.9	EES resource migrate unsubscribe request

Table 8.Z.3.9-1 describes the information elements for EES resource migrate unsubscribe request from the EEC or EAS to the EES.

Table 8.Z.3.9-1: EES resource migrate unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be unsubscribed |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |

#### 8.Z.3.10	EES resource migrate unsubscribe response

Table 8.Z.3.10-1 describes the information elements for EES resource migrate unsubscribe response from the EES to the EEC or EAS.

Table 8.Z.3.10-1: EES resource migrate unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |

#### 8.Z.3.11	EES server set provisioning request

Table 8.Z.3.11-1 describes information elements for the EES server set provisioning request from the EEC or EAS to the EES.

Table 8.Z.3.11-1: EES server set provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor instance (e.g., EECID or EAS endpoint) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |

#### 8.Z.3.12	EES server set provisioning response

Table 8.Z.3.12-1 describes information elements for the EES server set provisioning response from the EES to the EEC or EAS.

Table 8.Z.3.3-1: EES server set provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EES resource migrate request was successful. |
| > EES Server set information | M | The information about the EES server set as described in table 8.Z.3.6-3 |
| Failure response | O | Indicates that the EES resource migrate request failed. |
| > Cause | O | Indicates the cause of EES resource migrate request failure. |

* * * End of Changes * * * *